3GPP TSG RAN WG4 Meeting #107	R4-2309152

Incheon, Korea (Republic Of), 22nd – 26th May 2023

Source:		Rohde & Schwarz

Title:	Discussion on UL MIMO EVM measurement methodology

Agenda Item:		4.1

Document for:	Approval

# Introduction

During the RAN4#106 meeting a new method for calculating the UL MIMO EVM was proposed in [1]. As part of the discussions during the meeting a WF [2] was agreed detailing the next steps. In this contribution we share our input on this topic.

# Discussion

## EVM overestimation

As a first step we take a look at the overestimation or degradation for the EVM measured using Method 1 from [2].

Applying Method 1 as described in Annex F of TS 38.101-1 [3] requires to use a single DMRS symbol for the channel estimation:

## F10.2	MIMO Equalization

The MIMO equalization is based only on reference signals (DMRS) without using any data symbols. In order to obtain comparable EVM results independent of the number of DMRS symbols per slot, only the first DMRS symbol in each slot is used.

Unfortunately, this text was copied from TR 38.810, where one DMRS symbol was used to compare different algorithms and not corrected before integrating into TS 38.101-1 [3] or TS 38.101-2 [4]. As the RMCs in the specification provide 3 DMRS symbols all those symbols shall be used in the EVM measurement.

Observation 1: Current versions of TS 38.101-1 and TS 38.101-2 incorrectly capture the number of DMRS symbols to be used for channel estimation.

Since this error needs to be corrected, we provide CRs [5], [6] to TS 38.101-1 and 38.101-2 to correct the number of DMRS symbols to be used during the EVM measurement.

Proposal 1: Update the specification to use all available DMRS in a RMC.

In Figure 2-1 below we now analyse the impact of using 1 DMRS symbol as currently described in TS 38.101-1 or TS 38.101-2 and using all 3 DMRS symbols available in the RMCs on the overall EVM. The SISO EVM derived according to the Annex E of TS 38.521-1 [7] serves as the baseline.

This reduces the overestimation from roughly 0.6 dB (1 DMRS symbol) to 0.2 (3 DMRS symbols), which brings it close to the performance achieved using Method 3 [1].

Figure 2-1 Comparison of EVM using different numbers of DMRS symbols

Observation 2: Using all available DMRS symbols greatly improves the performance of Method 1.

## Complexity considerations

Since Method 1 and 3 now show comparable results with the updates to Method 1 as discussed above (using all 3 DMRS symbols), we need to take a look at the complexity of both methods, especiall when considering that for Release 18 up to 4x4 MIMO needs to be supported. In Figure 2-2 to 2-5 below, we show the schematics for Method 1 and 3 for both 2x2 (schematics taken from [2]) and 4x4 MIMO.

For Method 1 the extension is more or less straight forward since the layers are separated right away and then the EVM is measured on each layer using the established procedures from 38.521-1 [7].

In comparison, Method 3 for 4 layers becomes even more complex than for 2 layers, since mainly the multiple decisions that need to be taken before EVM can be calculated now need to be done for 4 layers.

Figure 2-2 Method 1 2x2 MIMO [2]

Figure 2-3 Method 1 4x4 MIMO

Figure 2-4 Method 3 2x2 MIMO [2]

Figure 2-5 Method 3 4x4 MIMO

Observation 3: Method 3 is more complex than Method 1 for 2 layer MIMO. This becomes much worse for 4 layer MIMO.

## Averaging and spectrum flatness

According to Annex F from TS 38.101-1 [3] the channel coefficients shall be interpolated using a moving average window size of 7. In [1] it is proposed to not perform any frequency smoothing, given that it is not a common procedure for EVM measurements and it may falsely improve the channel flatness.

While it may be true that in the past averaging was not used as part of the UE specification, it has been and is still used in the definition for the measurement of base station EVM without any issues [8].

Observation 4: The usage of averaging for improving the channel estimation is an established concept in 3GPP.

While for Method 1 it is be feasible to use no averaging, for Method 3 this is not the case. In Method 3, as described in [1]. According to the description the LSE channel estimation is done based on all symbols in one slot and at 4 or 6 RE granularity in frequency domain CDM group. This provides an inherit frequency smoothing which cannot be removed from this methodology. This is also visible in the graph below from [1], where no difference in the results for different averaging lengths can be seen for Method 3.

Observation 5: Method 3 inherently uses a frequency smoothing over 1 CDM group without the option to remove it.

## Channel matrix invertibility

In [1] the channel matrix invertibility is discussed in the Annex. In general, we agree with the assessment that there is for the DMRS based channel equalization (Method 1 or Method 3 step 1).

However for Method 3 in the second step both DMRS and data symbols are used for the estimation, in this case there are constellation where the invertibility is not yet clear from our side. This is the case, when e.g. UE is configured for transmissions with antenna port configurations with number of DMRS CDM groups without data = 2 (i.e. the DMRS symbols are not multiplexed with data) and DMRS antenna ports 0 and 2 are used. This may be especially critical when only few symbols are used in a slot and the transmission mode is extended to 4 layer MIMO.

Observation 6: Further clarification is needed on the matrix invertibility for Method 3.

## Conclusion

Based on our discussions above, we think that RAN4 should keep the current basic procedure for EVM UL MIMO measurements with the improvement proposed above, that all DMRS symbols shall be used for channel estimation not only the first one.

In that case, the results from Method 1 and 3 are comparable and the increased complexity from Method 3 compared to Method 1 as shown in section 2.2 is not needed.

Proposal 2: RAN4 keeps the current procedure as described in TS 38.101-1 and TS 38.101-2 with the improvement of using all DMRS symbols.

# Proposals

In this contribution, we discussed the UL MIMO EVM measurement procedures according to Method 1 and 3 and make the observations and proposals as below.

Observation 1: Current versions of TS 38.101-1 and TS 38.101-2 incorrectly capture the number of DMRS symbols to be used.

Proposal 1: Update the specification to use all available DMRS in a RMC.

Observation 2: Using all available DMRS symbols greatly improves the performance of Method 1.

Observation 3: Method 3 is more complex than Method 1 for 2 layer MIMO. This becomes much worse for 4 layer MIMO.

Observation 4: The usage of averaging for improving the channel estimation is an established concept in 3GPP.

Observation 5: Method 3 inherently uses a frequency smoothing over 1 CDM group without the option to remove it.

Observation 6: Further clarification is needed on the matrix invertibility for Method 3.

Proposal 2: RAN4 keeps the current procedure as described in TS 38.101-1 and TS 38.101-2 with the improvement of using all DMRS symbols.

# References

- R4-2302297, “EVM measurement for UL MIMO”, Keysight Technologies, RAN4#106, February 2023

- R4-2303653, “WF on UL MIMO EVM update”, Rohde & Schwarz, Keysight Technologies, Anritsu, RAN4#106, February 2023

- TS 38.101-1 v18.1.0

- TS 38.101-2 v18.1.0

- CR

- CR

- TS 38.521-1 v17.8.0

- TS 38.104