3GPP TSG-RAN WG4 Meeting #107	R4-2309481

Incheon, Korea,  May 22 – May 26,  2023

# Title:	Draft LS response on Rel-16 UL Tx Switching period

# Response to:	R4-2304008

# Release:	Release 16

# Work Item:	NR_RF_FR1-Core

Agenda Item:	    10.2.1

# Source:	Apple [to be RAN4]

# To:	RAN1

# Cc:	-

Contact Person:

Name:	Emmanuel Ngompe

E-mail Address:	e_ngompe@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN4 thanks RAN1 for the LS on Rel-16 Multi-carrier enhancement for NR.

RAN4 would like to provide a response to the following RAN1 agreement on the question of Uplink Tx Switching related interruption to the uplink and downlink transmissions when the network provided a gap between the end of the switch-from carrier’s UL transmission and the start of the switch-to carrier’s transmission that was sufficiently long to absorb the switching period

RAN1 Agreement

- If the gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period,

  - The time of no UL transmission allocated absorbs the switching period

  - Neither of the uplink transmissions (the one ending on the switch-from carrier nor the one starting on the switch-to carrier) are interrupted by the switching period.

  - The setting of uplinkTxSwitchingPeriodLocation has no impact.

  - Defer the discussion on whether/how to define the exact location of the switching period indicated by the UE capability in time domain to RAN4

    - From RAN1 point of view, for Rel-16, the implication is to the time domain location of potential interruption of downlink reception if reported by the UE for the band combination

  - Defer the potential RAN1 spec change until RAN4 has had the time to react to the RAN1 LS to RAN4.

- Send an LS to RAN4 requesting RAN4 to, in this regard, clarify TS38.101-1 subclauses 6.3A.3.3.2 and 6.3C.3.1 for CA, and SUL based UL Tx Switching, and to TS38.101-3 subclause 6.3B.4.1 for EN-DC.

Reply to RAN1 Agreement

RAN4 would prefer the switching period end to be anchored to the start of the UL allocation in the “switch to” carrier with the assumption that DL interruption will be applied for the band pair involved in the switching, as highlighted in Tables 5.2A.2.1-1, 5.2A.2.2-1, and 5.2A.2.3-1 of Release 18 specifications.

2. Actions:

To RAN1:

RAN4 respectfully requests RAN1 to take the provided information into account and update the indicated RAN1 specifications accordingly.

3. Date of Next RAN4 Meetings:

RAN4 #108	August 21st – 25th, 2023		Toulouse, France

RAN4 #108-bis	October 9th – 13th, 2023		Xiamen, China