3GPP TSG-RAN WG4 Meeting #107 	R4-2309496

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Less than 5MHz RRM requirements

Agenda item:

Document for:	Discussion

Introduction

Simulation assumptions were discussed and agreed in the last meeting [2]. Based on the assumptions, we performed simulations on DMRS detection, and MIB reading. The full set of simulation results is captured in R4-2309659 [3].

Discussion

SSB index reading simulations

The following table provides a summary of the simulations performed to derive the number of samples for SSB-index reading, and whether soft-combining needs to be assumed to meet the performance criteria for specific SNR. We present results with 99% performance criteria with single SSB detection, maximum correlation detection from multiple SSBs and non-coherent combining over 2 or 3 SSBs.

20PRB case is used as a baseline performance, and we expect to reach similar performance with less PRBs.

To be able to reach similar performance level with 12PRBs as baseline 20PRBs, combining of either 2or 3 SSBs is needed. Additionally, we note that a single SSB detection with 12-RB PBCH is sufficient at SNR of -8 dB.

| Scenario | Comments | SNR for meeting performance criteria | Scenario meeting 99% criteria |
| --- | --- | --- | --- |
| DMRS detection, AWGN, (1x2), 20PRBs | Baseline | -13 dB | Single SSB detection |
| DMRS detection, AWGN, (1x2), 12PRBs |  | -10 dB | Single SSB detection |
|  |  | -12 dB | 2 SSB combine max correlation |
|  | Meets the baseline | -14 dB | 3 SSB combine non-coherent |
| DMRS detection, TDL-A 30ns, (1x2), 3 km/h, 20 RBs | Baseline | -15 dB | Single SSB detection |
| DMRS detection, TDL-A 30ns, (1x2), 3 km/h, 12 RBs |  | -12 dB | Single SSB detection |
|  |  | -14 dB | 2 SSB combine max correlation |
|  | Meets the baseline | -15 dB | 2 SSB combine non-coherent |
| DMRS detection, TDL-B 100ns, (1x2), 3 km/h, 20 RBs | Baseline | -12 dB | Single SSB detection |
| DMRS detection, TDL-B 100ns, (1x2), 3 km/h, 12 RBs |  | -9 dB | Single SSB detection |
|  |  | -11 dB | 2 SSB combine max correlation |
|  | Meets the baseline | -12 dB | 2 SSB combine non-coherent |
| DMRS detection, TDL-C 300ns, (1x2), 3 km/h, 20 RBs | Baseline | -12 dB | Single SSB detection |
| DMRS detection, TDL-C 300ns, (1x2), 3 km/h, 12 RBs |  | -10 dB | Single SSB detection |
|  |  | -11 dB | 2 SSB combine max correlation |
|  | Meets the baseline | -12 dB | 2 SSB combine non-coherent |
| DMRS detection, HST single tap, FO = 834 Hz, (1x2), 20 RBs | Baseline | -13 dB | Single SSB detection |
| DMRS detection, HST single tap, FO = 834 Hz, (1x2), 12 RBs |  | -10 dB | Single SSB detection |
|  |  | -11 dB | 2 SSB combine max correlation |
|  | Meets the baseline | -13 dB | 3 SSB combine non-coherent |

Table 1: Summary of the DMRS detection simulations

For DMRS detection, single SSB detection is sufficient for 12-RB PBCH down to –8 dB SNR. Soft combining can be left up to UE implementation

Feasible operating point / delay is selected based on non-coherent or max correlation results.  Any implementation with similar performance is allowed.

MIB-reading reading simulations

| Scenario | Comments | SNR for meeting performance criteria | Scenario meeting 99% criteria |
| --- | --- | --- | --- |
| MIB reading, AWGN, (1x2), 20PRBs | Baseline | -9 dB | Single SSB detection |
| MIB reading, AWGN, (1x2), 12PRBs |  | -4 dB | Single SSB detection |
|  |  | -5 dB | No combining, 2 tries |
|  | Meets the baseline | -9 dB | 3 SSB combine |
| MIB reading, TDL-A 30ns, (1x2), 3 km/h, 20 RBs | Baseline | -10 dB | Single SSB detection |
| MIB reading, TDL-A 30ns, (1x2), 3 km/h, 12 RBs |  | -6 dB | Single SSB detection |
|  |  | -8 dB | No combining, 3 tries |
|  | Meets the baseline | -10 dB | 3 SSB combine |
| MIB reading, TDL-B 100ns, (1x2), 3 km/h, 20 RBs | Baseline | -8 dB | Single SSB detection |
| MIB reading, TDL-B 100ns, (1x2), 3 km/h, 12 RBs |  | -3 dB | Single SSB detection |
|  |  | -5 dB | No combining, 3 tries |
|  | Meets the baseline | -8 dB | 4 SSB combine |
| MIB reading, TDL-C 300ns, (1x2), 3 km/h, 20 RBs | Baseline | -8 dB | Single SSB detection |
| MIB reading, TDL-C 300ns, (1x2), 3 km/h, 12 RBs |  | -4 dB | Single SSB detection |
|  |  | -6 dB | No combining, 3 tries |
|  | Meets the baseline | -8 dB | 3 SSB combine |
| MIB reading, HST single tap, FO = 834 Hz, (1x2), 20 RBs | Baseline | -8 dB | Single SSB detection |
| MIB reading, HST single tap, FO = 834 Hz, (1x2), 12 RBs |  | -2 dB | Single SSB detection |
|  |  | -5 dB | No combining, 3 tries |
|  | Meets the baseline | -8 dB | 5 SSB combine |

Table 2: Summary of the MIB-reading simulations

MIB reading is impacted by PBCH detection. The simulations show the impact of MIB reading with AWGN 3km/h and 500km/h. We can observe that the soft combining can help to mitigate the issues caused by the reduced number of PRBs.

The number of SS-blocks required for 99% to successfully decode the PBCH is used as a performance metric.  PBCH detection error vs SNR for different number of SS-blocks. 1% detection error at -6 dB SNR after 10 or 5 SS-blocks.

RAN4 to discuss about MIB reading simulation results for 3km/h and 500km/h, and the impact of soft combining.

Soft combining is assumed for MIB reading.

Nokia has shown through our simulations that with the use of soft combining performance gains of approximately 6 dB can be achieved

Whilst we note that the method of achieving the performance gains achieved within these simulations is down to UE implementation, we note that soft combining through belief propagation or list decoding is core to the efficient decoding of 5G error correction schemes, such as an iterative LDPC decoder for PDSCH decoding, or indeed the processing of a number of decoding approaches for Polar Code (such as successive cancellation list decoding, or any other method described in [1]), as used within PBCH, UCI and PUCCH. Therefore it is unlikely that a UE capable of decoding LDPC or polar codes would be necessarily hardware resource constrained for soft combination/summing of MIB LLRs prior to polar decoding, or there would be constraints with the experience of these soft combination algorithms.

Furthermore, we believe that the performance should be set with a margin of 2dB from the performance observed with 7 combinations, and how the UE achieves this, is up to manufacturer implementation.

We propose RAN4 to introduce MIB requirements based on a performance achieved in the Nokia simulations.

Feasible MIB-reading delay is selected based on soft combining results. Any implementation with similar performance is allowed

Ongoing issues

SSB-based measurements

We think that a reasonable starting point for < 5MHz work is to assume SSB-based intra-frequency measurements

We suggest focusing the work on SSB based intra-frequency measurements. Hence, RAN4 will develop Intra-frequency measurement requirements for less than 5MHz only based on SSB. CSI-RS based requirements can be developed later if this is being requested. The impact on the SSB-based intra-frequency measurement requirements due to <5MHz BW would potentially be on the PBCH reading.

RAN4 to develop SSB-based intra-frequency measurement requirements for < 5MHz.The impact on the SSB-based intra-frequency measurement requirements due to <5MHz BW would potentially be on the PBCH reading.

< 5MHz RAN4 work assumes SSB-based measurements only.

RAN4 to prioritise SSB-based inter-frequency measurement requirements for < 5MHz

SSB-related requirements are prioritised over CSI-RS related requirements.

Currently a number of issues have impact due to higher bandwidth required by CSI-RS. We think that SSB-related requirements should be prioritised over CSI-RS related requirements.

The following proposal applies to the CSI-RS related issues above.

SSB-related requirements are prioritised over CSI-RS related requirements.

RLM requirements

In the issue Issue 1-7 and Issue 1-9, PDCCH transmission parameters Aggregation level (CCE) is 8 or 4 and Bandwidth (PRBs) is 24 in 8.1.2 Requirements for SSB based radio link monitoring. RAN4 can discuss whether to emphasize more reducing aggregation level due to reduced transmission bandwidth or to increase number of control OFDM symbols from 2 to 3 to mitigate the impact from the reduced transmission bandwidth.

RAN4 to discuss PDCCH transmission parameters changes (TS 38.133: 8.1.2)Requirements for SSB based radio link monitoring in rel-18. FFS whether to reduce aggregation level (CCE) or to increase the number of control OFDM symbols from 2 to 3

From the RLM requirements we observe they are based on 24PRBs. RAN4 will need to discuss how to develop RLM requirements when the BW is reduced and the currently assumed BW of 24PRB. Both in-sync and out-of-sync requirements would need discussing.

PDCCH transmission parameters for out-of-sync evaluation is impacted by the reduced bandwidth.

RAN4 to analyze how to mitigate the RLM performance loss in out-of-sync scenario.

PDCCH transmission parameters for in-sync evaluation is impacted by the reduced bandwidth.

RAN4 to analyze how to mitigate the RLM performance loss in In-sync scenario.

Summary table

We propose to analyse the impact of SSB index reading and SIB reading before proceeding to define requirements. The following table summarises our view of the open issues.

The following table summarizes our view of the issues:

| Issue # | 3MHz channel bandwidth, 12 PRB PBCH | 5MHz channel bandwidth, 20 PRB PBCH |
| --- | --- | --- |
| Issue 1-1: Impact on Idle mode Requirements | - SSB index reading - MIB reading delay | No impact |
| Issue 1-2: Impact on Inactive mode Requirements | - SSB index reading - MIB reading delay | No impact |
| Issue 1-3: Impact on Handover Requirements | - SSB index reading - MIB reading delay | No impact |
| Issue 1-4: Impact on RRC Re-establishment Requirements | - SSB index reading - MIB reading delay | No impact |
| Issue 1-5: Impact on RRC Connection Release with Redirection Requirements | - SSB index reading - MIB reading delay | No impact |
| Issue 1-7: Impact on UE RLM Requirements | - SSB reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz)) - CSI-RS - reduced BW (Current hypothetical BW for CSI-RS is 48PRBs(8.64MHz)) SSB based L1 measurements are prioritised over CSI-RS based L1-RSRP measurements | No impact |
| Issue 1-9: Impact on UE Link Recovery Procedure Requirements | Impact: reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz)) | FFS: reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz)) |
|  | Impact: reduced BW (Current hypothetical BW for CSI-RS is 48PRBs(8.64MHz)) SSB based L1 measurements are prioritised over CSI-RS based L1-RSRP measurements | Impact: reduced BW (Current hypothetical BW for CSI-RS is 48PRBs(8.64MHz)) SSB based L1 measurements are prioritised over CSI-RS based L1-RSRP measurements |
|  | Impact: reduced BW (Current hypothetical BW for SSB is 24PRBs (4.32MHz)) | No impact |
|  | Impact: reduced BW (Current hypothetical BW for CSI-RS is 48PRBs(8.64MHz)) SSB based L1 measurements are prioritised over CSI-RS based L1-RSRP measurements | No impact |
| Issue 1-13: Impact on UE L3 Intra-frequency measurements Requirements | Impact | No impact |
|  | Prioritize SSB based L3 measurements | Prioritize SSB based L3 measurements |
| Issue 1-14: Impact on UE L3 Inter-frequency measurements Requirements | Impact | No impact |
|  | SSB based L1 measurements are prioritised over CSI-RS based L1-RSRP measurements | Prioritize SSB based L3 measurements |
| Issue 1-15: Impact on UE L1-RSRP Requirements | SSB based L1 measurements are prioritised over CSI-RS based L1-RSRP measurements |  |
| Issue 1-16: Impact on UE L1-SINR Requirements | FFS if L1-SINR requirements are needed. Baseline assumption: not to define | No impact |
| Issue 1-17: Impact on Measurement performance Requirements | No impact | No impact |
|  | Wait Issue 1-13 and 1-14 | Wait Issue 1-13 and 1-14 |
|  | No impact | No impact |
|  | Impact | Impact |
| Issue 1-19: Impact on CGI | Not not to define requirements for CGI reading as a baseline | No impact |
| Issue 1-20: Impact on MIB/SIB | Impact, MIB reading is FFS and simulated. | No impact |
| Issue 1-25: HST requirements due to 12 and 20 PRB PBCH bandwidth | RAN4 will work on HST requirements for NR less than 5 MHz | FFS |

Conclusion

A couple of sentences on what was analyzed in the paper.

Observation 1: For DMRS detection, single SSB detection is sufficient for 12-RB PBCH down to –8 dB SNR. Soft combining can be left up to UE implementation

Proposal 1: Feasible operating point / delay is selected based on non-coherent or max correlation results.  Any implementation with similar performance is allowed.

Proposal 2: RAN4 to discuss about MIB reading simulation results for 3km/h and 500km/h, and the impact of soft combining.

Proposal 3: Soft combining is assumed for MIB reading.

Observation 2: Nokia has shown through our simulations that with the use of soft combining performance gains of approximately 6 dB can be achieved

Proposal 4: We propose RAN4 to introduce MIB requirements based on a performance achieved in the Nokia simulations.

Proposal 5: Feasible MIB-reading delay is selected based on soft combining results. Any implementation with similar performance is allowed

Observation 3: RAN4 to develop SSB-based intra-frequency measurement requirements for < 5MHz.The impact on the SSB-based intra-frequency measurement requirements due to <5MHz BW would potentially be on the PBCH reading.

Proposal 6: < 5MHz RAN4 work assumes SSB-based measurements only.

Proposal 7: RAN4 to prioritise SSB-based inter-frequency measurement requirements for < 5MHz

Proposal 8: SSB-related requirements are prioritised over CSI-RS related requirements.

Observation 4: In the issue Issue 1-7 and Issue 1-9, PDCCH transmission parameters Aggregation level (CCE) is 8 or 4 and Bandwidth (PRBs) is 24 in 8.1.2 Requirements for SSB based radio link monitoring. RAN4 can discuss whether to emphasize more reducing aggregation level due to reduced transmission bandwidth or to increase number of control OFDM symbols from 2 to 3 to mitigate the impact from the reduced transmission bandwidth.

Proposal 9: RAN4 to discuss PDCCH transmission parameters changes (TS 38.133: 8.1.2)Requirements for SSB based radio link monitoring in rel-18. FFS whether to reduce aggregation level (CCE) or to increase the number of control OFDM symbols from 2 to 3

Observation 5: PDCCH transmission parameters for out-of-sync evaluation is impacted by the reduced bandwidth.

Proposal 10: RAN4 to analyze how to mitigate the RLM performance loss in out-of-sync scenario.

Observation 6: PDCCH transmission parameters for in-sync evaluation is impacted by the reduced bandwidth.

Proposal 11: RAN4 to analyze how to mitigate the RLM performance loss in In-sync scenario.

Proposal 12: The following table summarizes our view of the issues:

- References

- Egilmez, Zeynep B. Kaykac, et al. "The development, operation and performance of the 5G polar codes." IEEE Communications Surveys & Tutorials 22.1 (2019): 96-122

- R4-2306400, WF on simulation assumptions for NR_FR1_lessthan_5MHz_BW, Moderator (Nokia, Nokia Shanghai Bell)

- 3GPP RAN4 Meeting #106bis-e, E-Meeting, April 17th ‒ April 26th, 2023

- R4-2309659, RRM simulation results for less than 5MHz BW, Nokia, Nokia Shanghai Bell, 3GPP RAN4 #107 Incheon, Korea, May 22 – 26, 2023