3GPP TSG-RAN WG4 Meeting #107	R4-2309675

Incheon, Korea, May 22 - 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Case 2 requirements

Agenda item:	8.10.2.3

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on open issues for Case 2 requirements as discussed at RAN4 #106bis-e [2], i.e. the combination of NCSG and concurrent MG and lists corresponding proposals.

Discussion

Agreements in [2]

- Sub-topic 4-1: Gap combinations - Issue 4-1-1: [Case 2] Measurement gaps combinations principles < Agreement >: - The previous agreements from meeting #105 and #106 on the maximum number of configured MGs still apply: - Previous agreement 1: Do not increase the max number of gaps for Case 2 (NCSG and multiple concurrent MGs) comparing to Rel-17 concurrent MG design. - Previous agreement 2: Gap combination configuration of case 2 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be NCSG or Type-2 MG. - New agreement: - For UE capable of [Concurrent NCSG], one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured and the configuration in the other FR. - UE capabilities for [Concurrent NCSG] are FFS. - For gap combination configuration of case 2, the per-UE gap in gap combination #3, #4 and #5 is associated with PRS measurement and cannot be NCSG. - FFS the wording of the notes to be added in the specification - Issue 4-1-2: [Case 2] Detail measurement gaps combinations for UE supporting per-FR gap < Agreement >: - Discuss the notes for the detail measurement gaps combinations for UE supporting per-FR gap during the CR drafting phase.

- Sub-topic 4-2: Parallel measurements for NCSG + NCSG - Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from GTW >: - Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability - Option 2: Do not support parallel measurements upon NCSGs collision - Issue 4-2-4: [Case 2] Whether the same RF chain is assumed for the two NCSG (i.e. NCSG + NCSG) patterns < Agreement >: - No need to discuss whether the same RF chain is assumed for the two NCSG patterns (not necessary). - Sub-topic 4-4: Other Rel-17 rules to be revisited - Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

Open issues in [2]

At last RAN4 #106bois-e, discussion on Case 2 requirements was continued. Following open issues are contained in [2].

Parallel measurements for NCSG + NCSG upon gap collision

RAN4 #106bis-e discussed the issue whether to consider parallel measurements upon gap collision.

- Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from GTW >: - Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability - Option 2: Do not support parallel measurements upon NCSGs collision

- For the gap combination NCSG+NCSG, parallel measurement support should be considered in the requirements, subject to UE capability. If the UE has spare receivers, the network can assign also overlapping NCSG patterns to speed up e.g. measurement of deactivated SCells. Thus, we support option 1.

Support of parallel measurements upon NCSGs collision is up to UE capability.

Parallel measurements for NCSG + NCSG - scenarios

RAN4 #106bis-e discussed the following issue.

- Issue 4-2-2: [Case 2] Whether to support parallel measurements in the following scenarios for two NCSG < Way forward >: - FFS the following options Scenario 1: NW only configures deactivated SCells’ measurement - Option 1: - No. - Option 2: - Yes. Scenario 2: NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’ - Option 1: - No. - Option 2: - Yes. Scenario 3: NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands - Option 1: - No. - Option 2: - Yes.

Regarding issue 4-2-2, whether to support parallel measurements in the above scenarios for two NCSGs, we consider all three scenarios as meaningful and hence these should be considered as use case scenarios for NCSG+NCSG support.

Consider all three scenarios, i.e. scenario 1 (NW only configures deactivated SCells’ measurement), scenario 2 (NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’) and scenario 3 (NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands) for parallel measurement support for NCSG+NCSG.

Parallel measurements for NCSG + NCSG – support in general way

RAN4 #106bis-e discussed the following issue.

- Issue 4-2-3: [Case 2] Whether to support parallel measurements in a general way for two NCSGs < Way forward>: - FFS the options: - Option 1: - RAN4 to study a general solution to allow both NW and UE to know the parallel measurements combination when UE supports NCSG parallel measurement capability.

There was no consensus on option 1. The proponent clarified that this issue is about whether to allow both NW and UE knowing which bands X and Y can perform parallel measurements in the RRC reconfiguration together with serving cell operation. In our view, this is a valid issue for signalling support, in case parallel measurement support is agreed in issue 4.2.1 based on two or more spare RF chains. Thus, we propose to keep this FFS for now, as this is also a RAN2 signalling issue.

Whether to support parallel measurements for two NCSGs for a combination of measurements on band X and Y together with serving cell operation should be studied further once issue 4.2.1 on parallel measurement support for NCSG is agreed.

Parallel measurements for NCSG + NCSG – new UE capability

Regarding issue 4-1-1 the discussion at RAN4 #106bis-e was not conclusive

- Issue 4-2-5: [Case 2] Whether to consider a new capability for NCSG + NCSG in an FR < Way forward >: - FFS the following options - Option 1a: - No, without UE capability if two NCSG patterns are supported based on single RF chain. - Option 2: - Yes, with UE capability - Option 2a: - Yes, with UE capability if two RF chains are in use to support NCSG+NCSG, we are fine to introduce new UE capability - Option 3: - Postpone the discussion on new capability for NCSG + NCSG until RAN4 has a consensus on parallel measurement.

As mentioned during RAN4 #106bis-e, we support options 1a and 2a. Only in case of indicated NCSG support and two or more spare RF chains this capability is needed. For option 2a, whether this capability is specified for a combination of bands X and Y, as discussed under issue 4-2-3 can be FFS. No additional capability is needed in case of NCSG support and 1 spare RF chain.

For issue 4-2-5, options 1a and 2a are supported. For option 2a, whether this capability is specified for a combination of bands X and Y, as discussed under issue 4-2-3 can be FFS.

Collision handling – Potential changes to UE behaviour upon gap collision

RAN4 #106bis-e discussed the issue of collision handling and the potential changes due UE behaviour upon gap collision for Case 2 requirements.

- Sub-topic 4-3: collision handling - Issue 4-3-1: [Case 2] Potential changes to UE behaviour upon gap collision < Way forward >: RAN4 to postpone the gap collision rule changes discussion until RAN4 has a consensus on parallel measurement.

- Regarding issue 4-3-1, on potential changes to UE behaviour upon gap collision, a case distinction needs to be done

- for UE supporting parallel measurements and

- UE not supporting parallel measurements.

- Case a:

- UE needs to adopt Rel-17 priority rules for concurrent measurements and drop either the MG occasion or any of the configured NCSG occasions for the gap combinations (concurrent MG + NCSG) and (NCSG+NCSG), depending on its configured lower priority level.

- Case b:

- UE identifies overlapping in time of MG and NCSG occasion(s), i.e. for the gap combinations (concurrent MG + NCSG, case b1) and (NCSG+NCSG, case b2).

- Case b1:

- A concurrent MG pattern and a NCSG pattern are configured with partial overlap. For instance, the concurrent MG occasion is ahead of the NCSG occasion as shown in Figure 1.

Figure 1: Partial overlap of concurrent MG and NCSG occasions

- Then, UE cannot start VIL1 of NCSG occasion, performing retuning, during the concurrent MG occasion, when measuring or retuning to intra-/inter-frequency layers. It may start VIL1 immediately after concurrent MG occasion, or it may start VIL1 even prior to concurrent MG occasion, so that it does not overlap with concurrent MG occasion, i.e. its MGL, in which case ML can start immediately after concurrent MG occasion. Likewise, if NCSG occasion is ahead of concurrent MG, UE cannot start VIL2 of NCSG occasion, performing retuning, during the concurrent MG occasion, when measuring or retuning to intra-/inter-frequency layers. It is required to end ML and VIL2 prior to concurrent MG occasion or at least ML needs to end prior to concurrent MG occasion and VIL2 may be shifted to after concurrent MG occasion.

If UE supports parallel measurements for concurrent MG+NCSG, in case of collision of concurrent MG and NCSG measurement occasions, UE performs NCSG measurement (ML) prior to or right after concurrent MG occasion and shifts VIL2 right after concurrent MG occasion or VIL1 just prior to MG occasion, respectively.

- Case b2:

- Two NCSG patterns (NCSG1 and NCSG2) are configured with partial overlap, that is NCSG1 occasion is ahead of NCSG2 occasion as shown in Figure 2.

Figure 1: Partial overlap of two NCSG occasions

- Then, UE cannot start VIL1 of NCSG2 occasion, performing retuning, when measuring with NCSG1 during ML. Likewise, UE cannot start VIL2 of NCSG1 occasion, when measuring with NCSG2 during ML. Thus, UE may, upon detection of NCSG collision, need to align VIL1 and VIL2 of both NCSG occasions so that they are outside both ML intervals. The network is aware of the NCSG collision and will not schedule the UE in the time-aligned VIL1 periods and VIL2 periods, respectively.

If UE supports parallel measurements for NCSG+NCSG, in case of collision of NCSG measurement occasions, UE performs time alignment of VIL1 periods and VIL2 periods, respectively.

In both cases b1 and b2, it is assumed that no proximity requirement exists. Hence the UE supporting parallel measurement support for Case 2 requirements supports proximity 0 ms.

If UE supports parallel measurements, proximity is zero in case of collision of MG and/or NCSG occasion(s).

NCSG changes upon SCell activation

Following issue was left open at RAN4 #106bis-e related to NCSG changes for SCell activation case.

- Sub-topic 4-4: Other Rel-17 rules to be revisited - Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

In our view, a case distinction is needed based on scenarios. Option 1, i.e. legacy UE behaviour applies for a UE if the NCSG does not cover the SMTC of the deactivated SCell’s MO or NCSG is not being configured by the network, whilst option 2 applies for a configured NCSG upon SCell deactivation and SMTC of deactivated SCell’s MO is at least partially covered by NCSG. However, as mentioned during RAN4 #106bis-e, this issue is also relevant for Rel-17 MGE, if NSCG is configured for measuring deactivated SCells. Hence a solution for Rel-17 needs to be defined first.

Upon SCell deactivation, in case of configured NCSG, the deactivated SCell’s MO will be measured within NCSG if the SMTC of the deactivated SCell’s MO is partially or fully overlapped, else it will be measured gapless (legacy UE behaviour). This issue is also relevant for Rel-17 MGE, when NSCG is configured for measuring deactivated SCells. Hence a solution for Rel-17 needs to be defined first.

Gap interruption requirements

Regarding issue 4-5-1, it is about gap interruption requirements for Case2. However, it was already agreed at RAN4 #106, which defines whether a slot of the victim cell is interrupted or not.

## Issue 4-5-1: [Case 2] Gap interruption < Way forward >: - FFS the options: - Option 1: - The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as: Tint_ncsg+Tint_other-Toverlap, wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length. - Option 2: - Over optimization on the interruption length is not necessary. The FFS part in option 1 in this issue could be stopped without any conclusion.

This considers both cases, i.e. non-overlapping and overlapping of NCSG and MG. Thus, we do not see a need to define a further requirement and support option 2.

No further gap interruption requirement for Case 2 is needed beyond that one agreed at RAN4 #106.

Conclusion

This paper has presented Nokia’s views on Case 2 requirements as discussed at RAN4 #106bis-e [2], i.e. the combination of NCSG and concurrent MG.

Resulting from this discussion, we make following proposals:

- Support of parallel measurements upon NCSGs collision is up to UE capability.

- Consider all three scenarios, i.e. scenario 1 (NW only configures deactivated SCells’ measurement), scenario 2 (NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’) and scenario 3 (NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands) for parallel measurement support for NCSG+NCSG.

- Whether to support parallel measurements for two NCSGs for a combination of measurements on band X and Y together with serving cell operation should be studied further once issue 4.2.1 on parallel measurement support for NCSG is agreed.

- For issue 4-2-5, options 1a and 2a are supported. For option 2a, whether this capability is specified for a combination of bands X and Y, as discussed under issue 4-2-3 can be FFS.

- If UE supports parallel measurements for concurrent MG+NCSG, in case of collision of concurrent MG and NCSG measurement occasions, UE performs NCSG measurement (ML) prior to or right after concurrent MG occasion and shifts VIL2 right after concurrent MG occasion or VIL1 just prior to MG occasion, respectively.

- If UE supports parallel measurements for NCSG+NCSG, in case of collision of NCSG measurement occasions, UE performs time alignment of VIL1 periods and VIL2 periods, respectively.

- If UE supports parallel measurements, proximity is zero in case of collision of MG and/or NCSG occasion(s).

Upon SCell deactivation, in case of configured NCSG, the deactivated SCell’s MO will be measured within NCSG if the SMTC of the deactivated SCell’s MO is partially or fully overlapped, else it will be measured gapless (legacy UE behaviour). This issue is also relevant for Rel-17 MGE, when NSCG is configured for measuring deactivated SCells. Hence a solution for Rel-17 needs to be defined first.

No further gap interruption requirement for Case 2 is needed beyond that one agreed at RAN4 #106.

- References

- RP-230755, Revised WID: Further enhancements on NR and MR-DC measurement gaps and measurements without gaps, Intel, MediaTek

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek