3GPP TSG-RAN WG4 Meeting #107	R4-2309705

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.13.4.2

Source: 	Qualcomm Incorporated

Title: 	Intra-band CA requirements for FR2 HST

Document for:	Discussion

# Introduction

PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements

- The enhanced FR2 HST requirements for intra-frequency detection in Rel-17 FR2 HST WI are also applied for SCells inter-frequency cell detection if introduced.

  - FFS RAN4 need to specify the requirements for inter-frequency PSS/SSS detection and

  - FFS RAN4 need to specify the requirements for time index detection.

SCell activation delay

- Check SCell activation delay and applicable conditions in HST scenario.

RAN4 has been discussing the intra-band CA requirements for FR2 HST for past few meetings and the WF [1] from RAN4#106bis-e lists the latest agreements and open issues. Some of the agreements from the last meeting are listed below:

In this contribution, we provide our views on the above-mentioned open issues.

# Intra-band CA requirements for FR2-HST UE

Regarding which requirements are needed to support intra-band CA operation, we think only inter-frequency measurement requirements are needed. Intra-frequency PSS/SSS detection and intra frequency measurement requirements are already specified for the PCell and the same shall apply to SCell as well. However, inter frequency cell detection and measurement requirements are missing from the spec which are needed to support intra-band CA for FR2-HST.

Proposal 1: RAN4 to specify inter-frequency cell identification (PSS/SSS detection) and measurement requirements to support intra-band CA for FR2-HST.

Since deriveSSB-IndexFromCell is always enable for FR2, SSB index detection requirements are not needed for intra-frequency CA as PCell timing can be used to acquire the SCell timing in this case. But SSB time index detection requirements are still needed for inter-frequency CA.

Proposal 2: RAN4 to specify inter-frequency SSB index identification requirements to support intra-band CA for FR2-HST. Intra-frequency SSB index identification requirements are not needed.

For exact inter-frequency requirements, we can follow the same principles that were used to specify FR2-HST requirements in Release 17 – use beam-sweeping scaling factor of 2 and 6 for highSpeedMeasFlagFR2-r17 = set1 and highSpeedMeasFlagFR2-r17 = set2, respectively.

Proposal 3: RAN4 to use Rel-17 FR2-HST principles to define inter-frequency cell detection, SSB index identification and measurement requirements, i.e., use beam-sweeping scaling factor of 2 and 6 for set1 and set2, respectively.

# Conclusion

Proposal 1: RAN4 to specify inter-frequency cell identification (PSS/SSS detection) and measurement requirements to support intra-band CA for FR2-HST.

Proposal 2: RAN4 to specify inter-frequency SSB index identification requirements to support intra-band CA for FR2-HST. Intra-frequency SSB index identification requirements are not needed.

Proposal 3: RAN4 to use Rel-17 FR2-HST principles to define inter-frequency cell detection, SSB index identification and measurement requirements, i.e., use beam-sweeping scaling factor of 2 and 6 for set1 and set2, respectively.

# References

[1] R4-2306341, “WF on intra-band CA for FR2 HST PC6 UE”, Ericsson, RAN4#106bis-e