# 3GPP TSG-RAN WG4 Meeting # 107	R4-2310080

# Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3240 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for R18 RRM enhancement - FR1+FR1 NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh3-Core |  |  |  |  |  | Date: |  |  | 2023-05-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is to approve formal CR R18 RRM enhancement - FR1+FR1 NR-DC. The draft Big CR R4-2306329 has been endorsed including the changes in following draft CRs in RAN4#106-bis-e. Tdoc number; Title; Source; Note. R4-2304380; DraftCR applicability Rule for FR1+FR1; Qualcomm Incorporated; endorsed. R4-2305057; Draft CR on PSCell addition/release delay for FR1+FR1 NR-DC; vivo; endorsed. R4-2305236; Draft CR on CSSF requirements for FR1+FR1 NR-DC; OPPO; endorsed. R4-2306323; DraftCR Scheduling availability for FR1+FR1; Qualcomm Incorporated; endorsed. R4-2306324; Draft CR for FR1-FR1 NR-DC on scheduling availability of UE during candidate beam detection; MediaTek inc.; endorsed. R4-2306325; DraftCR on scheduling availability during L1-RSRP measurement for FR1-FR1 NR-DC; Nokia, Nokia Shanghai Bell; endorsed. R4-2306326; Draft CR on HO with PSCell requirements for FR1-FR1 NR-DC; Xiaomi; endorsed. R4-2306327; draft CR for Rel-18 eFeRRM NR-DC FR1-FR1 SCG activation; Ericsson; endorsed. R4-2306328; Draft CR on scheduling availability of UE during intra-frequency measurements for FR1+FR1 NR-DC; OPPO; endorsed. |
| Summary of change: |  | Changes endorsed RAN4#106-bis-e: Tdoc; Summary of change; Note. R4-2304380; Add requirements for number of serving carriers for FR1+FR1.. R4-2305057; • Change#1 Specify the number of serving carriers for FR1+FR1 NR-DC • Change#2 Specify the PSCell addition delay requirement for FR1+FR1 NR-DC scenario and the corresponding values of Tprocessing and Tsearch in this scenario.. R4-2305236; Introduce requirements for FR1+FR1 NR-DC scenarios: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps and within gaps.. R4-2306323; Add requirements of RLM/BFD measurement for inter-band FR1+FR1 NR-DC sceanrio.. R4-2306324; Updating clause 8.5.8.2 of 38.133 to capture scheduling availability requirements for FR1-FR1 NR-DC of UE during candidate beam detection.. R4-2306325; Introduce Scheduling availability of UE during L1-RSRP measurement for FR1+FR1 NR-DC according to the agreed WF R4-2303304 in RAN4#106 meeting.. R4-2306326; Introduce HO with PSCell requirements for FR1-FR1 NR-DC.. R4-2306327; Change 1: Editory change Change 2: FR1 RACH based Unknown PScell with Es/Iot ≥ -2 dB side condtion Change 3: FR1 RACH-less activation Change 4: add FR1 in known definition. R4-2306328; Introduce scheduling restriction requirements of UE during intra-frequency measurements for FR1+FR1 NR-DC scenarios. |
| Consequences if not approved: |  | The RRM requirements for FR1-FR1 NR-DC will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6.2.4, 6.1.5.1, 6.1.5.4, 8.1.7, 8.2.4, 8.5.7, 8.5.8, 8.9, 8.17, 9.1.5, 9.2.5.3.1, 9.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2308484 |  |  |  |  |  |  |  |  |

# <Start of Change 1>

3.6.2.4	Number of serving carriers for NR-DC

Requirements for NR-DC (FR1+FR1) are applicable for the UE configured with the following number of serving NR CCs:

-	up to [6] NR DL CCs in total, with [1] UL in PCell, [1] UL in PSCell, [1] UL in SCell(s).

Requirements for NR-DC (FR1+FR2) are applicable for the UE configured with the following number of serving NR CCs:

-	up to 2 NR DL CCs in total in FR1, up to 8 NR DL CCs in total in FR2, with 1 UL in PCell, 1 UL in PSCell, and up to 1 UL in each SCell.

# <End of Change 1>

# <Start of change 2>

8.2.4	NR-DC: Interruptions

8.2.4.1	Introduction

This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when

up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated when UE is operating in NR-DC (FR1+FR2) or,

up to [4] SCell(s) in FR1 are configured, deconfigured, activated or deactivated when UE is operating in NR-DC (FR1+FR1) or,

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

measurements on the deactivated PSCell in NR SCG, or

UL/DL BWP is switched on PCell, PSCell or SCell,

UE-specific CBW is changed on PCell, PSCell or SCell, or

transitions between active and non-active during DRX, or

transitions from non-DRX to DRX, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

NR SRS carrier based switching, or

NR SRS antenna port switching.

RLM/BFD Measurement on deactivatd NR PSCell, or

NR SCell is activated based on aperiodic CSI-RS.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

For a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

# <End of change 2>

# <Start of change 3>

8.9	NR-DC: PSCell Addition and Release Delay

8.9.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to configure an PSCell in NR dual connectivity. The requirements are applicable to an NR dual connectivity capable UE.

8.9.2	PSCell Addition Delay Requirement

The requirements in this clause shall apply for the UE configured with only PCell in FR1.

Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in subframe n + Tconfig_PSCell Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in slot n+Tconfig_PSCellNR slot length:

where:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms when target cell is in FR1 and Tprocessing = 40 ms when target cell is in FR2.

Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 3*N* Trs ms. N = 1 when target cell is in FR1, N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In FR1 and FR2, the PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the PSCell configuration command:

-	the UE has sent a valid measurement report for the PSCell being configured and

-	One of the SSBs measured from the PSCell being configured remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being configured also remains detectable during the PSCell configuration delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

# <End of change 3>

# <Start of change 4>

# <<  Start of text proposal 1>>

### 9.1.5	Carrier-specific scaling factor

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps

For UE configured with NR-DC operation, the carrier-specific scaling factor CSSFoutside_gap,i for intra-frequency SSB-based measurement, inter-frequency SSB-based measurements performed outside measurements gaps and intra-frequency CSI-RS based L3 measurement will be as specified in Table 9.1.5.1.3-1.

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | N/A | 2x(1+ NPSCC_CSIRS) Note 2 | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |
| FR1 + FR1 NR-DC (FR1 pCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |
| Note 1: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. Note 2: CSSFoutside_gap,i =1 if no SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on PSCC; CSSFoutside_gap,i =2 if no SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on PSCC. Note 3: Y is the number of configured inter-frequency SSB based frequency layers without MG that are being measured outside of MG; otherwise, it is 0. Note 4: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 5: NPSCC_CSIRS=1 if PSCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPSCC_CSIRS =0. Note 6: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. |  |  |  |  |  |  |

<< End of text proposal 1>>

<<  Start of text proposal 2>>

##### 9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning frequency layer i with Tavailable_PRS,i >160ms, where Tavailable_PRS,i is defined in clauses 9.9.2.5, 9.9.3.5 and 9.9.4.5 for RSTD, PRS-RSRP and UE Rx-Tx time difference measurements, respectively.

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

When multiple positioning frequency layers are configured,

-	for each positioning frequency layer i, CSSFwithin_gap,i is derived with the following steps assuming no other positioning frequency layer is configured.

-	for each RRM frequency layer i, CSSFwithin_gap,i is derived as follows:

-	an intermediate CSSFwithin_gap,i,k is derived with the following steps assuming only positioning frequency layer k is configured, and

-	CSSFwithin_gap,i= max(CSSFwithin_gap,i,k), where k=0…K-1, and K is the number of configured positioning frequency layers.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and NR PRS measurements on all positioning frequency layers which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An NR measurement object with CSI-RS measurement configured is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	A positioning frequency layer is counted as candidate for a MG occasion if at least one PRS resource on that positioning frequency layer is fully covered by the MGL excluding RF switching time.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects and up to one NR PRS measurement on any one positioning frequency layer belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : Number of NR inter-frequency layers including both SSB and CSI-RS based, EUTRA inter-RAT and UTRA inter-RAT measurement objects and up to one positioning frequency layer, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects of MCG belong to group A

FR1 intrafrequency measurement objects of SCG belong to group B for FR1+FR1 NR-DC, or FR2 intrafrequency measurement objects of SCG belong to group B for FR1+FR2 NR-DC.

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0. The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

<< End of text proposal 2>>

# <End of change 4>

# <Start of Change 5>

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to radio link monitoring performed with a same subcarrier spacing as PDSCH/PDCCH on FR1.

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM -RS.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

When intra-band carrier aggregation in FR1 is performed, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols. When inter-band carrier aggregation within FR1 is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

# <End of Change 5>

# <Start of Change 6>

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to beam failure detection performed on SSB and CSI-RS configured for BFD with the same SCS as PDSCH or PDCCH in FR1.

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to beam failure detection when SSB is configured as BFD. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to beam failure detection when SSB is configured as BFD.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for beam failure detection.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell or PSCell is configured.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in the bands due to beam failure detection performed on FR1 serving PCell or PSCell in different bands.

# <End of Change 6>

# <Start of Change 7>

8.5.8	Scheduling availability of UE during candidate beam detection

Scheduling availability restrictions when the UE is performing L1-RSRP measurement for candidate beam detection are described in the following clauses.

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as link recovery detection resource with the same SCS as PDSCH or PDCCH in FR1.

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as link recovery detection resource. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured as link recovery detection resource.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, TRS, CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for L1-RSRP.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on one serving cell apply to all other serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in one band due to candidate beam detection performed on FR1 serving cell(s) in other bands.

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to candidate beam detection

-	For FR2-1 or the reference symbols to be measured for candidate beam detection is not using 480 kHz SCS or 960 kHz SCS on FR2-2, the UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, CSI-RS for tracking or CSI-RS for CQI on reference symbols to be measured for candidate beam detection.

-	For FR2-2 and the reference symbols to be measured for candidate beam detection is using 480 kHz SCS or 960 kHz SCS, the UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH, CSI-RS for tracking or CSI-RS for CQI on reference symbols to be measured for candidate beam detection, and on one data symbol before each reference symbol to be measured for candidate beam detection and one data symbol after each reference symbol to be measured for candidate beam detection.

When intra-band carrier aggregation in FR2 is configured, the scheduling restrictions on to one serving cell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to candidate beam detection performed on FR2 serving cell(s) in different band(s), provided that the FR2 serving cell(s) and the FR2 serving cell(s) for candidate beam detection are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

For FR2, if following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for CBD mesurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for CBD mesurement.

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC

There are no scheduling restrictions on FR1 serving cell(s) due to L1-RSRP measurement performed on FR2 serving cell(s).

There are no scheduling restrictions on FR2 serving cell(s) due to L1-RSRP measurement performed on FR1 serving cell(s).

# <End of Change 7>

# < Start of change 8>

### 9.5.6	Scheduling availability of UE during L1-RSRP measurement

Scheduling availability restrictions described in the following clauses apply when the UE is performing L1-RSRP measurement on serving cell, and UE is receiving PDCCH/PDSCH from serving cell and/or cell(s) with different PCI.

#### 9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

#### 9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on serving cell where L1-RSRP measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured.

When inter-band FR1+FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured.

#### 9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	For the case where RS for L1-RSRP measurement is CSI-RS which is QCLed with active TCI state for PDCCH/PDSCH and not in a CSI-RS resource set with repetition ON, and N=1 applies as specified in clause 9.5.4.2

-	There are no scheduling restrictions due to L1-RSRP measurement performed based on the CSI-RS.

-	Otherwise

-	In non-HST scenario, for FR2-1 or the reference symbols to be measured for L1-RSRP is not using 480 kHz SCS or 960 kHz SCS on FR2-2, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on

-	symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and/or

-	symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement, and/or

-	symbols corresponding to the semi-perssitent CSI-RS resource configured for L1-RSRP measurement when the resource is activated, and/or

-	symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement when the reporting is triggered.

-	For FR2-2 and the reference symbols to be measured for L1-RSRP is using 480 kHz SCS or 960 kHz SCS, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on

-	symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and on one data symbol before and one data symbol after the symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and/or

-	symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement, and on one data symbol before and one data symbol after the symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement, and/or

-	symbols corresponding to the semi-perssitent CSI-RS resource configured for L1-RSRP measurement when the resource is activated, and on one data symbol before and one data symbol after the symbols corresponding to the semi-perssitent CSI-RS resource configured for L1-RSRP measurement when the resource is activated, and/or

-	symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement when the reporting is triggered, and on one data symbol before and one data symbol after the symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement when the reporting is triggered.

-	In HST scenario, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on

-	symbols corresponding to the SSB indexes configured for L1-RSRP measurement and 1 data symbol before each consecutive SSB symbols to be measured for L1-RSRP and 1 data symbol after each consecutive SSB symbols to be measured for L1-RSRP, and/or

-	symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement and 1 data symbol before each periodic CSI-RS resource to be measured for L1-RSRP and 1 data symbol after each periodic CSI-RS for L1-RSRP measurement symbols to be measured for L1-RSRP, and/or

-	symbols corresponding to the semi-persistent CSI-RS resource configured for L1-RSRP measurement and 1 data symbol before each semi-persistent CSI-RS resource to be measured for L1-RSRP and 1 data symbol after each semi-persistent CSI-RS resource to be measured for L1-RSRP when the resource is activated, and/or

-	symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement and 1 data symbol before each aperiodic CSI-RS resource to be measured for L1-RSRP measurement and 1 data symbol after each aperiodic CSI-RS resource to be measured for L1-RSRP measurement when the reporting is triggered.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions on serving cell where L1-RSRP measurement is performed apply to all serving cells in the band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to L1-RSRP measurement performed on FR2 serving cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement.

#### 9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 serving cell(s) due to L1-RSRP measurement performed on FR2 serving cell(s).

There are no scheduling restrictions on FR2 serving cell(s) due to L1-RSRP measurement performed on FR1 serving cell(s).

# < End of change 8>

# < Start of change 9>

#### 6.1.5.1	Introduction

The purpose of NR handover with PSCell is to change the NR PCell to another NR cell or E-UTRA cell and add or change the PSCell along with PCell handover. The requirements in this clause are applicable to:

-	Handover with PSCell from NR SA to EN-DC

-	Handover with PSCell from NR-DC to NR-DC

-	Handover with PSCell from NE-DC to NE-DC

-	Requirements in this clause only applies to NE-DC with FR1 PCell

# < End of change 9>

# < Start of change 10>

#### 6.1.5.4	HO with PSCell from NR-DC to NR-DC

The requirements in this clause are applicable to handover with PSCell from NR-DC to NR-DC. The requirements in this clause are applicable to

-	FR1-FR1 NR-DC to FR1-FR1 NR-DC,

-	FR1-FR1 NR-DC to FR1-FR2 NR-DC,

-	FR1-FR2 NR-DC to FR1-FR1 NR-DC,

-	FR1-FR2 NR-DC to FR1-FR2 NR-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from NR cell to NR cell and add NR PSCell in the meantime.

When the UE receives a RRC message implying handover with PSCell,

-	The UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within DHOwithPSCell_PCell ms from the end of the last TTI containing the RRC command, and

-	The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than DHOwithPSCell_PSCell ms from the end of the last TTI containing the RRC command.

Where:

-	DHOwithPSCell_PCell equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.5.4.1.

-	DHOwithPSCell_PSCell is the PSCell change delay stated in clause 6.1.5.4.2.

##### 6.1.5.4.1	HO with PSCell – PCell Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When handover with PSCell from NR-DC to NR-DC is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

-	Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period.

For FR1-FR1 NR-DC to FR1-FR1 NR-DC or FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

For FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover.

##### 6.1.5.4.2	HO with PSCell – PSCell change delay

The requirements in this section shall apply for PSCell change during handover with PSCell from NR DC to NR-DC.

When handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DHOwithPSCell_PSCell:

-	DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms

Where:

-	TRRC_delay Tprocessing, Tsearch_PSCell, T∆ and TPSCell_ DU are the same as defined in clause 8.9.2.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period.

For FR1-FR1 NR-DC to FR1-FR1 NR-DC or FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

For FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

-	Tsearch_PCell is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell = Tsearch + TΔ + Tmargin, where Tsearch, TΔ and Tmargin are specified in clause 6.1.5.4.1. Otherwise, Tsearch_PCell = 0 ms.

The Trs definition from clause 8.9.2 is modified as following for requirements in this section:

-	Trs is the SMTC periodicity of the target NR cell if target PSCell is unknown and SMTC configuration of target unknown PSCell is present in either targetcellSMTC-SCG-r16 or reconfigurationWithSync, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

PSCell known and unknown condition is as defined in clause 8.9.2.

# < End of change 10>

# < Start of change 11>

8.17	SCG Activation and Deactivation Delay

8.17.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to activate one SCG and deactivate on SCG.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

8.17.2	SCG Activation Delay Requirement

The requirements in this clause shall apply for the UE configured with one deactivated SCG in NR-DC and when PScell in one SCG is being activated.

The delay within which the UE shall be able to activate the deactivated SCG depends upon the specified conditions.

Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot length ,

where:

Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms.

Tsearch is the time for AGC settling and PSS/SSS detection.

For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

For RACH-less based PSCell activation, if [RLM and BFD are configured] and TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 or FR1 PScell. [There are no requirements if PSCell is unknown, or TCI state is unknown].

T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms.

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH].

Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

In FR1 and FR2, the PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the SCG activation command:

-	the UE has sent a valid measurement report for the PSCell being activated and

-	One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3.

-	One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tactivation_time according to the cell identification conditions specified in clause 9.3.

otherwise it is unknown.

If the UE is configured to perform BFD while the SCG is deactivated

-	The TCI state is known if the following conditions are met:

-	During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated,

-	UE has not detected beam failure

-	Otherwise, the TCI state is unknown.

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

8.17.3	SCG Deactivation Delay Requirement

The requirements in this clause shall apply for a UE which is configured with at least PCell and PScell.

Upon receiving RRC-based SCG deactivation command in subframe n, the UE shall accomplish the deactivation actions specified in TS 38.331 [2] no later than in slot n+TRRC_delayNR slot length:

where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

# < End of change 11>

# < Start of change 12>

##### 9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

When inter-band FR1+FR1 NR-DC is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

# < End of change 11>