3GPP TSG-RAN WG4 Meeting # 107																R4-2310092

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.27.6

Source:	Moderator (Qualcomm Incorporated)

Title:	WF on NR NTN enhancements RRM requirements

Document for:	Approval

# Introduction

The WF covers the contributions submitted under the following AI

- 8.27.5	RRM core requirements

# Coverage enhancement

- No agreement.

# NR-NTN deployment in above 10 GHz

### Issue 2-1: UE UL Timing Accuracy Requirements for UL SCSs of 60kHz and 120kHz

Agreement:

- The assumption of the maximum total positioning error due to UE location and Satellite position estimation error shall be tightened as below:

  - For UL SCS of 60kHz, [X] meters.

  - For UL SCS of 120kHz, [Y] meters

  - The above is applicable only when SSB SCS is equal to or higher than 120kHz

  - FFS on whether applicable to UE in mobile platform.

  - FFS on whether and how to connect the tightened UE positioning error to the advanced GNSS capability or UE type.

  - FFS whether to use different requirements for different types of devices defined in the RF session and/or different satellite types

### Issue 2-2: Measurement and Mobility Requirements in NR-NTN above 10 GHz bands

- No agreement.

# Network verified UE location

Agreement:

- FFS: RAN4 to define measurement period and accuracy requirements for UE Rx-Tx measurement

  - Existing TN requirements can be used as baseline, e.g. 9.9.4.6 Measurement Period Requirements without Measurement Gaps

  - NTN specific aspects including necessary side conditions, if any, will be further discussed based on RAN1 progress

# NTN-TN and NTN-NTN cell reselection enhancements

### Issue 4-1-A: Time-based cell reselection in earth moving cell NTN deployments

- No agreement.

Issue 4-1-B: For location-based cell reselection in earth moving cell NTN deployments

Agreement:

- RAN4 to define UE requirements on location-based cell reselection in earth moving cell NTN deployments if the existing requirements are not applicable for earth moving system. The new requirement can be based on the existing requirement, and the updates may include “the margin for distance threshold” and “the definition of the reference Location.”

  - Do not further restrict DRX cycle beyond Rel-17 NR NTN.

  - FFS: Whether the coverage information of serving cell is (absolutely) necessary

### Issue 4-1-C: whether and how to manipulate Kcarrier in Ttrigger

- No agreement.

### Issue 4-2: NTN-TN Cell reselection enhancements (to reduce UE power consumption)

Agreement:

- FFS: RAN4 to define RRM requirements on TN-to-NTN reselection.

  - RAN4 to agree to reuse same side condition of assistance information for neighbour NTN cells from R17 WF R4-2207114.

  - Additional side conditions can be added, as needed, and the conditions may depend on satellite types (GSO vs. non-GSO) and/or NTN cell deployments for non-GSO (earth-fixed vs. earth-moving cell).

Agreement:

- RAN4 to define RRM requirements on NTN-to-TN cell reselection requirement without measurement enhancement for the following cases:

  - both GSO and non-GSO

  - both earth fixed cell and earth moving cell

  - FFS on

    - whether/how to define the detailed UE behaviour when the UE is out of TN coverage.

    - how to define TN coverage for both TN band is or is not shared with NTN band

Agreement:

- RAN4 to not define accuracy of TN coverage information provided by NTN cell.

### Issue 4-3: NTN-NTN Cell reselection enhancements (to reduce UE power consumption)

- No agreement.

# NTN-NTN handover enhancements

### Issue 5-1: RACH-less (C)HO

Agreement:

- RAN4 to define RACH-less HO requirements for intra-/inter-satellite handover with and without gateway/gNB switch.

  - Known and Unknown target cell condition:

    - Reuse the R17 known/unknown condition applied in NTN HO requirements.

    - If any other new aspects to be additionally added are identified based on RAN1/2 progress, those aspects will be further discussed in RAN4.

  - Whether to define the requirements for both known and unknown target cells

    - Consider both known and unknown target cells.

  - Whether to include fine time tracking latency in the HO latency requirement,

    - Fine time tracking latency will be included even when target cell is known.

  - Additional latency can be added, if identified and needed, based on the detailed design of the feature to be made by RAN1 and RAN2

- The detailed formula and exact values in the requirements, e.g. T_IU, T_SI-NR, T_RACH, T_delta, T_processing, T_margin, T_search, etc. if relevant, will be defined in the next meeting accordingly.

### Issue 5-2: HO for satellite switching without PCI change

- No agreement.

### Issue 5-3: Group-based HO for signalling overhead reduction

- No agreement. Wait for further progress from RAN2.

### Issue 5-4: Location based CHO enhancements for earth moving NTN cell

- No agreement. Wait for further progress from RAN2.

# References

[1] R4-2309971, “Topic summary for [107][226] NR_NTN_enh,” 3GPP TSG-RAN WG4 Meeting #107