3GPP TSG-RAN4 Meeting #107	R4-2310252

# Incheon, KR, May 22th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5981 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of 5G broadcast UHF bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, SWR, EBU, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2-Core |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of bands in the UHF frequency range to support 5G broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | System parameters for 5G broadcast in bands 107 and 108 are introduced. Suffix H is added for 5G broadcast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bands are not available for the deployment of 5G broadcast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.3A, 5.5H, 5.6H, 5.6H.1, 5.7.1H, 5.7.3H, 6.1, 7.3.1H, 7.4.1H, 7.5.1H, 7.6.1.1H, 7.6.2.1H, 7.6.3.1H, 7.7.1H, 7.8.1H, A.3.1.1, A.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<<  Start of changes  >>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

AWGN	Additive White Gaussian Noise

BS	Base Station

CA	Carrier Aggregation

CA_X	Intra-band contiguous CA of component carriers in one sub-block within Band X where X is the applicable E-UTRA operating band

CA_X-X	Intra-band non-contiguous CA of component carriers in two sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X	Intra-band non-contiguous CA of component carriers in three sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X-X	Intra-band non-contiguous CA of component carriers in four sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-Y	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

CA_X-X-Y	CA of component carriers in two sub-blocks within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating bands

CC	Component Carriers

CG	Carrier Group

CPE	Customer Premise Equipment

CPE_X	Customer Premise Equipment for E-UTRA operating band X

CW	Continuous Wave

DC	Dual Connectivity

DC_X-Y	Inter-band DC of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

DL	Downlink

DIP	Dominant Interferer Proportion

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

EIRP	Effective Isotropic Radiated Power

EPRE	Energy Per Resource Element

E-UTRA 	Evolved UMTS Terrestrial Radio Access

EUTRAN	Evolved UMTS Terrestrial Radio Access Network

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite Systems

HD	Half-Duplex for Sidelink Operation

HD-FDD	Half- Duplex FDD

ITS	Intelligent Transportation Systems

MCS	Modulation and Coding Scheme

MCG	Master Cell Group

MOP	Maximum Output Power

MPR	Maximum Power Reduction

MSD	Maximum Sensitivity Degradation

OCNG	OFDMA Channel Noise Generator

OFDMA	Orthogonal Frequency Division Multiple Access

OOB	Out-of-band

PA	Power Amplifier

PCC	Primary Component Carrier

PMCH	Physical Multicast Channel

P-MPR	Power Management Maximum Power Reduction

ProSe	Proximity-based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSS	Primary Synchronization Signal

PSS_RA	PSS-to-RS EPRE ratio for the channel PSS

SSSS	Secondary Sidelink Synchronization Signal

PSSCH	Physical Sidelink Shared CHannel

PSSS	Primary Sidelink Synchronization Signal

RE	Resource Element

REFSENS	Reference Sensitivity power level

r.m.s	Root Mean Square

SCC	Secondary Component Carrier

SCG	Secondary Cell Group

SDO	Standalone Downlink Only

SINR	Signal-to-Interference-and-Noise Ratio

SNR	Signal-to-Noise Ratio

SSS	Secondary Synchronization Signal

SSS_RA	SSS-to-RS EPRE ratio for the channel SSSSSSS	Secondary Sidelink Synchronization Signal

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

UL-MIMO	Up Link Multiple Antenna transmission

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

V2X	Vehicle to Everything

xCH_RA	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols not containing cell-specific RS

xCH_RB	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols containing cell-specific RS

### <<<  Unchanged sections omitted  >>>

## 4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE, LTE based 5G terrestrial broadcast)

The requirements in clauses 5, 6 and 7 which are specific to CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, and LTE based 5G terrestrial broadcast are specified as suffix A, B, C, D, E, F, G and H where;

a)	Suffix A additional requirements need to support CA

b)	Suffix B additional requirements need to support UL-MIMO

c)	Suffix C additional requirements need to support Dual Connectivity

d)	Suffix D additional requirements need to support ProSe

e)	Suffix E additional requirements need to support UE category 0, category M1, category M2, and category 1bis

f)	Suffix F additional requirements need to support UE category NB1 and NB2

g)	Suffix G additional requirements need to support V2X Communication

h)  Suffix H additional requirements needed to support LTE based 5G terrestrial broadcast

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional subclause (suffix A, B, C, D, E, F, G and H) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional subclause requirements (suffix A, B, C, D, E, F, G and H) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional subclause.

A terminal which supports more than one feature (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, and LTE based 5G terrestrial broadcast) in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal supporting CA, compliance with minimum requirements for non-contiguous intra-band carrier aggregation in any given operating band does not imply compliance with minimum requirements for contiguous intra-band carrier aggregation in the same operating band.

For a terminal supporting CA, compliance with minimum requirements for contiguous intra-band carrier aggregation in any given operating band does not imply compliance with minimum requirements for non- contiguous intra-band carrier aggregation in the same operating band.

A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination.

A terminal which supports CA, for each supported CA configuration, shall support Pcell transmissions in each of the aggregated Component Carriers unless indicated otherwise in clause 5.6A.1.

Terminal supporting Dual Connectivity configuration shall meet the minimum requirements for corresponding CA configuration (suffix A), unless otherwise specified.

For a terminal that supports ProSe Direct Communication and/or ProSe Direct Discovery, the minimum requirements are applicable when

-	the UE is associated with a serving cell on the ProSe carrier, or

-	the UE is not associated with a serving cell on the ProSe carrier and is provisioned with the preconfigured radio parameters for ProSe Direct Communications and/or ProSe Direct Discovery that are associated with known Geographical Area, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and the radio parameters for ProSe Direct Discovery on the ProSe carrier are provided by the serving cell, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and has a non-serving cell selected on the ProSe carrier that supports ProSe Direct Discovery and/or ProSe Direct Communication.

When the ProSe UE is not associated with a serving cell on the ProSe carrier, and the UE does not have knowledge of its geographical area, or is provisioned with preconfigured radio parameters that are not associated with any Geographical Area, ProSe transmissions are not allowed, and the requirements in Section 6.3.3D apply.

A terminal that supports simultaneous E-UTRA ProSe sidelink transmissions and E-UTRA uplink transmissions for the inter-band E-UTRA ProSe/E-UTRA bands specified in Table 5.5D-2, shall meet the minimum requirements for the corresponding inter-band UL CA configuration (suffix A), unless otherwise specified. For transmitter characteristics specified in clause 6, the terminal is required to meet the conformance tests for the corresponding inter-band UL CA configuration and is not required to be retested with simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions.

A terminal that supports E-UTRA V2X intra-band multi-carrier operation including carrier aggregation for the band specified in Table 5.5G-3, shall meet the corresponding transmitter characteristics requirements (in subclauses with suffix G in Section 6) only when there are multiple active transmissions on all of the configured carrier components. When there is only one active transmission on one of the configured carrier components, the corresponding requirements for V2X single carrier operation apply for the corresponding active carrier component.

A terminal which supports MBMS (including 15 kHz, 7.5 kHz ,1.25 kHz, 2.5 kHz and 0.37 kHz subcarrier spacing), shall meet the minimum requirements in clauses 5 and 7. A terminal which supports MBMS is not required to support all kinds of subcarrier spacing.

A terminal that supports multiple TTI patterns in different carriers, different TTI patterns can only be used when the carriers are aggregated in inter-band manner. For intra-band carrier aggregation, only same TTI patterns and same TAG are allowed in aggregated carriers.

### <<<  Unchanged sections omitted  >>>

## 5.5H	Operating bands for LTE based 5G terrestrial broadcast

LTE based 5G terrestrial broadcast is designed to operate in the the operating bands defined in Table 5.5H-1.

Table 5.5H-1 5G terrestrial broadcast operating bands

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 107 |  | N/A |  | 612 MHz | – | 652 MHz | SDO |
| 108 |  | N/A |  | 470 MHz | – | 698 MHz | SDO |

### <<<  Unchanged sections omitted  >>>

## 5.6H	PMCH bandwidth for LTE based 5G terrestrial broadcast

Requirements in the present document are specified for the bandwidths listed in Table 5.6H-1.

Table 5.6H-1: Transmission bandwidth configuration NRB for 5G terrestrial broadcast

| PMCH bandwidth [MHz] | 6 | 7 | 8 |
| --- | --- | --- | --- |
| Transmission bandwidth configuration NRBPMCH | 30 | 35 | 40 |

The LTE based 5G terrestrial broadcast network operates on 6, 7, and 8 MHz channels and the requirements in this specification apply according to configuration by the higher layer parameter pmch-Bandwidth (see TS 36.213 [6]) in the MBSFN area (see TS 36.331 [7]).

Note:  Upon configuration of the PMCH bandwidth, the UE is assumed to configure its baseband filtering to 10 MHz.  This assumption is not intended to restrict the UE implementation.

### 5.6H.1	PMCH bandwidths per operating band for LTE based 5G terrestrial broadcast

LTE based 5G terrestrial broadcast bandwidths and operating bands are shown in Table 5.6H.1-1

Table 5.6H.1-1: 5G terrestrial broadcast bandwidths per operating band

| Operating Band | PMCH bandwidth |  |  |
| --- | --- | --- | --- |
|  | 6 MHz | 7 MHz | 8 MHz |
| 107 | Yes | Yes | Yes |
| 108 | Yes | Yes | Yes |

### <<<  Unchanged sections omitted  >>>

### 5.7.1H	Channel spacing for LTE based 5G terrestrial broadcast

Nominal channel spacing between adjacent broadcast channels is defined as follows

Nominal Channel spacing = PMCH bandwidth

where PMCH bandwidth is the broadcast bandwidth for all broadcast carriers in the same geographical area is indicated by upper layer signaling pmch-Bandwidth in the MBSFN area (see TS 36.331 [7]).  The requirements in this specification do not apply for heterogeneous broadcast bandwidths in the same geographical area.

### <<<  Unchanged sections omitted  >>>

### 5.7.3H	Carrier frequency and EARFCN for LTE based 5G terrestrial broadcast

The EARFCN for applicable bands designated for LTE based 5G terrestrial broadcast according to Table 5.5H-1 are specified in Table 5.7.3H-1.

Table 5.7.3H-1: E-UTRA channel numbers for LTE based 5G terrestrial broadcast

| Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 107 | 612 | 70656 | 70656 – 71055 | N/A |  |  |
| 108 | 470 | 71056 | 71056–73335 | N/A |  |  |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the broadcast carrier with PMCH bandwidth extends beyond the operating band edge shall not be used. |  |  |  |  |  |  |

### <<<  Unchanged sections omitted  >>>

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Unless otherwise stated, NB1 and NB2 requirements specified for an E-UTRA band shall also apply for the re-farmed NR band (e.g. if NB1/NB2 requirements are specified for E-UTRA band 1, they shall also apply for NR band n1).

The requirements of clause 6 do not apply to devices only supporting LTE based 5G terrestrial broadcast.

### <<<  Unchanged sections omitted  >>>

### 7.3.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annexes A.3.18 with parameters specified in Table 7.3.1H-1.

Table 7.3.1H-1: Reference sensitivity for LTE based 5G terrestrial broadcast

| Operating Band | PMCH bandwidth |  |  | Duplex Mode |
| --- | --- | --- | --- | --- |
|  | 6 MHz (dBm) | 7 MHz (dBm) | 8 MHz (dBm) |  |
| 107 | [-99.2] | [-98.5] | [-97.9] | SDO |
| 108 | [-99.2] | [-98.5] | [-97.9] | SDO |
| NOTE: The signal power is specified per port |  |  |  |  |

### <<<  Unchanged sections omitted  >>>

### 7.4.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annexes A.3.18 with parameters specified in Table 7.4.1H-1.

Table 7.4.1H-1: Maximum input level for LTE based 5G terrestrial broadcast

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -22 |  |  |
| NOTE: Reference measurement channel is Annex A.3.18 64QAM, R=3/4 variant |  |  |  |  |

### <<<  Unchanged sections omitted  >>>

### 7.5.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The UE shall fulfil the minimum requirement specified in Table 7.5.1H-1 for all values of an adjacent channel interferer up to –22 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1H-2 and Table 7.5.1H-3 where the throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18.

Table 7.5.1H-1: Adjacent channel selectivity for LTE based 5G terrestrial broadcast

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| ACS | dB | [29.0] | [30.5] | [31.5] |

Table 7.5.1H-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |
| PInterferer | dBm | REFSENS +41.5dB | REFSENS +43.0dB | REFSENS +44.0dB |
| BWInterferer | MHz | 5 |  |  |
| FInterferer (offset) | MHz | 5.5+0.0125 / -5.5-0.0125 | 6.0+0.0075 / -6.0-0.0075 | 6.5+0.0025 / -6.5-0.0025 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |  |  |

Table 7.5.1H-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | PMCH bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 MHz | 7 MHz | 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -49.5 | -51 | -52 |
| PInterferer | dBm | -22 |  |  |
| BWInterferer | MHz | 5 |  |  |
| FInterferer (offset) | MHz | 5.5+0.0125 / -5.5-0.0125 | 6.0+0.0075 / -6.0-0.0075 | 6.5+0.0025 / -6.5-0.0025 |
| NOTE: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |  |  |

### <<<  Unchanged sections omitted  >>>

#### 7.6.1.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18 with parameters specified in Table 7.6.1.1H-1 and 7.6.1.1H-2.

Table 7.6.1.1H-1: In band blocking parameters

| Rx parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 6, 7, 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| BWInterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5+0.0025 |
| FIoffset, case 2 | MHz | 12.5+0.0125 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.18. |  |  |

Table 7.6.1.1H-2: In-band blocking

| Operating band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 107, 108 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies NOTE 4: BW refers to 10 MHz |  |  |  |  |

### <<<  Unchanged sections omitted  >>>

#### 7.6.2.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18 with parameters specified in Table 7.6.2.1H-1 and 7.6.2.1H-2.

For Table 7.6.2.1H-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration (see Figure 5.6H-1). For these exceptions the requirements of subclause 7.7 Spurious response are applicable.

Table 7.6.2.1H-1: Out-of-band blocking parameters

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 6, 7, or 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| NOTE: Reference measurement channel is specified in Annex A.3.18. |  |  |

Table 7.6.2.1H-2: Out of band blocking

| Operating band | 1 | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| 107 | PInterferer | dBm | -44 | -30 | -15 |
| 108 |  |  | -44 | -35 | -20 |

### <<<  Unchanged sections omitted  >>>

#### 7.6.3.1H	Minimum requirements for LTE based 5G terrestrial broadcast

Narrow-band blocking requirements are not applicable to LTE based 5G terrestrial broadcast.

### <<<  Unchanged sections omitted  >>>

#### 7.7.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18 with parameters specified in Table 7.7.1H-1 and 7.7.1H-2.

Table 7.7.1H-1: Out-of-band blocking parameters

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 6, 7, or 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| NOTE: Reference measurement channel is specified in Annex A.3.18. |  |  |

Table 7.7.1H-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

### <<<  Unchanged sections omitted  >>>

#### 7.8.1H	Minimum requirements for LTE based 5G terrestrial broadcast

The throughput shall be ≥ 95% of the maximum throughput as represented by a reported BLER of <5% for the reference measurement channels as specified in Annex A.3.18 with parameters specified in Table 7.8.1H-1.

Table 7.8.1H-1: Wide band intermodulation parameters

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 6, 7, or 8 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| FInterferer 1 (Offset) | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: Reference measurement channel is specified in Annex A.3.18. NOTE 2: BW refers to 10 MHz |  |  |

### <<<  Unchanged sections omitted  >>>

Table A.3.1.1-1Y: Overview of DL reference measurement channels (PMCH)

| Duplex | Table | Name | PMCH BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SDO |  |  |  |  |  |  |  |  |  |
| SDO | Table A.3.18.1-1 | R.PMCH.1 – R.PMCH.6 | 6. 7, 8 | QPSK, 64QAM | 1/3, 3/4 | 30, 35, 40 |  |  | 15 kHz SCS |
| SDO | Table A.3.18.1-2 | R.PMCH.7 – R.PMCH.12 | 6. 7, 8 | QPSK, 64QAM | 1/3, 3/4 | 30, 35, 40 |  |  | 2.5 kHz SCS |
| SDO | Table A.3.18.1-3 | R.PMCH.13 – R.PMCH.18 | 6. 7, 8 | QPSK, 64QAM | 1/3, 3/4 | 30, 35, 40 |  |  | 1.25 kHz SCS |
| SDO | Table A.3.18.1-4 | R.PMCH.19 – R.PMCH.24 | 6. 7, 8 | QPSK, 64QAM | 1/3, 3/4 | 30, 35, 40 |  |  | 0.37 kHz SCS |

### <<<  Unchanged sections omitted  >>>

## A.3.18	Reference Measurement Channels for LTE based 5G broadcast PMCH receiver requirements

### A.3.18.1	SDO

Table A.3.18.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 | R.PMCH.3 | R.PMCH.4 | R.PMCH.5 | R.PMCH.6 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 2152 | 2472 | 2856 | 14112 | 17568 | 18336 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 6120 | 7140 | 8160 | 18360 | 21420 | 24480 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.18.1-2 Fixed Reference Channel for PMCH Receiver Requirements (2.5 kHz SCS)

| Parameter | PMCH (2.5 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.7 | R.PMCH.8 | R.PMCH.9 | R.PMCH.10 | R.PMCH.11 | R.PMCH.12 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 2152 | 2472 | 2856 | 14112 | 17568 | 19848 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 6480 | 7560 | 8640 | 19440 | 22680 | 25920 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.18.1-3 Fixed Reference Channel for PMCH Receiver Requirements (1.25 kHz SCS)

| Parameter | PMCH (1.25 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.13 | R.PMCH.14 | R.PMCH.15 | R.PMCH.16 | R.PMCH.17 | R.PMCH.18 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 2152 | 3112 | 3496 | 16416 | 19080 | 21384 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 7200 | 8400 | 9600 | 21600 | 25200 | 28800 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.18.1-4 Fixed Reference Channel for PMCH Receiver Requirements (0.37 kHz SCS)

| Parameter | PMCH (0.37 kHz SCS) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  | R.PMCH.19 | R.PMCH.20 | R.PMCH.21 | R.PMCH.22 | R.PMCH.23 | R.PMCH.24 |
| PMCH bandwidth | MHz | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 | 10 | 13 | 13 | 13 |
| Modulation |  | QPSK | QPSK | QPSK | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits |  |  |  |  |  |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 1 | 1 |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 7200 | 8400 | 9600 | 72900 | 85050 | 97200 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

### <<<  End of changes  >>>