3GPP TSG-RAN WG4 Meeting # 107	                  R4-2310315

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.27.6

Source:	Moderator (ZTE)

Title:	Ad hoc minutes for NR_NTN_enh_UERF

Document for:	Information

# Introduction

In this contribution, we mainly summarized the ad-hoc meeting minutes for [107][143] NR_NTN_enh_UERF (NTN UE RF requirement in Ka-band), we mainly discussed the following issues.

Issue 1-3: Differentiate UE types from RF requirement perspective

  - Proposal 1: RAN4 to discuss whether and how to categorize UE with above information in terms of regulation, possible impact on RAN4 spec can be power limit, off-axis pattern, tracking accuracy, schemes for uplink transmission, like cease transmission when tracking failed, or reported interference, etc. [Samsung,R4-2308784]

  - Proposal 2: NTN UE power can be used to differentiate different types of NTN UE depending on the deployment scenario (e.g. land, vessel, aircraft, etc) in concordance with regulatory requirements. However, in order to simplify the normalisation of NTN UE above 10 GHz, only one single NTN UE power class could be specified in Rel-18. [THALES,R4-2309717]

- Recommended for GTW discussion:

  - Companies’ views are encouraged during the meeting.

Discussion:

Thales: not against the proposal 1;

Samsung: fine with option 1 and option 2;

Thales: do we need to consider different power class for different steering mechanism?

Ericsson: not sure understand the proposal 2;

Thales: maybe the same power could be applicable for both electrical beam steering NTN UE and also mechanism steering UE;

No agreement reached:

Issue 2-1:  Noise figure

- Proposals

  - Proposal 1: When NTN UE Rx requirements are discussed, at least RAN4 need to assume noise figure as 6 dB considering the outcomes in 7~24GHz SI, ETSI TR 101 854 and IEEE published papers.[Huawei,R4-2308578]

  - Proposal 2:  Noise figure assumption is 6.2 dB for the Ka band UE. [Qualcomm, R4-2309508]

  - Proposal 3:  Consider and further discuss beam peak EIS reference sensitivity as follows for GSO receiver [Qualcomm, R4-2309508]

| 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- |
| -125.5 dBm | -122.5 dBm | -119.5 dBm | -116.5 dBm |

  - Proposal 4:  Discuss the following options [Qualcomm, R4-2309508]

- Option 1:  The reference sensitivity for the NGSO receiver can be relaxed compared to GSO.

- Option 2:  The same reference sensitivity applies to both NGSO and GSO UE receivers.

  - Proposal 5: [Option 1] RAN4 to use the following NTN UE parameters: [THALES,R4-2309717]

Table 1. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

NOTE1: T_a = T_Sky + T_Ground

NOTE2: The antenna temperatures are based on e.g. ITU-R Rec. P372 and Rec. P618.

NOTE3: T_sky is computed using [ITU-R Rec. P.618-13] as expressed below

# 3 Noise temperature As attenuation increases, so does emission noise. For earth stations with low-noise front-ends, this increase of noise temperature may have a greater impact on the resulting signal-to-noise ratio than the attenuation itself. The sky noise temperature at a ground station antenna may be estimated by: Tsky = Tmr (1 – 10–A/10) + 2.7 × 10–A/10 K (63) where: Tsky : sky noise temperature (K) at the ground station antenna A : total atmospheric attenuation excluding scintillation fading (dB) Tmr : atmospheric mean radiating temperature (K).

  - Proposal 6: [Option 2] RAN4 to use the following NTN UE parameters: [THALES,R4-2309717]

Table 2. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

  - Proposal 6: Adopt option 2(5.9dB) for NTNUE NF. [CATT, R4-2307381]

Discussion:

QC: RAN4 requirement is the minimum requirement and not the optimized value for the achieved the target ;

Recommended Agreement:

For the August meeting, more detailed background should be provided to derive the corresponding noise figure..

For the coexistence study, companies are encouraged to consider the following NF for NTN UE :

- Option 1: 2dB;

- Option 2: 6dB

Issue 3-1: NTN UE reference architecture for above 10GHz

- Proposals

  - Proposal 1: RAN4 shall specify a generalized NTN UE terminal reference architecture for above 10 GHz as follows (the details can be left for implementation): [THALES,R4-2309717 ]

Note 1: The Up-Converter and the Tx Power Amplifier are part of the Transmission chain.

Note 2: The Rx LNA (Low-Noise Amplifier) and the Down-Converter are part of the Reception chain.

Note 3: RF represents the Radio Frequency region and IF the Intermediate Frequency region.

Note 4: DP is the Duplexer and ACU is the Antenna Control Unit.

  - Proposal 3:  For the NTN UE supporting LEO, the reference architecture includes a phased array antenna.  For the NTN UE supporting GEO, the reference architecture includes a parabolic antenna.  For the NTN UE supporting both LEO and GEO, the reference architecture includes both a parabolic and a phased array antenna. [Qualcomm ,R4-2309508]

  - Proposal 4:  For the reference architecture with parabolic antenna only, assume two parabolic antennas are available each aimed at different satellites. [Qualcomm ,R4-2309508]

  - Proposal 5:  For the phased array antenna, the baseline assumption is only a single beam can be formed in one direction at any instant in time. [Qualcomm ,R4-2309508]

  - Proposal 6:  A more advanced UE with phased array capable of two simultaneous beams should be further discussed as there are some challenges to be addressed.  For the sake of expediency of the standardization work, it is proposed to focus only on the baseline ability to form only a single beam at any given instant in time.  [Qualcomm ,R4-2309508]

  - Proposal 7: Further consider the following table for discussion in RAN4: [THALES,R4-2309717 ]

| Question | Initial Working Hypothesis | Feedback from Companies |
| --- | --- | --- |
| Should HO aspects in Ka-band be considered for Rel-18? | Yes. Preference for considering HO aspects in Ka-band for Rel-18. | Company X: Company Y: Company Z: |
| Which is the (minimum) number of satellites to which a VSAT/NTN UE will connect in Ka-band? | At least 2 satellites in Rel-18. It is further assumed: - UE is capable of forming 1 single beam only at any given instant of time (regardless of antenna system implementation). - At least two cases should be identified based on single-beam re-pointing time between satellites - The following example cases are considered: A) 1 mechanically-steered UE beam; B) 1 electronically-steered UE beam. | Company X: Company Y: Company Z: |
| Which is the maximum (recommended) interruption time (switching time) between 2 consecutive satellites during HO procedure? | Preference for seamless HO. At physical layer, an interruption of ~20-40ms may be acceptable. No packet loss is expected. TBD | Company X: Company Y: Company Z: |
| Any view for the initial Satellite search time requirement of the NTN UE? | Implementation issue not 3GPP related. TBD | Inmarsat: it’s not necessary to have such kind of requirement; Nokia: it’s fine for GEO and not fine for LEO. |
| Any view of the tracking accuracy? | Implementation issue based on regulatory requirement (see e.g. EN 303 978 and EN 303 979). TBD | Company X: Company Y: Company Z: |
| Other | TBD | Company X: Company Y: Company Z: |

Discussion:

Thales: this figure is very generic which could be applicable for above 10GHz, the duplex should be possible, however this also depend on the implementation.

Huawei: at least common antenna and separate antenna for Tx and Rx side should be considered.

Thales: this is only to show the reference interface for the measurement.

Inmarsat: not to restrict the implementation. Want to have the black box ;

Skyworks:  do you want to standardize the RF in the spec/

Immarsat: yes

Qualcomm:  we need to consider the trade off between antenna system and RF system.

Nokia: more inputs for beam steering capability are needed for the RRM discussion;

Inmarsat: it’s not necessary to have such kind of requirement for the initial satellite search time

Nokia: it’s fine for GEO and not fine for LEO;

No Agreement and further discussion are needed.

Issue 3-2: Beam pointing/accuracy related requirements

- Proposals

  - Proposal 1: RAN4 should specify antenna pointing stability and accuracy requirements for fixed type of NTN satellite UE.  [Ericsson,R4-2308538]

  - Proposal 2: RAN4 should specify antenna pointing accuracy requirement for mobile type of NTN satellite UE (ESIM).  [Ericsson,R4-2308538]

  - Proposal 3: RAN4 to discuss whether or how to build requirements for tracking accuracy to guarantee its compliance during turbulence circumstances, and how fast should it adjust its beam back to desired direction. [Samsung,R4-2308784]

  - Proposal 4 (resubmitted since agreed WF R4-2306624 indicates “tentative agreement”): It is assumed for the NTN capable UE operating in above 10 GHz that the (Rx and Tx) beam pointing error are compliant with the relevant ETSI harmonized standard, e.g.: [THALES,R4-2309717]

- EN 303 978, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

- EN 303 979, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”.

  - Proposal 5: Tracking accuracy can be considered as implementation issue by RAN4 based on regulatory requirement (relevant regulatory requirement will be referenced in the NTN UE TS, however no requirement for tracking accuracy will be specifically designed by RAN4). [THALES,R4-2309717 ]

- Recommended Agreement:

  - It is assumed for the NTN capable UE operating in above 10 GHz that the (Rx and Tx) beam pointing error are compliant with the relevant ETSI harmonized standard, e.g.:

- EN 303 978, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

- For the next meeting, it’s encouraged to provide the technical requirement inputs for fixed VSAT;

Issue 3-3: on-axis and off-axis EIRP requirements

- Proposals

  - Proposal 1: RAN4 should specify on-axis and off-axis EIRP requirements based on Regulations.  [Ericsson,R4-2308538]

  - Proposal 2: RAN4 to discuss whether or how to build requirements for NTN UE with phased array antenna operation to guarantee its compliance to the off-axis eirp mask.  [Samsung,R4-2308784]

- The output power limit may be reduced when the phased array antenna perform electronical tilting to point its beam to the desired SAN.

- If use same power level, the NTN UE with phased array may also required to perform some mechanical rotation for its antenna to reduce the electrical tilting range, and to reduce the effect of the side lobe distortion.

  - Proposal 3: For NTN UE in Ka-band, RAN4 to use/cite/reference ITU recommendation S.524 for radiated off-axis requirement.[THALES,R4-2309717 ]

  - Proposal 4: to consider the Off-axis and On-axis EIRP and spurious emission requirement in EN 303 978 and 303 979. [ZTE,R4-2309183]

  - Proposal 5: 	RAN4 shall define the off-axis EIRP density requirement for both SAN and ESIM type of devices and shall meet the specified envelopes [Verizon,R4-2309381]

  - Proposal 6: 	The off-axis EIRP requirements shall be applied to the ESIM devices operating over the band n511. [Verizon,R4-2309381]

  - Proposal 7: 	The detailed baseline requirements can be based on either FCC order or ITU recommendation S.524.  [Verizon,R4-2309381]

- Recommend

  - Companies’ views are encouraged during the meeting.

Recommended Agreement:

  - To define the off-axis EIRP requirement;

- Note: this requirement for wanted signal within the assigned channel;

  - For on-axis EIRP limit, this depend on the power class

  - To capture the maximum EIRP limit for certain power class;