3GPP RAN WG4 Meeting #107		R4-2310412

Incheon, South Korea, May 22nd – 26th, 2023

Title:	WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW

Agenda item:	7.1.1.1

Release:	Rel-18

Source:	Skyworks Solutions, inc.

Document for:	Approval

# 1	Background

- The guidelines for the new NR-CA MSD table template were captured in WF [1] in Rel-17 for interference due to harmonic and interference due to cross-band isolation.

- In Rel-18, EN-DC MSD test points will be migrated to this new template and the guidelines have been agreed in WF [2].

- Under the new template, there is a maximum of two test-points per band combination:

  - the so-called “first MSD test point” is mandatory. It captures the worst-case MSD which is specified for the lowest DL CBW:

    - For cross-band isolation MSD, the UL is configured with the highest UL CBW and full UL RB allocation.

    - For harmonic MSD, the UL is configured with CBW and RB allocation that ensures the harmonic PSD is entirely integrated by the DL channel.

  - the so-called “second MSD test point” is optional: it is intended to accommodate exceptions due to say, regional spectrum allocations, or proponent concerns on specific CBW of interest etc.  This test point does not necessarily need to be 100% compliant to the WF [1,2] guidelines.

- Even with the new table template, the request for BCS4/5 for an inter-band CA or DC band combination may introduce a new highest of a new lowest channel bandwidth. In turn, this means that MSD may need to be re-evaluated for MSD due to harmonic interference, MSD due to cross-band interference, and for MSD due to dual UL interference. WF [1] recommended to check the impact on the agreed MSD test points.

- This WF clarifies:

  - Which MSD test point shall be revisited, and which MSD test point may be re-visited on a case-by-case basis,

  - How to handle the maintenance of MSD test points for legacy UEs that may not support the new highest/lowest channel bandwidth introduced by the introduction of BCS4/5,

  - How to handle the impact of introducing BCS4/5 on MSD test point due to Rx harmonic mixing for CA_n2-n77 [3] and CA_n5_n77 [4].

# Way Forward on handling MSD test points and legacy UEs

<WF1 on handling MSD test points for harmonic and cross-band isolation interference>

WF1 agreement:

Table 1 and Table 2 indicate in which case the MSD test point shall be re-visited, and in which case the MSD test point may be revisited on a case-by-case basis for:

- NR-CA combinations when a BCS4/5 request introduces a new highest CBW or a new lowest CBW,

- EN-DC combinations and legacy BCS4/5 NR-CA combinations when a new highest CBW or a new lowest CBW is introduced in one of the constituent frequency band,

Table 1: Handling of MSD test points due to cross-band isolation interference

|  | new highest CBW | new lowest CBW |
| --- | --- | --- |
| UL band | SHALL be revisited | May be revisited on case-by-case |
| DL band | May be revisited on case-by-case | SHALL be revisited |

Table 2: Handling of MSD due to harmonic interference (uplink and/or Rx harmonic mixing)

|  | new highest CBW | new lowest CBW |
| --- | --- | --- |
| UL band | May be revisited on case-by-case | May be revisited on case-by-case |
| DL band | May be revisited on case-by-case | SHALL be revisited |

It is recommended that proponents provide justification for the case when the MSD test point may need to be revisited.

<WF2 on handling MSD test points for dual uplink or triple uplink IMD interference>

Background:

For MSD due to dual and triple UL (triple beat) IMD interference:

- There is only 1 test point per IMD order per victim band, hence the concept of using a second MSD test point is not applicable. Adding a 2nd MSD test point would go against the agreed BC simplification guidelines,

- The introduction of a new lowest CBW may impact the legacy MSD test points. Such requests have been made for:

  - band n41: the new lowest 5MHz CBW replaces the previous lowest of 10MHz CBW. Some MSD test points may need to be revisited.

  - band n79:  the new lowest 10MHz CBW replaces the previous lowest of 40MHz. Some MSD test point may need to be revisited.

- The introduction of a new highest CBW is unlikely to impact the 2 UL or 3 UL MSD test points since the UL configurations are chosen to ensure direct hit IMD collision.

WF2 agreement:

The MSD test points due to 2 UL or 3 UL (triple-beat) IMD interference may be revisited on a case-by-case basis for:

- NR-CA combinations when a BCS 4/5 request introduces a new lowest DL CBW,

- EN-DC combinations and legacy BCS 4/5 NR-CA combinations when a new lowest CBW is introduced in one of the constituent frequency band,

The proponent may decide this change is not necessary, in which case no change is needed to the MSD test points.

<WF3 on handling legacy UEs>

Background: Legacy UEs may not support the new highest/lowest CBW introduced by BCS4/5 request.

WF3 agreement:

For MSD test points due to harmonic interference and cross-band isolation interference:

- To ensure the conformance testing of legacy UEs is not impacted, the so-called “1st MSD test point” remains unchanged, i.e., it remains the baseline mandatory MSD test point.

- When the MSD test point shall be revisited, the new MSD test point becomes the 2nd optional MSD test point in the current Release where BCS 4/5 is introduced,

- When the MSD test point may be revisited, the new MSD test point might become the 2nd optional MSD test point in the current Release where BCS 4/5 is introduced,

- If the 2nd optional test point is already specified:

    - when the MSD test point shall be revisited, the new MSD test point replaces/overwrites the legacy agreed 2nd optional test point,

    - when the MSD test point may be revisited:

    - the new MSD test point might replace/overwrite the legacy agreed 2nd optional test point

    - the proponent may decide this change is not necessary, in which case no change is needed to the MSD test points.

# Way Forward on handling BCS4/5 requests for CA_n2-n77 and CA_n5-n77

<WF4 on handling new MSD test point for CA_n2-n77>

Background:

- For CA_n2-n77, the highest supported CBW for band n2 is 20MHz (BCS0 and BCS1).

- The introduction of BCS4/5 introduces a new band n2 highest DL channel of 40MHz.

- CA_n2-n77 is subject to MSD due to Tx harmonic, Rx harmonic mixing (UL1/DL2) and dual UL IMD interference. The new highest DL 40MHz CBW may impact only the Rx harmonic mixing MSD test point for which there are already 2 test points.

- According to WF1 Table 2, the MSD test point may be revisited with justification from proponents.

WF4 agreement:

Provided that proponents justify the need to revisit the band n2 40MHz CBW MSD due to UL1/DL2 Rx harmonic mixing, the 2nd “optional” MSD test point may be updated, and MSD is FFS for PC3, PC2 and PC1.5 as captured in Table 3, Table 4, Table 5. The legacy “first MSD test point” remains unchanged.

Table 3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | [20] | [15] | [100 (RBstart=0)] | 40 | [FFS] | NOTE 7 | UL1/DL2 |
| NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that fDLLB=fULHB/0.20.1 in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | [20] | [15] | [100 (RBstart=0)] | 40 | [FFS] | NOTE 4 | UL1/DL2 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that fDLLB=fULHB/0.20.1 in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 5: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | [20] | [15] | [100 (RBstart=0)] | 40 | [FFS] | NOTE 4 | UL1/DL2 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that fDLLB=fULHB/0.20.1 in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

<WF5 on handling new MSD test point for CA_n5-n77>

Background:

- For CA_n5-n77, the highest supported CBW for band n5 is 20MHz (BCS0 and BCS1).

- The introduction of BCS4/5 introduces a new band n5 highest DL channel of 25MHz.

- CA_n5-n77 is subject to MSD due to Tx harmonic, Rx harmonic mixing (UL1/DL4) and dual UL IMD interference.

- According to WF1 Table 2, the MSD test point may be revisited with justification from proponents.

WF5 agreement:

Proponents do not see the need to update the 2nd MSD test point. Therefore, according to WF1 and WF3 the MSD test points for CA_n5_n77 UL1/DL4 Rx harmonic mixing remain unchanged.

# References

R4-2210565 “WF on criteria on Rel-17 enhanced MSD table format”, 3GPP TSG RAN WG4 Meeting #103-e, Online, May 9th – 22nd, 2022, Skyworks Solutions Inc.

- R4-2306582, “WF on cross-band isolation MSD simplification for EN-DC”, 3GPP TSG RAN WG4 Meeting #106bis-e, Online, April 17th – 26th, 2023, Skyworks Solutions Inc.

- R4-230560, “Updated TP for TR 38.718-02-01 to introduce BCS4 and BCS5 for CA_n5-n77”, 3GPP TSG-RAN WG4 Meeting # 107, Incheon South Korea, May 22nd – 26th, 2023, Huawei, HiSilicon, CATT.

R4-2308077, “Draft CR for TS 38.101-1 Support of CA_n2-n77 CA_n5A-n77B CA_n12-n77 CA_n66-n77 CA_n71-n77”, 3GPP TSG-RAN WG4 Meeting # 107, Incheon South Korea, May 22nd – 26th, 2023, Nokia, USC.