3GPP TSG-RAN WG4 Meeting # 107												R4- 2310440

Incheon, KR, May 22 – May 26, 2023

Agenda item:			5.4

Source:	Moderator (Ericsson)

Title:	Summary for [107][301] BSRF_Maintenance

Document for:	Information

# Introduction

The scope of this topic summary is BS RF maintenance agenda items. Topics are divided according to the agenda:

Up to Rel-16 maintenance:

- BS RF requirements and BS conformance testing	(4.2)

Rel-17 maintenance:

- NR Repeater	(5.2.1)

- NTN: System parameters, SAN RF requirements & conformance	(5.2.8.1, 5.2.8.2)

- Extending current NR operation to 71GHz: BS RF requirements & conformance	(5.2.9.2, 5.2.9.3)

- Other NR/LTE WIs: BS RF requirements	(5.2.10.1)

# Topic #1: BS RF requirements and BS conformance testing (up to Rel-16) (4.2)

## Companies’ contributions summary

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2307367 | CATT | CR for TS 38.141-1, Add sweep time for true RMS detection mode for receiver spurious emissions Summary of change: Add sweep time for true RMS detection mode for receiver spurious emssion in sub-clause 7.6.4.2. |
| R4-2307376 | CATT | CR for TS 38.174, Correction on OTA IAB output power etc. Summary of change: - Change “24 dBm + 10log(NTXU,counted)” to “+ 33dBm” in Table 9.3.1-2. - Change “IAB-DU type 1-O” to “IAB-DU type 2-O” in sub-clauses 9.5.2.3 and 9.5.2.5. - Remove “and clause 9.7.5.2.5” in sub-clause 9.8.2. - Change title of sub-clause 10.3.3.1 from “Minimum requirement for IAB-MT type 1-O” to “General”. |
| R4-2307770 | Huawei, HiSilicon | CR to 37.104: Clarification on the OBUE limites when narrow carrier adjacent to the sub block edge Summary of change: It is clarified that the specific requirement applied When narrow carrier adjacent to the sub block edge. |
| R4-2307773 | Huawei, HiSilicon | CR to 37.141: Clarification on the OBUE limites when narrow carrier adjacent to the sub block edge |
| R4-2307776 | Huawei, HiSilicon | CR to 37.105: Clarification on the OBUE limites when narrow carrier adjacent to the sub block edge |
| R4-2307779 | Huawei, HiSilicon | CR to 37.145-1: Clarification on the OBUE limites when narrow carrier adjacent to the sub block edge |
| R4-2307782 | Huawei, HiSilicon | CR to 37.145-2: Clarification on the OBUE limites when narrow carrier adjacent to the sub block edge |
| R4-2307785 | Huawei, HiSilicon | CR to 36.141: Receiver spurious emissions Summary of change: Add the same sentence from 38.141-1 to clarify that the transmitter is OFF for the TDD case. |
| R4-2307788 | Huawei, HiSilicon | CR to 37.141: Receiver spurious emissions |
| R4-2307791 | Huawei, HiSilicon | CR to 37.145-1: Receiver spurious emissions |
| R4-2308490 | Huawei, Hisilicon | CR for TS 36104 Operating band unwanted emissions for Single RAT multi-band BS Summary of change: Add a note：for BS supporting multi-band operation, either this limit or -16dBm/100kHz with correspondingly adjusted f_offset shall apply for this frequency offset range for operating bands <1GHz |
| R4-2308491 | Huawei, Hisilicon | Draft CR for 36.141 Operating band unwanted emissions for Single RAT multi-band BS |
| R4-2308492 | Huawei, Hisilicon | CR for TS 38.104: Operating band unwanted emissions for Single RAT multi-band BS |
| R4-2308493 | Huawei, Hisilicon | CR for TS 38.141-1: Operating band unwanted emissions for Single RAT multi-band BS |
| R4-2308996 | Nokia, Nokia Shanghai Bell | CR to TS 38.141-1 on corrections of Wide Area BS Category B operating band unwanted emission limits Summary of change: Correct the clauses references for the Wide Area BS Category B operating band unwanted emission limits. |
| R4-2309471 | ROHDE & SCHWARZ | Update to table format for enabling automated data scraping [CR to 38.104] Summary of change: Moving up the SCS outside the ream of BS channel bandwith is the only change needed |
| R4-2309607 | Ericsson | CR to 38.104: Correction to ACLR and CACLR requirement Summary of change: The text reference for BS channel bandwidth in ACLR and CACLR tables for non-contiguous spectrum is changed to “BS channel bandwidth of carrier transmitted below or above the sub-block edge or Base Station RF Bandwidth edge” (FR1) and to “BS channel bandwidth of carrier transmitted below or above the sub-block edge” (FR2). |
| R4-2309611 | Ericsson | CR to 38.141-1: Correction to ACLR and CACLR requirement |
| R4-2309615 | Ericsson | CR to 38.141-2: Correction to ACLR and CACLR requirement |
| R4-2309642 | Huawei, HiSilicon | CR to TR 37.941: correction of n259-related frequency range for MU of the EIRP test requirement, Rel-16 Summary of change: - Correction of the upper edge of band n259 in table 9.3.4-2. |

## Open issues summary

N/A

# Topic #2: NR Repeater (Rel-17) (5.2.1)

## Companies’ contributions summary

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2307371 | CATT | CR for TS 38.106, Correction on EIRP accuracy for repeater type 2-O Summary of change: - Change EIRP accuracy in normal conditions from 2dB to 3.4dB according to EIRP accuracy for BS type 2-O. - Change EIRP accuracy in extreme conditions from 2.5dB to 4.5dB according to EIRP accuracy for BS type 2-O. |
| R4-2307373 | CATT | CR for TS 38.115-1, Add manufacturer declarations for test configurations and RF channels Summary of change: - Add Repeater RF Bandwidth terms in sub-clause 3.1. - Add Maximum passband Bandwidth identifier (D.18) and Maximum repeater RF Bandwidth for multi-band operation declaration identifier (D.19) in Table 4.6-1. - Change Maximum passband Bandwidth identifier from D.11 to D.18 in sub-clause 4.7. - Change Maximum repeater RF Bandwidth for multi-band operation declaration identifier from D.12 to D.19 in sub-clause 4.7. - Change “BS” to “Repeater” in Table 4.8.4-1. - Remove Occupied bandwidth and CA related content in sub-clause 4.9.1. |
| R4-2307375 | CATT | CR for TS 38.115-2, Add manufacturer declarations for test configurations and RF channels Summary of change: - Add Repeater RF Bandwidth terms in sub-clause 3.1. - Add Maximum passband Bandwidth identifier (D.27) in Table 4.6-1. - Change Maximum passband Bandwidth identifier from D.11 to D.27 in sub-clause 4.7. - Remove Occupied bandwidth and CA related content in sub-clause 4.9.1. - Remove multi-band operation related content in sub-clause 4.9.1. |
| R4-2307488 | NEC | CR to 38.106: Corrections on repeater OTA output power requirements (Rel-17) Summary of change: Minimum requirements for OTA output power (EIRP) were corrected to be same as those for BS type 2-O. |
| R4-2307490 | NEC | CR to 38.115-2: Corrections on repeater output power requirements Summary of change: Frequency range, clause number, and note number were corrected. |
| R4-2307491 | NEC | CR to 38.115-1: Corrections on repeater transient period requirements (Rel-17) Summary of change: Corrected the reference to the repeater core specification. |
| R4-2308610 | Nokia, Nokia Shanghai Bell | CR to TS 38.115-1: Addition of missing bands for repeater co-existence and co-location requirements Summary of change: - Addition of band n67 and band n100 for repeater co-existence reuqirements with other systems - Addition of band n100 for repeater co-location |
| R4-2308616 | Nokia, Nokia Shanghai Bell | CR to TS 38.115-2: Clarifcations for repeater test models Summary of change: - Corrected list of requirements for test model RUL-FR2-TM1.1 - Editorial correction of RUL-FR2-TM2 - Corrected list of requirements for test model RUL-FR2-TM2a - Editorial correction of RUL-FR2-TM3.1a |
| R4-2308617 | Nokia, Nokia Shanghai Bell | CR to TS 38.115-1: Clarifcations for repeater test models Summary of change: - Corrected list of requirements for test model RUL-FR1-TM1.1 - Corrected list of requirements for test model RUL-FR1-TM2 - Corrected list of requirements for test model RUL-FR1-TM2a - Corrected list of requirements for test model RUL-FR1-TM3.1 - Corrected list of requirements for test model RUL-FR1-TM3.1a |

## Open issues summary

N/A

# Topic #3: NTN: System parameters, SAN RF requirements & conformance	(Rel-17) (5.2.8.1, 5.2.8.2)

## Companies’ contributions summary

Discussion papers

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2309722 | THALES | Proposal for Extension to 30 MHz Channel Bandwidth for NR NTN FR1 (Discussion paper) |

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2307379 | CATT | CR for TR 38.863, Correction on interfering signal RMS field-strength for OTA out-of-band blocking - Change maximum EIRP of SpaceOps (Space Operations) TC (TeleCommand) stations 80dBW to 80dBm in sub-clause 7.3.3.2.4.1. - Change maximum EIRP of SpaceOps (Space Operations) TC (TeleCommand) stations 80dBW to 80dBm in sub-clause 7.3.3.3.5.2.1. - Change 0.091 V/m to 0.0029V/m |
| R4-2307380 | CATT | CR for TS 38.108, Correction on out-of-band emissions Summary of change: - Add an explanation “with whole transceiver unit array of SAN” to definition of SAN transponder in sub-clause 3.1. - Add an explanation for necessary bandwidth in sub-clause 6.6.4.1. - Add an explanation for BWChannel. - Change BWSAN in note1 for PSDChannel equation in Table 6.6.4.2-1 to BWChannel. |
| R4-2309653 | Ericsson | CR to TS 38.181: Introduction of testing under extreme test environment Summary of change: Introduce requirement formulation for testing under extreme test environment Editorial changes |
| R4-2309654 | Ericsson | CR to TS 38.181: Introduction of testing under extreme test environment - OTA Summary of change: Introduce requirement formulation for testing under extreme test environment Replaced “satellite” with “SAN” . |
| R4-2309729 | Thales | Corrections to SAN TS 38.108 Summary of change: Correct symbols, correct definitions. |
| R4-2309731 | Thales | Corrections to SAN TS 38.181 Summary of change: Correct definitions, correct references. |

## Open issues summary

N/A

# Topic #4: Extending current NR operation to 71GHz: BS RF requirements & conformance (Rel-17) (5.2.9.2, 5.2.9.3)

## Companies’ contributions summary

Approval papers

| T-doc number | Company | Title / Proposals / Observations |
| --- | --- | --- |
| R4-2307223 | Nokia, Nokia Shanghai Bell | Proposals on measurement uncertainties of BS OTA transmitter requirements for extending current NR operation to 71 GHz Proposal 1: To use 0.9 dB for UID C1-1 for in-band TRP for BS output power. Proposal 2: To use 0.9 dB for UID C1-7 and UID C1-8 for ACLR. Proposal 3: To use 0.9 dB for UID C1-7 for OBUE. Proposal 4: To use 0.9 dB for UID C1-7 for spurious emission for 71 to 110 GHz and 110 to 142 GHz. Proposal 5: Not to use 1.5 dB additional LNA MU UID for low level requirements. Proposal 6: To take the maximum total MU value of the two methods (CATR and RC) for the final OOB EM total MU. Proposal 7: To use 0.4 for UID A2-5a, 0.51 for UID A2-5b. Proposal 8: To use EVM MU value of 1.0%. (CR in R4-2307221) |
| R4-2307696 | Ericsson | Final touch on remaining open issues related to FR2-2 conformance testing Proposal 1: For FR2-2 MU evaluation use SA MU of 2.00 dB for 71<f<110 GHz and 2.30 dB for 110<f<142 GHz and for PM MU use 1.50 dB for 71<f<110 GHz. Proposal 2: For RC, adopt the described additional absolute power calibration for TRP BS output power MU evaluation. Proposal 3: Adopt MU evaluation for BS output power measured in RC according to Table 2.1.3-1 (Expanded uncertainty of 2.93 dB). Proposal 4: Adopt MU evaluation for ACLR and OBUE measured in RC according to Table 2.2-1 (Expanded uncertainty of 4.50 dB). Proposal 5: Adopt MU evaluation for transmitter spurious emission measured in RC according to Table 2.3-1 (Expanded uncertainty of 5.35 dB for 71<f<110 GHz) and Table 2.3-2 (Expanded uncertainty of 5.89 dB for 110<f<142 GHz). Proposal 6: No UID is required for additional LNA. Proposal 7: To allow for using multiple test methods for a specific requirement, set TT based on maximum MU. Proposal 8: Use EVM MU of 1.0 % for spectrum/signal analyzer. (CR in R4-2307697) |
| R4-2309098 | Keysight Technologies UK Ltd | FR2-2 BS conformance, about Test Model text clarification Proposal, we propose Definition A, which maintain Phy definition from TS38.211 for test model, not to create test model specific Phy. 80 slot repetition is for payload data only, and clarification to add on PDCCH and PDSCH. (CR in R4-2309099) |
| R4-2309101 | Keysight Technologies UK Ltd, Murata Manufacturing Co Ltd. | FR2-2 LNA MU update for BS conformance testing Observation-1 - Amplifier gain is sensitive to temperature change and measurement takes long time. Practical measurement point of view, it should be assumed some level of temperature variation on amplifier. Proposal-1 - It is reasonable to assume ±10℃ temperature variation on amplifier, then use this for calculating LNA MU values. Proposal-2 - Proposed LNA MU values (1 sigma) are - @71 GHz, ±0.34 dB - 71G ~ 110 GHz, ±0.51 dB - 110G ~142 GHz, ±0.51 dB (CR in R4-2309103) |
| R4-2309102 | Keysight Technologies UK Ltd | FR2-2 TE MU and various update for BS conformance testing • Proposal-1, UID C1-1 term for in-band TRP is 2.00 (1-sigma) • Proposal-2, UID C1-7 and C1-8 for ACLR absolute and relative, both are 2.26 (1-sigma) rather 2.36 • Proposal-3, UID C1-7 for OBUE is 2.26 (1-sigma) • Proposal-4, UID C1-7 for spurious emission for frequency 71 G ~ 110 G is 2.26 and for 110 G ~ 142 G is 2.38 • Proposal-5a, Yes, LNA is required because of link budget. LNA MU needs to be added to MU budget for TxOff, ACLR, OBUE and spurious measurement. • Proposal-5b, Take LNA MU value proposed in our other contribution [3] (R4-2309101) • Proposal-6, For OOB EM MU, take larger number from CATR or Reverb Chamber, currently two numbers are shown as FFS. Previously, number from CATR is taken because CATR considered as provider of larger value. • Proposal-7, For mismatch value, A2-5a as 0.4 and A2-5b as 0.40 for frequency up to 110 GHz. For 110 GHz to 142 GHz, use 0.51 for both A2-5a and A2-5b. • Proposal-8, EVM MU for FR2-2, we propose 1.1%. 0.1% increase from existing number for FR2-1. • Propose calculated test system MU values in Table 3 and 4 above (CR in R4-2309104) |
| R4-2309631 | Huawei, HiSilicon | FR2 TX Excel spreadsheet update for TR 37.941 Proposal 1: Approve the attached TX MU spreadsheet capturing FR2-2 MU budget calculations updates, as baseline for TR 37.941 update. (CRs in R4-2309633, R4-2309635) |
| R4-2309632 | Huawei, HiSilicon | FR2 RX Excel spreadsheet update for TR 37.941 Proposal 1: Approve the attached RX MU spreadsheet capturing FR2-2 MU budget calculations updates, as baseline for TR 37.941 update. (CRs in R4-2309634, R4-2309637) |

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2307221 | Nokia, Nokia Shanghai Bell | CR to TS 38.141-2 on completion of measurement uncertainties for extending current NR operation to 71GHz Summary of change: Complete measurement uncertainties for extending current NR operation to 71GHz. |
| R4-2307387 | CATT | Draft CR for 37.941, On FR2-2 RX directional requirements in clauses 10.1-10.5 and respective MU and TT summary Summary of change: 1) Add Expanded uncertainty (dB) for the OTA sensitivity for 52.6 GHz < f ≤ 71 GHz in Table 10.2.7-2. 2) Add Test Tolerance values for the OTA sensitivity for 52.6 GHz < f ≤ 71 GHz in Table 10.2.8-2. 3) Add MU for adjacent channel selectivity, in-band blocking, in-channel selectivity for 52.6 GHz < f ≤ 71 GHz in Table 10.5.4-4. 4) Add Maximum OTA Test System uncertainty for OTA reference sensitivity level, OTA adjacent channel selectivity, and OTA in-band blocking (General) for 52.6 GHz < f ≤ 71 GHz in Table 17-4. 5) Add Test Tolerance values derivation for OTA reference sensitivity level for 52.6 GHz < f ≤ 71 GHz in Table 18-4. |
| R4-2307697 | Ericsson | CR to 37.941: Addition of technical background information for EIRP and EIS Summary of change: The additions in this CR: - In subclause 7.3.1, addition of description related to usage of up/down converters within FR2. - In subclause 8.3, addition of new section in subclause 8.3.1 for additional absolute power level calibration. - In subclause 9.2.3.4, addition of table with CATR MU for EIRP in table 9.2.3.4-3. - In subclause 9.2.7, addition of table column for FR2-2 in table 9.2.7-2. - In subclause 9.2.8, addition of table column for FR2-2 in table 9.2.8-2. - In subclause 9.3.3.4, addition of table for extreme condition CATR MU for EIRP in table 9.3.3.4-2. - In sub-clause 10.2.3, addition of table for CATR MU for EIS in table 10.2.3.4-2. - In sub-clause 10.2.7, addition of table column for FR2-2 in table 10.2.7-2 - In sub-clause 10.2.8, addition of table column for FR2-2 in table 10.2.8-2. |
| R4-2307698 | Ericsson | CR to TR 37.941: Improvement of BS output power, ACLR, OBUE and Spurious emission MU for RC in subclause 8.8, 11.2.5, 11.3.5, 11.4.5, 12.2.4 and 12.2.5 Summary of change: - An new section (8.8) is created to describe the additional power level calibration procedure. - In subclause 11.2.5, addition of RC MU evaluation BS output power - In subclause 11.3.5, addition of RC MU evaluation for ACLR - In subclause 11.4.5, addition of RC MU evaluation for OBUE - In subclause 12.2.4, addition of RC MU evaluation for spurious emissions - In subclause 12.2.5, addition of MU values for RC |
| R4-2307699 | Ericsson | CR to TS 38.141-2: Clean-up of FR2-2 in Clause 4, 6, 7 and Annex C Summary of change: - In sub-clause 4.1.2.2, clean up and [] removal and MU values for ACLR, OBUE is updated. For spurious emission, FFS is replaced with values - In sub-clause 4.1.2.3, clean up and [] removal. - In sub-clause 6.3.5.2, BS output power requirements is updated. - In sub-clause 6.7.3.5.2, ACLR requirement is updated. - In sub-clasue 6.7.4.5.2, OBUE requirement is updated. - In sub-clause 7.3.5.3, OTA reference sensitivity [] removed.. - In sub-clause 7.5.1.5.3, the table structure is updated. The orignal table format is broken. - In sub-clause 7.9.5.2, ICS requirement [] removed. - In Annex C, TT values are updated and [] removed. |
| R4-2309035 | Nokia, Nokia Shanghai Bell | CR to TS 38.104 on corrections of table references for extending current NR operation to 71GHz Summary of change: 1) Add the missing word before 5.3.5-3 in clause 5.3.5. 2) Correct the table reference in table 5.4.3.3-2. |
| R4-2309099 | Keysight Technologies UK Ltd | CR to 38.141-2: 71 GHz Extension BS conformance test Test Model clarification Summary of change: In Clause 4.9.2.3 - Added clarificaiton on repetition is about payload data. - Not to repeat other physical layer related sequence such as DM-RS and PT-RS which raised as question requires clarification during discussion. |
| R4-2309103 | Keysight Technologies UK Ltd | CR to 37.941: 71 GHz Extension TE MU update Summary of change: Iist of updated clause - CATR MU table for FR2-2 (9.2 EIRP, 9.3 EIRP extreme, 9.10 Tx OFF, 10.2 EIS, 11.2 Inband TRP, 11.3 ACLR, 11.4 OBUE, 12.2 Spurious) - Maximum MU and TT for FR2-2 (9.2 EIRP, 9.3 EIRP extreme, 9.10 Tx OFF, 10.2 EIS, 10.5 ACS, 10.6 Rx IMD, 11.2 Inband TRP, 11.3 ACLR, 11.4 OBUE, 12.2 Spurious, 14.2 Out of band blocking) - New UID (C.1) - TE MU table updated (Annex C.1, C.2) |
| R4-2309104 | Keysight Technologies UK Ltd | CR to 38.141-2: 71 GHz Extension BS conformance test MU update Summary of change: In Clause following, values are updated - 4.1.2 MU table for FR2 - 6.5.2 Tx OFF power requirement - 6.6.3 EVM requirement - 6.7.3 ACLR relative and absolute requirement - 6.7.4 OBUE requirement - C.1 TT table for FR2 Tx |
| R4-2309633 | Huawei, HiSilicon | CR to TR 37.941: FR2-2 MU budget calculations implementation for TX requirements Summary of change: - Implementation of the FR2-2 TX MU budget calculations - Implementation of the FR2-2 TX expanded uncertainty values - Implementation of the FR2-2 TX TT values |
| R4-2309634 | Huawei, HiSilicon | CR to TR 37.941: FR2-2 MU budget calculations implementation for RX requirements Summary of change: - Implementation of the FR2-2 RX MU budget calculations - Implementation of the FR2-2 RX expanded uncertainty value - Implementation of the FR2-2 RX TT values |
| R4-2309635 | Huawei, HiSilicon | CR to TS 38.141-2: Removal of [] from FR2-2 TX test requirements, Rel-17 Summary of change: Removal of [] from MU, TT and related TX requirements for FR2-2. |
| R4-2309637 | Huawei, HiSilicon | CR to TS 38.141-2: Removal of [] from FR2-2 RX test requirements, Rel-17 Summary of change: Removal of [] from MU, TT and related RX requirements for FR2-2. |

## Open issues summary

### Sub-topic 4-1

Issue 4-1: MU values and related spread sheets

- Proposals for MU values are in Approval papers R4-2307223, R4-2307696, R4-2309101, R4-2309102, R4-2309631, 
R4-2309632

- Recommended WF

  - Proposals for MU values need to be discussed and merged off-line before any CRs for 37.941 and 38.141-2 can be agreed.

# Topic #5: Other NR/LTE WIs: BS RF requirements (Rel-17) (5.2.10.1)

## Companies’ contributions summary

Discussion papers

| T-doc number | Company | Title |
| --- | --- | --- |
| R4-2307694 | Ericsson, NIST, Bluetest | Improvement of RC test method description in TR 37.941 (Discussion paper; CR in R4-2307695) |

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2307224 | Nokia, Nokia Shanghai Bell | CR to TS 38.104 on corrections and clarifications of operating band unwanted emission limits for band n104 Summary of change: 1) In NOTE 1 of Tables 6.6.4.2.2.1-2a, 6.6.4.2.3-1a, 6.6.4.2.3-2a, 6.6.4.2.4-1a, and 6.6.4.2.4-1b, correct the boundary for the absolute limit for Wide Area BS type 1-C to 40 MHz to align with the final step of the emission mask. 2) Clarify that Tables 6.6.4.2.3-1 and 6.6.4.2.3-2 are not applicable to Medium Range BS for Band n104. |
| R4-2307226 | Nokia, Nokia Shanghai Bell | CR to TS 38.141-1 on corrections and clarifications of operating band unwanted emission limits for band n104 Summary of change: 1) Correct the incorrect and redundant statements in clauses 6.6.4.5.2 and 6.6.4.5.3. 2) Clarify that Table 6.6.4.5.4-1 to 6.6.4.5.4-4 are not applicable to Medium Range BS for Band n104. |
| R4-2307363 | CATT | CR for TR 38.104, Correction on reference of PREFSENS for in-band blocking and out-of-band blocking for band n104 Summary of change: - Add “For band n104, PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1a, 7.2.2-2c, and 7.2.2-3c.” in Table 7.4.2.2-1. - Add “For band n104, PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1a, 7.2.2-2c, and 7.2.2-3c.” in Table 7.5.2-1. |
| R4-2307365 | CATT | CR for TR 38.141-1, Correction on reference of PREFSENS for in-band blocking and out-of-band blocking for band n104 Summary of change: - Add “For band n104, PREFSENS depends also on the BS channel bandwidth as specified in TS 38.104 [2], table 7.2.2-1a, 7.2.2-2c and 7.2.2-3c.” in Table 7.4.2.5-1. - Change “tables 7.2.5-1a, 7.2.5-2c and 7.2.5-3c” in Table 7.5.5.1-1a to “tables 7.2.2-1a, 7.2.2-2c, and 7.2.2-3c of TS 38.104 [2]”. |
| R4-2307695 | Ericsson, NIST, Bluetest | CR to TR 37.941: Improvement of RC description in clause 2 and sub-clause 7.8 Summary of change: - In clause 2, one reference to technical backgrund information is added. - In sub-clause 7.8, additional technical information is added. |
| R4-2308612 | Nokia, Nokia Shanghai Bell | CR to TS 38.176-1: Addition of missing bands for IAB co-existence and co-location requirements Summary of change: - Addition of band n67 and band n100 for IAB co-existence reuqirements with other systems - Addition of band n100 for IAB co-location |
| R4-2308614 | Nokia, Nokia Shanghai Bell | CR to TS 38.176-2: Addition of missing bands for IAB co-existence and co-location requirements Summary of change: - Addition of band n67 and band n100 for IAB co-existence reuqirements with other systems - Addition of band n100 for IAB co-location |
| R4-2309193 | ZTE Corporation | CR to TS 38.174 Maintenance of IAB for supported BW R17 Summary of change: Add 35MHz and 45MHz BW supported by IAB to the specification. Correct a typo in Table 7.4.2.3-3. |
| R4-2309195 | ZTE Corporation | CR to TS 38.176-1 Maintenance of IAB for supported BW R17 Summary of change: Add 35MHz and 45MHz BW supported by IAB to the specification. |
| R4-2309197 | ZTE Corporation | CR to TS 38.176-2 Maintenance of IAB for supported BW R17 Summary of change: Add 35MHz and 45MHz BW supported by IAB to the specification. |
| R4-2309639 | Huawei, HiSilicon | CR to TR 37.941: correction for missing 35MHz and 45MHz channel bandwidths, Rel-17 Summary of change: - Table 17-1: introduction of channel bandwidth agnostic correction of the channel bandwidths considered, to cover also 35MHz and 45MHz channel bandwidths - Other editorial corrections |

## Open issues summary

N/A