3GPP TSG-RAN WG4 Meeting #108	R4-2312840

Toulouse, France, 21 – 25 August, 2023

Title: 	Updated simulation results for PRS measurement with 1RX

Source: 	Huawei, HiSilicon

Agenda item:	8.22.3.4

Document for:	Discussion

Introduction

RRM requirements for RedCap positioning are discussed in RAN4#107, and outcomes are captured in WF [1]. Besides, An updated simulation assumption [2] is agreed based on following agreements.

Issue 2-2-1: Side conditions for 1Rx without FH: Agreements: - For AWGN channel - Re-use the Rel-17 side conditions for both 1 and 4 measurement samples - For fading channel use two side conditions for the neighbor cell to define requirements - Condition #1: -3 dB (for Rx-Tx and RSRP only) - Condition #2: - Option 1: -10 dB - Option 2: -6 dB Note: Updated simulation assumptions in R4-2310075 to identify side conditions in fading.

In this paper we will provide our updated simulation results for RedCap with 1RX.

Discussion

Based on [2], we simulated the following cases. The highlighted cases are new cases to derive the feasible Es/Iot condition based on issue 2-2-2.

- RSTD

  - 4-sample, Es/Iot of (-6,-13)dB, AWGN

  - 4-sample, Es/Iot of (-6,-13)dB, TDL-A

  - 4-sample, Es/Iot of (-6,-10)dB, TDL-A

  - 4-sample, Es/Iot of (-6,-6)dB, TDL-A

  - 1-sample, Es/Iot of (-3,-6)dB, AWGN

- UE Rx-Tx

  - 4-sample, Es/Iot of -3dB, AWGN

  - 4-sample, Es/Iot of -13dB, AWGN

  - 4-sample, Es/Iot of -3dB, TDL-A

  - 4-sample, Es/Iot of -13dB, TDL-A

  - 4-sample, Es/Iot of -10dB, TDL-A

  - 4-sample, Es/Iot of -6dB, TDL-A

  - 1-sample, Es/Iot of 0dB, AWGN

  - 1-sample, Es/Iot of -6dB, AWGN

The simulation results for RSTD are in Table 1-3 and Table 2a/2b, and UE Rx-Tx in Table 4-9 and Table 7a/7b. Current requirements for 2RX in the spec are also listed for comparison. The value 9999 means for the TOA error is very large and like a random value, i.e. the first path is not detected.

Table 1: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, AWGN

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 180 | 132 |
|  |  | 52 | 1 | 136 | 98 |
|  |  | 104 | 1 | 75 | 42 |
|  | 30 | 24 | 4 | 112 | 75 |
|  |  | 48 | 1 | 75 | 48 |
|  | 60 | 24 | 4 | 78 | 50 |

Table 2: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, TDL-A

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 267 | 247 |
|  |  | 52 | 1 | 9999 | 140 |
|  |  | 104 | 1 | 9999 | 86 |
|  | 30 | 24 | 4 | 192 | 118 |
|  |  | 48 | 1 | 9999 | 109 |
|  | 60 | 24 | 4 | 153 | 147 |

Table 2a: RSTD error for 4-sample, Es/Iot of (-6,-10)dB, TDL-A

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 150 | 247 |
|  |  | 52 | 1 | 165 | 140 |
|  |  | 104 | 1 | 110 | 86 |
|  | 30 | 24 | 4 | 118 | 118 |
|  |  | 48 | 1 | 120 | 109 |
|  | 60 | 24 | 4 | 96 | 147 |

Table 2b: RSTD error for 4-sample, Es/Iot of (-6,-6)dB, TDL-A

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 132 | 247 |
|  |  | 52 | 1 | 140 | 140 |
|  |  | 104 | 1 | 105 | 86 |
|  | 30 | 24 | 4 | 110 | 118 |
|  |  | 48 | 1 | 115 | 109 |
|  | 60 | 24 | 4 | 93 | 147 |

Table 3: RSTD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN

| Parameter | Value |  |  | RSTD error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 52 | 1 | 172 | 98 |
|  |  | 104 | 1 | 98 | 42 |
|  | 30 | 48 | 1 | 102 | 48 |

Table 4: UE Rx-Tx error for 4-sample, Es/Iot of -3dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 112 | 78 |
|  |  | 52 | 1 | 88 | 59 |
|  |  | 104 | 1 | 42 | 30 |
|  | 30 | 24 | 4 | 75 | 57 |
|  |  | 48 | 1 | 48 | 30 |
|  | 60 | 24 | 4 | 50 | 29 |

Table 5: UE Rx-Tx error for 4-sample, Es/Iot of -13dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 135 | 101 |
|  |  | 52 | 1 | 118 | 75 |
|  |  | 104 | 1 | 52 | 37 |
|  | 30 | 24 | 4 | 92 | 58 |
|  |  | 48 | 1 | 63 | 39 |
|  | 60 | 24 | 4 | 60 | 36 |

Table 6: UE Rx-Tx error for 4-sample, Es/Iot of -3dB, TDL-A

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 216 | 137 |
|  |  | 52 | 1 | 172 | 96 |
|  |  | 104 | 1 | 124 | 62 |
|  | 30 | 24 | 4 | 156 | 87 |
|  |  | 48 | 1 | 122 | 68 |
|  | 60 | 24 | 4 | 118 | 59 |

Table 7: UE Rx-Tx error for 4-sample, Es/Iot of -13dB, TDL-A

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 245 | 180 |
|  |  | 52 | 1 | 9999 | 98 |
|  |  | 104 | 1 | 9999 | 68 |
|  | 30 | 24 | 4 | 175 | 87 |
|  |  | 48 | 1 | 9999 | 85 |
|  | 60 | 24 | 4 | 144 | 139 |

Table 7a: UE Rx-Tx error for 4-sample, Es/Iot of -10dB, TDL-A

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 140 | 180 |
|  |  | 52 | 1 | 163 | 98 |
|  |  | 104 | 1 | 138 | 68 |
|  | 30 | 24 | 4 | 125 | 87 |
|  |  | 48 | 1 | 122 | 85 |
|  | 60 | 24 | 4 | 106 | 139 |

Table 7b: UE Rx-Tx error for 4-sample, Es/Iot of -6dB, TDL-A

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 24 | 4 | 120 | 180 |
|  |  | 52 | 1 | 130 | 98 |
|  |  | 104 | 1 | 113 | 68 |
|  | 30 | 24 | 4 | 119 | 87 |
|  |  | 48 | 1 | 112 | 85 |
|  | 60 | 24 | 4 | 103 | 139 |

Table 8: UE Rx-Tx error for 1-sample, Es/Iot of 0dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 52 | 1 | 75 | 59 |
|  |  | 104 | 1 | 47 | 30 |
|  | 30 | 48 | 1 | 46 | 30 |

Table 9: UE Rx-Tx error for 1-sample, Es/Iot of -6dB, AWGN

| Parameter | Value |  |  | Rx-Tx error | Current requirement |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |  |
|  | 15 | 52 | 1 | 96 | 75 |
|  |  | 104 | 1 | 58 | 37 |
|  | 30 | 48 | 1 | 55 | 39 |

Based on the results, we made the following observations.

Observation 1: For RSTD and UE Rx-Tx, with Es/Iot of -13dB and TDL-A channel, the first path detection performance is poor (detection rate <90%) with 1RX.

Observation 2: For RSTD and UE Rx-Tx, for other cases than Es/Iot of -13dB and TDL-A channel, the TOA estimation error is larger with 1RX.

Observation 3: For RSTD and UE Rx-Tx, with Es/Iot of -10dB and TDL-A channel, reasonable accuracy performance can be achieved.

Observation 4: For RSTD and UE Rx-Tx, the accuracy difference between -10dB and -6dB is minor.

The performance degradation with 1RX is due to the fact that UE cannot do combining across two RX branches, and the effective Es/Iot is lower compared to 2RX.

Conclusions

In this paper we will provide our updated simulation results for RedCap with 1RX.

Observation 1: For RSTD and UE Rx-Tx, with Es/Iot of -13dB and TDL-A channel, the first path detection performance is poor (detection rate <90%) with 1RX.

Observation 2: For RSTD and UE Rx-Tx, for other cases than Es/Iot of -13dB and TDL-A channel, the TOA estimation error is larger with 1RX.

Observation 3: For RSTD and UE Rx-Tx, with Es/Iot of -10dB and TDL-A channel, reasonable accuracy performance can be achieved.

Observation 4: For RSTD and UE Rx-Tx, the accuracy difference between -10dB and -6dB is minor.

Reference

- R4-2310072, WF on NR Positioning – RedCap and BW aggregation, Ericsson

- R4-2310075, Updated simulation assumptions for 1Rx RedCap positioning, Ericsson