3GPP TSG-RAN WG4 Meeting #108	R4-2313994

Toulouse, France, Aug 21 – Aug 25, 2023

Title:	TP for TR 38.878 Receiver structure of MU-MIMO

Source: 	ZTE Corporation

Agenda item:	8.18.1.2

Document for:	Approval

- Background

In RAN#100, the revised WID[1] on NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined.  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. In this TP, we summarized the candidate receiver structure based on previous WF[2][3][4].

Advanced receiver to cancel inter-user interference for MU-MIMO

---------------------------------------------  Start of text proposal ---------------------------------------------

4.2	Receiver structure

4.2.1	General

In this clause, we provide the receiver structure to mitigate the intra-cell inter-user interference.

The NRx-dimensional received signal vector r of the k-th subcarrier and the l-th OFDM symbol is assumed to be expressed as a sum of target’s UE own signal H1k,ld1k,l, and co-scheduled UEs’ interference signals Hjk,ldjk,l (j>1) and the white noise n(k,l);

rk, l= H1k,ld1k,l+j=2NUEHjk,ldjk,l+n(k,l)

Where,

djk,l and Hjk,l, j={2,…NUE} represent the Nlayer,j x1 transmitted signal vector and the (NRx x Nlayer,j) channel matrix between the j-th co-scheduled UE’s interference and the UE containing the contribution from receiver branches, with Hj=(Hj,1)H(Hj,2)H for two receiver antennas and Hj=(Hj,1)H(Hj,2)H(Hj,3)H(Hj,4)H for four receiver antennas, where, Hj,i channel-matrix of size Nlayer,j x1 for the i-th receiver antenna, respectively.

NUE is the number of paired UEs.

4.2.2	E-MMSE IRC receiver

To suppress the co-scheduled UE’s interference, the candidate E-MMSE IRC receiver type is captured in this subclause. The E-MMSE IRC receiver weight matrix is expressed as follow:

WRX,1k,l=H1H(k,l)R−1

R= P1H1k,lH1Hk,l+Pjj=2NUEHjk,lHjHk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H,

rk,l=rk, l−j=1NUEHjk,ldjk,l .

Where,

HjHk,landdj(k,l) denote the estimated channel matrix and the transmit signal of all UE’s DMRS symbols, respectively, where the estimated channel matrix is also based on DMRS.

Nre is the number of sampling REs of intra-user’s DMRS.

Pj is the power of j-th UE.

### 4.2.3 R-ML receiver

ML receiver performs joint maximum likelihood detection of the useful and co-scheduled UE signals considering the constellations of both signals:

rk, l= H1k,ld1k,l+j=2NUEHjk,ldjk,l+nk,l =

=[H1k,lH2k,l… HNUEk,l]d1k,ld2k,l⋮dNUEk,l+nk,l

So the ML receiver can be rewritten as

dML=argmin                   dj∈Srk,l−j=1NUEHjk,ldjk,l2

Where,

·  denotes the l2-norm of a vector.

rk, l is the received signal vector.

Hjk,l is the channel matrix over target and interference layers for a RE at frequency location k and symbol l.

S is the set of all possible transmitted signal vectors across target and interference spatial layers.

Reduced complexity ML (R-ML):

Reduced complexity joint detection of useful and interference modulation symbols in accordance to the ML criterion (e.g. sphere decoding, QR-MLD, MLM, etc.).

---------------------------------------------  End of text proposal ---------------------------------------------

Conclusions

Reference

- RP-231188.  NR demodulation performance evolution. China Telecom

- R4-2302929. WF on advanced receiver for MU-MIMO scenario. China Telecom

- R4-2305914. WF on advanced receiver for MU-MIMO scenario. China Telecom

- R4-2309892 WF on advanced receiver for MU-MIMO scenario_v02