# 3GPP TSG-RAN WG4 Meeting #108                                                             R4-2314901

# Toulouse, FR, 21st – 25th August 2023

# Title:	TP for TR38.899: CA_n77-n79, CA_n3-n77, CA_n28-n77, CA_n1-n77, CA_n1-n79, CA_n8-n79

# Source:	SoftBank, LG Electronics

# Agenda Item:	7.20.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 38.899[5] to include the following HPCA combinations as requested in RAN4#107.

- CA_n77-n79 (PC2 2 DL and 1 or 2 UL, PC1.5 2 DL and single UL)

- CA_n3-n77 (PC2 2 DL and 1 or 2 UL, PC1.5 2 DL and single UL)

- CA_n28-n77 (PC2 2 DL and 1 or 2 UL, PC1.5 2 DL and single UL)

- CA_n1-n77 (PC1.5 2 DL and single UL)

- CA_n1-n79 (PC2 2 DL and 1 or 2 UL, PC1.5 2 DL and single UL)

- CA_n8-n79 (PC2 2 DL and single UL, PC1.5 2 DL and single UL)

- Reference

[3] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone", v18.2.0.

[4] 3GPP TR37.865-01-01: “NR Carrier Aggregation for intra-band (m Down Link (DL) / 1 Up Link (UL) bands) and inter-band (n Down Link (DL) / 1 Up Link (UL) bands)”, v15.3.0

[5] 3GPP TR38.899, “High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s); (Release 18)”, v0.4.0

[6] 3GPP TR38.716-02-00: “NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 16)”, v16.0.0

[7] 3GPP TR38.717-02-01: “Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 17)” v17.0.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted --

## 5.5	CA_n77-n79

### 5.5.1	Configurations

The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. These are shown in Table 5.2A.2.1-1 of TS 38.101-1 [3].

Table 5.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | n77A8,9, n79A8,9 CA_n77A-n79A8 | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.5.2	Maximum output power

Table 5.5.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.5.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. For PC3 CA_n77-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01 [4]. For PC2 CA_n77-n79 with UL CA_n77-n79, the co-existence study is provided in TR38.717-02-01[7]. For PC2 and PC1.5 CA_n77-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01[4]. Analysis is based on these studies.

- 5.5.3.0	Power class 2 case a, b, c and d

Considering both n77 and n79 are TDD bands and synchronous operation for CA_n77-n79 is assumed, no MSD is needed for dual UL of CA_n77-n79.

#### 5.5.3.1	Power class 2 for single uplink n77

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n77.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.3	Power class 1.5 for single uplink n77

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.4	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n77.

Therefore, there is no MSD issue for this CA configuration.

### 5.5.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

-- Unaffected parts omitted --

## 5.8	CA_n3-n77

### 5.8.1	Configurations

Table 5.8.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A | n778,9 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77(2A) | n778,9 CA_n77(2A) CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n77(3A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.8.2	Maximum output power

Table 5.8.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.8.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.8.3.1	Power class 2 case a, b

Based on calculation, IMD 2, 4, 5 of dual UL CA_n3-n77 falls into n3 DL, the MSD exception is defined as below, the values are reused from CA_n3-n78.

Additionally, IMD 5, 6, 7 and 8 of CA_n77(2A) UL falls into n3 DL. And for this combination PC3 with CA_n77(2A) UL is specified, with MSD defined for IMD7. Possible problematic 2UL IMD cases are:

- IMD5: There is no need to specify the MSD because for IMD5 to fall on n3 DL, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [1].

- IMD6:  There is no need to specify the MSD because of the same reason.

- IMD7: The MSD can be evaluated using the same test configuration as for PC3

- IMD8. Because test points for IMD2 and IMD4 exist already, hence it is not recommended to add third test point for even order IMD.

As such, we recommend to specify IMD7 MSD using the same test configuration as was used for PC3. The MSD exception is defined as Table below.

Table 5.8.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB |  | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n774 | n3 | 1740 | 5 | 25 |  | 1835 | 31.9 | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 |  | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 |  | 1860 | 18.5 | FDD | IMD4 |
|  | n77 | 3435 | 10 | 50 |  | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A |  | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A |  | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 |  | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 |  | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 |  | 1 (RBstart=0) | 3945 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |  |

#### 5.8.3.2	Power class 2 for single uplink n77

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		the 2th order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC2 n77 UL falling into n3 DL should be defined, the value is reused from CA_n3-n78.

Table 5.8.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 1 | NOTE 4 | UL1/DL2 |

#### 5.8.3.3	Power class 1.5 for single uplink n77

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		the 2th order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC1.5 n77 UL falling into n3 DL should be defined.

Table 5.8.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

### 5.8.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

-- Unaffected parts omitted --

## 5.11	CA_n28-n77

### 5.11.1	Configurations

Table 5.11.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A | n778,9 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77(2A) | n778,9 CA_n77(2A) CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n77(3A) | n778,9 CA_n77(2A) CA_n28A-n77A8 | n28 | 5, 10 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.11.2	Maximum output power

Table 5.11.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.11.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.11.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n28-n77 falls into n28 DLhe MSD exception is defined as below.

Table 5.11.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |

#### 5.11.3.2	Power class 2 for single uplink n77

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC2 n77 UL falling into n28 DL should be defined, the value is reused from CA_n28-n78.

Table 5.11.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 11.7 | NOTE 1 | UL1/DL5 |

#### 5.11.3.3	Power class 1.5 for single uplink n77

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC1.5 n77 UL falling into n28 DL should be defined,.

Table 5.11.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |

### 5.11.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

-- Unaffected parts omitted --

## 5.13	CA_n1-n77

### 5.13.1	Configurations

Table 5.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A | n778,9 CA_n1A-n77A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n77(2A) | n778,9 CA_n1A-n77A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.13.2	Maximum output power

Table 5.13.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.13.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.13.3.1	Power class 2 case a, b

Based on calculation, IMD2/4/5 of dual UL falls into n1 DL, the MSD exception is defined as below, which is reused from DC_1_n77.

Table 5.13.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 | 5 | 25 | 2140 | 35.8 | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

      - Power class 2 for single uplink n77

-   No harmonic mixing of n77 falls into Rx frequencies of n1.

-   No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.13.3.3	Power class 1.5 for single uplink n77

-   No harmonic mixing of n77 falls into Rx frequencies of n1.

-   No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

### 5.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

-- Unaffected parts omitted --

# 5.x	CA_n1-n79

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n79A | n798, 9 CA_n1A-n79A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

### Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

### 5.x.3.1	Power class 2 case a, b

For PC2 CA_n1-n79, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n1-n79.

Based on TR38.716-02-00[6] Table 6.6.1.3-1, there is no harmonic issue for the band combination of n1 and n79.

Based on TR38.716-02-00[6] Table 6.6.1.3-2, there is no harmonic mixing issue for the band combination of n1 and n79.

#### 5.x.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

#### Therefore, there is no MSD issue for this CA configuration.

#### 5.x.3.3	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

# 5.x.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n1A-n79A requirements are applied for PC2 CA_n1A-n79A and PC1.5 CA_n1A-n79A.

-- Unaffected parts omitted --

# 5.x	CA_n8-n79

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n79A | n798, 9 CA_n8A-n798 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

- 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n8-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

For PC2 and PC1.5 CA_n8-n79, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n79.

According to the study results in TR 37.865-01-01[4] Table 8.14.2-1,

- 5th order IMD may fall into Rx frequencies of band n8.

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n79 configuration in TS38.101-1[3] Table 7.3A.5-1.

#### 5.x.3.1	Power class 2 case a, b

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n79A, no additional MSD are expected for this PC2 CA_n8A-n79A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n79A, no additional MSD are expected for this PC2 CA_n8A-n79A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1, it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD5 MSD for PC2 2UL/2DL CA_n8A-n79A should be defined additionally.

Table 5.x.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

|  | NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| CA_n8-n79 |  | n8 | 897.5 | 5 | 25 | 942.5 | 21.5 | FDD | IMD5 |
|  |  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD5 MSD value defined in table 5.x.3.1-1 can be applied.

#### 5.x.3.2	Power class 2 for single uplink n79

Based on above,

- 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n8.

- 5th order harmonic mixing falls into Rx frequencies of n8.

Therefore MSD issue due to harmonic mixing from PC2 n79 UL falling into n8 DL should be defined.

Table 5.x.3.2-1 Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL bands for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

#### 5.x.3.2	Power class 1.5 for single uplink n79

Based on above,

- 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n8.

- 5th order harmonic mixing falls into Rx frequencies of n8.

Therefore MSD issue due to harmonic mixing from PC2 n79 UL falling into n8 DL should be defined.

Table 5.x.3.2-1 Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL bands for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 31.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

# 5.x.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n79A requirements are applied for PC2 CA_n8A-n79A and PC1.5 CA_n8A-n79A.

-- End of TP --