# 3GPP TSG-RAN WG4 Meeting #108	R4-2314934

# Toulouse, France, August 21 – August 25 , 2023

Title: 	WF on UE RF requirements for NR NTN

Agenda Item:	8.28.6

Source: 	ZTE Corporation, Samsung

Document for:	Approval

Sub-topic 1  UE types and antenna assumption for NTN UE

Issue 1-1: Differentiate UE types from mobility perspective

Agreement:

- Define two set of requirement for fixed and mobile VSAT

  - Differentiate the electronic and mechanical steering for both fixed and mobile VSAT

  - Strive to minimize the number of different requirements.

Issue 1-2: Phase antenna array

FFS

Sub-topic 2  Noise figure for NTN UE

Issue 2-1:  Noise figure

Agreement:

- Define two sets of requirements

  - Two sets of noise figures

    - Set #1: 2.5dB for LEO and GEO

    - Set #2: 6dB only for LEO

  - [Apply to test based on UE declaration for G/T which is used to derive EIS requirements].

Sub-topic 3  RF requirements for NTN UE in Ka-band

Issue 3-1: Beam pointing/accuracy related requirements

Agreement:

For mobile VSAT

  - RAN4 to specify NTN UE VSAT pointing accuracy based on manufacturer declaration. RAN4 to reuse the explanation from EN 303 978:

“The applicant shall declare the peak pointing accuracy () and the associated statistical basis.

The antenna shall maintain the declared peak pointing accuracy (), such that the off-axis EIRP emission density pattern projected onto the geostationary arc remains within the mask specified in clause [Total EIRP density specification] when shifted by an angle of ±(°), taking into account the following factors:

the worst case operational environmental conditions;

maximum ESOMP dynamics; and

the range of latitude, longitude and altitude relative to the satellite orbital position.”

For fixed VSAT

  - Proposal 3: RAN4 should specify antenna accuracy requirement for fixed VSAT, based on ETSI EN 301 360.