3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315641

Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for FR2 multi-RX

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM impacts of NR FR2 multi-Rx chain DL reception were further discussed, with agreement captured in [1][2]. In this paper, we provide our views on performance requirements for FR2 multi-Rx.

# Discussion

Most of the issues for core requirements are concluded in last meeting, and the performance part is scheduled to be triggered in this meeting. In this contribution, we provide our views on test design and test set up for RRM requirements.

4-layer DL MIMO

As elaborated in previous meeting for core requirements, companies commented that the features are mainly targeting 4 layers DL MIMO. However, from RRM perspective, whether 4 layers MIMO can be supported not only depending on beam management but also CQI status for the chosen beam pair, which will be further evaluated by Demod requirements. Based on RF agreements [3], total 2 layers are considered for RF requirements which shown as follows. Thus, it is straightforward to follow the same principle for RRM test.

R4-2214457 - Scope definition for UE RF requirements Agreement: - Proposal: UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

Proposal 1: 4-layer MIMO is not considered in RRM test cases.

AoA selection

In current FR2 test cases, the AoA selection are defined as follows:

### A.3.15.3 Setup 3: 2 AoAs There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class. Editor Note: If RAN5 finds the changing of angular offset between the directions (AoAs) of the 2 active probes per test iteration to be infeasible from the perspectives of EIS spherical coverage and other impacts, e.g.: testing time, then the test setup will be revised.

For Multi-Rx requirements, AoA separation is an essential condition to guarantee that UE can perform simultaneous reception with limited interference. Based on RF discussion, the status is summarized as follows [4][5]:

R4-2310491 Proposals: - Option 1: UE vendors declare 2 AoA offsets for meeting requirement, one from {30⁰, 60⁰, 90⁰} and one from{120⁰, 150⁰} respectively - Option 2: 2 AoA offsets are specified in the standard as test conditions, ex; 60⁰ and 150⁰ respectively. - Option 3: UE vendors declare 1 AoA offset from {30⁰, 60⁰, 90⁰, 120⁰, 150⁰} for meeting requirement. - Option 4: requirements for 2 AoA offsets are specified, e.g. 60⁰ and 150⁰. UE vendors can declare which offset to test for meeting the requirement. - Option 5: requirements for 2 AoA offset ranges are specified, one for {30⁰, 60⁰, 90⁰} and the other for {120⁰, 150⁰}. UE vendors can declare only one offset to test for meeting the requirement of the corresponding range. R4-2314668 WF: The UE only needs to meet the requirement for 1 AoA offset. Options: - Define a requirement for each candidate AoA offset. - The requirement is defined for just 1 AoA offset.

Based on RF agreements, it could be observed that:

- UE only needs to meet requirements for 1 AoA offset

- The AoA offset can be declared by UE or pre-defined in the specification

The AoA offset also has significant impact on RRM requirements. For instance, the TCI state switching test cases are implicitly verified by demodulation performance form two AoAs. For known conditions, RAN4 reached following agreement. As explained in previous meetings, awareness of the Rx beam for two AoA doesn't necessarily mean UE can receive simultaneously. Thus, the AoA pair for test shall be carefully selected.

### Sub-topic 2-5: Known conditions Agreements: - Dual TCI states are known if the - dual TCI states are QCL-ed to reported beam pair (i.e., RS resources pair) within one group - All the RSs in the QCL chain remain detectable - The dual TCI states remains detectable during the TCI state switching period - RSs configured for dual TCI states are reported in last [1280]ms Note: FFS whether additional conditions are needed for tests.

From RRM perspective, the AoA pair selected shall meet corresponding RF requirements, and UE only needs to be tested for 1 AoA offset. The selection of AoA offset shall wait for further RF conclusion.

Proposal 2: The AoAs for test cases shall be selected from the set that meet corresponding RF requirements. The selection of AoA offset shall wait for further RF conclusion.

Number of active probes

For number of active probed needed in the test, there are relative discussion in FR2 OTA SI, with following conclusions [6].

- Option 1 (Qualcomm): Dual TCI switches simultaneously, probe number for multiple AoA test system is at least 4 Figure 5: Illustration of Dual TCI switches simultaneously with 4 probes For option 1, in the period of T1, DUT connects TCI state 0 and TCI state 1 via probe#1 and probe#2 respectively. Then in the period of T2, TCI state 0 switches to TCI state 3 via switching between probe#1 and probe#4, and in the meanwhile, TCI state 1 switches to TCI state 2 via switching between probe#2 and probe#3. - Option 2: Dual TCI switches sequentially, probe number for multiple AoA test system is at least Figure 6: Illustration of Dual TCI switches simultaneously with 3 probes For option 2, in the period of T1, DUT connects TCI state 0 via probe#1. In the period of T2, TCI state 0 (anchor TCI) firstly switches to TCI state 2 via switching between probe#1 and probe#3. Then the TCI state 1 is added via probe#2. - Option 3: Dual TCI switches simultaneously, but the beam directions are not changed, probe number for multiple AoA test system is at least 2 Figure 7: Illustration of Dual TCI switches simultaneously with 2 probes For option 3, in the period of T1, DUT connects TCI state 0 and TCI state 1 via Pol.H of probe#1 and Pol.H of probe#2, respectively. Then in the period of T2, TCI state 0 switches to TCI state 3 via switching between Pol.H and Pol.V of probe 1, and in the meanwhile, TCI state 1 switches to TCI state 2 via switching between Pol.H and Pol.V of probe 2. Note that in option 3, different SSB IDs are transmitted from two polarizations in T1 and T2.

The essential problem is how many active probes are needed for RRM test cases. It is identified that the most critical case is for dual TCI state switching from dual TCI to dual TCI. In general, there are following cases for TCI state switching requiring different number if active probes:

Table I. Number of active probes needed for RRM test cases

| Case | TCI state switching | Number of active probes |
| --- | --- | --- |
| Case 1 | Single TCI to dual TCI [RS1] to [RS1, RS2] | 2 |
| Case 2 | Single TCI to dual TCI [RS1] to [RS2, RS3] | 3 |
| Case 3 | Dual TCI to single TCI [RS1, RS2] to [RS1] | 2 |
| Case 4 | Dual TCI to single TCI [RS1, RS2] to [RS3] | 3 |
| Case 5 | Dual TCI to dual TCI [RS1, RS2] to [RS1, RS3] | 3 |
| Case 6 | Dual TCI to dual TCI [RS1, RS2] to [RS3, RS4] | 4 |

Based on above table, Case 6 is the only case where 4 active probes are needed. However, based on current agreement about the test set up, for each point in the measurement grid, it is considered as “pass” if there are two AoA with specific AoA offset can satisfied the corresponding RF requirements [4].

### 3.1 Requirement metric Agreement: - For the requirement metric for 2 AoA - For a specific angular separation between 2 TRPs and a specific UE orientation under standardized DL power level which is equal between 2 TRPs, the result at each test point is constructed based on two AoA pairs containing that test point, i.e., AoA+ pair and AoA- pair. Overall result (probability to support 2TRP DL) is by averaging regional results.

For case 6, it means the test case can only be conducted if there exists two AoA pairs from which UE can perform simultaneous reception for a single test point (assuming that the position of the DUT will not be changed during one test). In details, for the example in the figure in option 1, for a single test point, {probe#1, probe#2} and {probe#3, probe#4} should all be the AoA pairs that UE can perform simultaneous reception. In other words, for each point, there should be two AoA pairs can meet corresponding RF requirements. However, the above condition is very difficult to be met. Based on the progress on test set up, there is only limited probes in the test environment (e.g. 5 or 6), which means it is very likely that at most 1 qualified AoA pairs can be found for each test point.

Observation 1: Based on current conclusion on test set up, it is hard to guarantee that there are two AoA pairs for simultaneous reception for a single test point.

For other cases in table I, the situation is less challenge that only one AoA pairs satisfying multi-Rx RF requirements + one single AoA satisfying legacy RF requirements are needed for one test point. From our understanding, Case 1-5 can almost cover most typical scenario for TCI state switching, we don't the necessity to included Case 6 which will extremely increase the complexity of the test set up.

Proposal 3: RAN4 don't define test cases for dual TCI state from dual TCI to dual TCI (e.g. [RS1, RS2] to [RS3, RS4]) where 4 active probes are needed.

# Conclusions

Proposal 1: 4-layer MIMO is not considered in RRM test cases.

Proposal 2: The AoAs for test cases shall be selected from the set that meet corresponding RF requirements. The selection of AoA offset shall wait for further RF conclusion.

Observation 1: Based on current conclusion on test set up, it is hard to guarantee that there are two AoA pairs for simultaneous reception for a single test point.

Proposal 3: RAN4 don't define test cases for dual TCI state from dual TCI to dual TCI (e.g. [RS1, RS2] to [RS3, RS4]) where 4 active probes are needed.

# References

[1] R4-2314477 WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)

[2] R4-2314283 WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2)

[3] R4-2214457 WF on FR2 multi-Rx chain DL reception

[4] R4-2310491 WF on UE RF requirements for FR2_multiRx_UERF

[5] R4-2314668 WF on FR2_multiRx_UERF

[6] R4-2313888 WF for FR2 OTA test SI