3GPP TSG-RAN WG4 Meeting # 108-bis												R4-23XXXXX

Xiamen, China, October 09 – October 13, 2023

Agenda item:			4.33.5

Source:	Moderator (Anterix)

Title:	Topic summary for [108-bis][115] US_900MHz

Document for:	Information

# Introduction

This document is a summary of contributions submitted to agenda item 4.33.  Contributions on AAS BS requirements, BS colocation requirements, and 3 MHz band n106 GSCN specifications have been submitted.  The CRs submitted help update some of the LTE band 106 and n106 parameter with clean versions of the changes.

# Topic #1: GSCN range addition for n106

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315951 | Huawei, Hisilicon | Proposal 1: Update to band n106 in for TS 38.104 Table 5.4.3.3-1: Adding in the applicable SS raster entries per operating band (FR1) for 3 MHz CBW. |

## Open issues summary

With the sync channel definition in RAN 1 for <5 MHz NR completed in RAN1-#114, the SS Block pattern and GSCN can be defined now and updated.

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n106 | 15 kHz | Case A | 31317– <1> –31329 |

### Sub-topic 1-1

- Recommended WF

  - Suggested parameter changes are agreeable with the WID for band n106.

# Topic #2: Colocation requirements for band 106/n106

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2315776 | Ericsson | Proposal 1: Colocation tables need to be updated to support bands 106 and n106. Both are being considered to reduce editorial constraints. |

## Open issues summary

Suggested change to colocation specifications in TS 37.145-1 and TS 37.145-2 to support band 106/n106.  Proposed changes are similar with existing bands and support the WID.  Unless otherwise noted the overall WF is to support the changes in this CR.

### Sub-topic 2-1

Issue 2-1: Blocking requirement for co-location with BS in other frequency bands

- Proposals

  - Option 1:

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 106 or NR band n106 | 935 - 940 | +16 | +8 | -6 | PREFSENS + x dB (NOTE 1) | CW carrier |

- Recommended WF

  - Suggested parameter changes to Table 7.5.2.2-1 are similar to existing bands and agreeable with the WID.

### Sub-topic 2-2

Issue 2-2: Additional spurious emissions requirements

- Proposals

  - Option 1:

| System type operating in the same geographical area | Band for co-existence requirement | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| E-UTRA Band 106 or NR band n106 | 896 – 901 MHz | -120 dBm | -115 dBm | -112 dBm |

- Recommended WF

  - Suggested parameter changes to Table 9.7.6.3.3-1 are similar to other bands and agreeable for band 106/n106 coexistence specifications.

### Sub-topic 2-3

Issue 2-3: UTRA AAS BS OTA Spurious emissions limits for AAS BS co-located with another BS

- Proposals

  - Option 1:

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA-BS) | Maximum Level (MR-BS) | Maximum Level (LA-BS) | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 106 or NR band n106 | 896 – 901 MHz | -120 dBm | -115 dBm | -112 dBm | 100 kHz |  |

- Recommended WF

  - Suggested parameter changes to Table 9.7.6.3.4.2-1 are similar to other bands and agreeable for band 106/n106 spurious emissions limits specifications.

### Sub-topic 2-4

Issue 2-4: AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands

- Proposals

  - Option 1:

| System type to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 106 or NR band n106 | 935 – 940 MHz | -43 dBm | 1 MHz |  |
|  | 896 – 901 MHz | -40 dBm | 1 MHz |  |

- Recommended WF

  - In RAN4-107 it was agreed that the UE has a -30 dBm/MHz for UE-to-UE coexistence requirements for LTE and NR US 900 MHz band to protect bands 5, n5, 26, and n26.

  - The suggested parameter changes to Table 9.7.6.4.4.2-1 are agreeable with coexistence specifications defined for band 106 and should be applicable to n106 but ask BS experts to make sure these specifications are inline with each other.

### Sub-topic 2-5

Issue 2-5: OTA Blocking requirement for co-location with BS in other frequency bands

- Proposals

  - Option 1:

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 106 or or NR band n106 | 935 – 940 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |

- Recommended WF

  - Suggested parameter changes to Table 9.7.6.3.3-1 are similar to other bands and agreeable for band 106/n106 blocking requirements.

### Sub-topic 2-6

Issue 2-6: UTRA additional OTA blocking requirement for co-location with BS in other frequency bands

- Proposals

  - Option 1:

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 106 or or NR band n106 | 935 – 940 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |

- Recommended WF

  - Suggested parameter changes to Table 10.6.3.2-1 are similar to other bands and agreeable for band 106/n106 blocking requirements.

### Sub-topic 2-7

Issue 2-7: E-UTRA additional OTA blocking requirement for co-location with BS in other frequency bands

- Proposals

  - Option 1:

| Type of co-located BS | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power for WA BS [dBm] | Interfering Signal mean power for MR BS [dBm] | Interfering Signal mean power for LA BS [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band 106 or or NR band n106 | 935 – 940 | +46 | +38 | +24 | EISminSENS + x dB (NOTE 1) | CW carrier |

- Recommended WF

  - Suggested parameter changes to Table 10.6.4.2-1 are similar to other bands and agreeable for band 106/n106 blocking requirements.