3GPP TSG-RAN WG4 Meeting #109	R4-2318876

# Chicago, US, November 13 – 17, 2023

Agenda item:	8.6.2

Source: 	Xiaomi

Title: 	TP for TR 38.891 to correct some simulation results for phase noise profile

Document for:	Approval

Introduction

This contribution is to correct some simulation results for phase noise profile, our simulation results for phase noise profile evaluation were 3dB smaller due to we misunderstood the definition of SSB phase noise profile, the phase noise were artificially reduced by half in our simulation platform. But it doesn’t affect the net benefit of PTRS.

Text Proposal

<Start of Text Proposal for TR 38.891>

5.2.3 Phase noise profiles evaluation

<Text omit>

5.2.3.3 Simulation results from Xiaomi

R4-2308805

Based on the agreed simulation assumptions and candidate phase noise profiles, the EVM floor is further evaluated for CP- OFDM and DFT-s-OFDM.

For 29GHz:

Table 5.2.3.3-1 EVM floor and benefit for with PTRS CP-OFDM at 29GHz

| CP-OFDM with phase noise， 29GHz, 120kHz, 64RB，K-PTRS=2 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1 from TR38.803 | -28.1 | -28.5 | 0.4 |
| Example 2 from TR38.803 | -23.6 | -25.3 | 1.7 |
| Phase noise Profile from Qualcomm | -31.9 | -33.1 | 1.2 |
| Phase noise Profile from MTK | -32.4 | -33.0 | 0.6 |

Table 5.2.3.3-2 EVM floor and benefit for with PTRS DFT-s-OFDM at 29GHz

| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 64RB，N_group = 8, N_samp = 4 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1 from TR38.803 | -28.2 | -27.1 | -1.1 |
| Example 2 from TR38.803 | -23.1 | -27.2 | 4.1 |
| Phase noise Profile from Qualcomm | -31.8 | -32.0 | 0.2 |
| Phase noise Profile from MTK | -32.4 | -31.7 | -0.7 |

From table 5.2.3.3-1 and table 5.2.3.3-2 for 29GHz, we can see no matter for CP- OFDM and DFT-s-OFDM:

- The EVM floors for new phase noise profiles from QC and MTK are reduced obviously compared to Example 1 and Example 2.

- Further compared new phase noise profiles from QC and MTK, we can see the EVM floor of MTK’s phase noise profile is lower than Qualcomm’s when without PTRS correction. And they have similar EVM floor when adding PTRS correction. Since the CPE (common phase error) impacts is closely related to the performance of phase noise profile, it will get less benefit from PTRS correction if the phase noise profile introduce less CPE (common phase error).

Therefore, we can’t only judge the phase noise profile according to the net benefit but also need consider the performance without PTRS correction.

These two new phase noise profiles have similar performance after PTRS correction for both of CP-OFDM and DFT-S-OFDM, from this point, both of them can be used in MPR simulation.

For 39GHz:

Table 5.2.3.3-3 EVM floor and benefit for with PTRS CP-OFDM at 39GHz

| CP-OFDM with phase noise， 39GHz, 120kHz, 64RB，K-PTRS=2 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1:VIVO parameter | -24.8 | -25.1 | 0.3 |
| Example 1:Anritsu parameter | -25.1 | -25.5 | 0.4 |
| Example 2 from TR38.803 | -20.9 | -22.7 | 1.8 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -29.2 | -30.3 | 1.1 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -29.3 | -30.3 | 1.0 |
| Phase noise Profile from MTK | -29.4 | -30.0 | 0.6 |

Table 5.2.3.3-4 EVM floor and benefit for with PTRS DFT-s-OFDM at 39GHz

| CP-OFDM with phase noise， 39GHz, 120kHz, 64RB，N_group = 8, N_samp = 4 |  |  |  |
| --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS | Net benefit of PTRS |
| Example 1:VIVO parameter | -24.8 | -23.7 | -1.1 |
| Example 1:Anritsu parameter | -25.2 | -24.0 | -1.2 |
| Example 2 from TR38.803 | -20.5 | -24.4 | 3.9 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -29.2 | -29.1 | -0.1 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -29.2 | -29.2 | 0.0 |
| Phase noise Profile from MTK | -29.5 | -28.7 | -0.8 |

From table 5.2.3.3-3 and table 5.2.3.3-4 for 39GHz, we can see no matter for CP- OFDM and DFT-s-OFDM:

- The EVM floors of Example 1-based from vivo and Anritsu is better than example 2, but still worse than -29.1 dB EVM requirement for 256QAM.

- And the EVM floors of the new phase noise profiles that adopting min(Example 1-based, Example 2) and from MTK are reduced obviously compared to Example 1-based and Example 2.

- Further compared new phase noise profiles from adopting min(Example 1-based, Example 2) and MTK, we can see the EVM floor of MTK’s phase noise profile is better a little than that adopting min(Example 1-based, Example 2) a little when without PTRS correction. But the EVM floor of MTK’s phase noise profile is worse a little than that adopting min(Example 1-based, Example 2) when with PTRS correction.

These two new phase noise profiles that adopting min(Example 1-based, Example 2) and from MTK have better EVM floor than -29.1 dB EVM requirement for 256QAM whether adding PTRS correction or not for both of CP-OFDM and DFT-S-OFDM, from this point, both of them can be used in MPR simulation.

But considering phase noise profiles that adopting min(Example 1-based, Example 2) hasn’t clearly physical description, we can use the new phase noise profile from MTK in MPR simulation.

<Text omit>

5.2.3.11   Summary the simulation results for 29GHz phase noise profile evaluation

CP-OFDM:

| Companies | Waveform | Phase noise model @29GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -28.1 | -28.5 | 0.4 |
|  |  | UE example 2 |  |  | -23.6 | -25.3 | 1.7 |
|  |  | From Qualcomm |  |  | -31.9 | -33.1 | 1.2 |
|  |  | From MTK |  |  | -32.4 | -33.0 | 0.6 |
| Vivo R4-2308227 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -28.3288 | -28.9129 | 0.5841 |
|  |  | UE example 2 |  |  | -25.3244 | -25.5883 | 0.2639 |
|  |  | From Qualcomm |  |  | -31.7997 | -33.0798 | 1.2801 |
|  |  | From MTK |  |  | -33.0942 | -33.3608 | 0.2666 |
| Anritsu R4-2307043 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -28.65 |  |  |
|  |  | UE example 2 |  |  | -28.6 |  |  |
|  |  | From Qualcomm |  |  | -32.5 |  |  |
|  |  | From MTK |  |  | -32.7 |  |  |
| MTK R4-2307931 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -27.7 | -28.3 | 0.6 |
|  |  | From Qualcomm |  |  | -30.7 | -32.5 | 1.8 |
|  |  | From MTK |  |  | -31.6 | -32.7 | 1.1 |
| LGE R4-2308223 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 |  | -30.75 |  |
|  |  | From Qualcomm |  |  |  | -31.57 |  |
|  |  | From MTK |  |  |  | -31.37 |  |
| Sony R4-2309028 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -28.3 dB |  |  |
|  |  | From Qualcomm |  |  | -32.9 dB |  |  |
|  |  | From MTK |  |  | -33 dB |  |  |

DFT-s-OFDM:

| Companies | Waveform | Phase noise model @29GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB | N_group = 8, N_samp = 4 | -28.2 | -27.1 | -1.1 |
|  |  | UE example 2 |  |  | -23.1 | -27.2 | 4.1 |
|  |  | From Qualcomm |  |  | -31.8 | -32.0 | 0.2 |
|  |  | From MTK |  |  | -32.4 | -31.7 | -0.7 |
| Vivo R4-2308227 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB | N_group = 8, N_samp = 4 | -28.8567 | -27.1551 | -1.7016 |
|  |  | UE example 2 |  |  | -25.4507 | -28.9614 | 3.5107 |
|  |  | From Qualcomm |  |  | -32.8215 | -29.8556 | -2.9659 |
|  |  | From MTK |  |  | -33.2358 | -30.0408 | -3.195 |
| Sony R4-2309028 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB |  | -26.9 dB |  |  |
|  |  | From Qualcomm |  |  | -31.4 dB |  |  |
|  |  | From MTK |  |  | -31.4 dB |  |  |

5.2.3.12   Summary the simulation results for 39GHz phase noise profile evaluation

CP-OFDM

| Companies | Waveform | Phase noise model @39GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | CP-OFDM | Example 1 from VIVO | 120kHz, 64RB | L = 1, K =2 | -24.8 | -25.1 | 0.3 |
|  |  | Example 1from Anritsu |  |  | -25.1 | -25.5 | 0.4 |
|  |  | Example 2 from TR38.803 |  |  | -20.9 | -22.7 | 1.8 |
|  |  | Min(Example1-VIVO, Example2) |  |  | -29.2 | -30.3 | 1.1 |
|  |  | Min(Example1- Anritsu, Example2) |  |  | -29.3 | -30.3 | 1.0 |
|  |  | From MTK |  |  | -29.4 | -30.0 | 0.6 |
| Vivo R4-2308227 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -25.1733 | -25.3235 | 0.1502 |
|  |  | UE example 2 |  |  | -22.7246 | -22.9743 | 0.2497 |
|  |  | From Qualcomm |  |  | -29.8719 | -30.2049 | 0.333 |
|  |  | From MTK |  |  | -29.9468 | -30.182 | 0.2352 |
| Anritsu R4-2307043 | CP-OFDM | UE example 1 | 120kHz, 64RB |  | -25.2 |  |  |
|  |  | UE example 2 |  |  | -26.2 |  |  |
|  |  | From Qualcomm |  |  | -29.3 |  |  |
|  |  | From MTK |  |  | -29.7 |  |  |
| MTK R4-2307931 | CP-OFDM | UE example 1 | 120kHz, 64RB | L = 1, K =2 | -25.3 | -25.7 | 0.4 |
|  |  | From MTK |  |  | -28.7 | -29.6 | 0.9 |
| LGE R4-2308223 | CP-OFDM | Example 1 from VIVO | 120kHz, 64RB | L = 1, K =2 |  | -28.56 |  |
|  |  | Example 1 from Anritsu |  |  |  | -28.77 |  |
|  |  | From MTK |  |  |  | -30.96 |  |

DFT-s-OFDM

| Companies | Waveform | Phase noise model @39GHz | SCS, NRB | PTRS configuration | EVM (dB) with no PTRS | EVM (dB) with PTRS corrections | Net benefit of PTRS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xiaomi R4-2308805 | DFT-s-OFDM | Example 1 from VIVO | 120kHz, 64RB | N_group = 8, N_samp = 4 | -24.8 | -23.7 | -1.1 |
|  |  | Example 1from Anritsu |  |  | -25.2 | -24.0 | -1.2 |
|  |  | Example 2 from TR38.803 |  |  | -20.5 | -24.4 | 3.9 |
|  |  | Min(Example1-VIVO, Example2) |  |  | -29.2 | -29.1 | -0.1 |
|  |  | Min(Example1- Anritsu, Example2) |  |  | -29.2 | -29.2 | 0.0 |
|  |  | From MTK |  |  | -29.5 | -28.7 | -0.8 |
| Vivo R4-2308227 | DFT-s-OFDM | UE example 1 | 120kHz, 64RB | N_group = 8, N_samp = 4 | -25.3171 | -23.5189 | -1.7982 |
|  |  | UE example 2 |  |  | -22.8672 | -26.3839 | 3.5167 |
|  |  | From Qualcomm |  |  | -30.0035 | -28.8569 | -1.1466 |
|  |  | From MTK |  |  | -30.0888 | -28.5366 | -1.5522 |

5.2.5	Simulation for PTRS configurations

5.2.5.3   Simulation results from Xiaomi

R4-2308805

To further evaluate the effects of different PTRS configurations for CP- OFDM and DFT-s-OFDM, the simulations are done for different RBs allocation (4RBs, 16RBs, 64RBs) separately.

Table 5.2.5.3-1 and table 5.2.5.3-2 show the simulation results for CP- OFDM at 29GHz and 39GHz. Table 5.2.5.3-3 and table 5.2.5.3-4 show the simulation results for DFT-s- OFDM at 29GHz and 39GHz.

For CP- OFDM:

Table 5.2.5.3-1 EVM floor and benefits with different PTRS configurations for CP-OFDM at 29GHz

| CP-OFDM with phase noise， 29GHz, 120kHz, 4RB， |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1 from TR38.803 | -31.0 | -31.2 | -30.5 | 0.2 | -0.5 |
| Example 2 from TR38.803 | -23.9 | -25.1 | -24.7 | 1.2 | 0.8 |
| Phase noise Profile from Qualcomm | -32.9 | -33.9 | -33.4 | 1.0 | 0.5 |
| Phase noise Profile from MTK | -34.8 | -35.4 | -34.8 | 0.6 | 0.0 |
| CP-OFDM with phase noise， 29GHz, 120kHz, 16RB， |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1 from TR38.803 | -28.9 | -29.3 | -29.0 | 0.4 | 0.1 |
| Example 2 from TR38.803 | -23.6 | -25.3 | -25.1 | 1.7 | 1.5 |
| Phase noise Profile from Qualcomm | -32.1 | -33.2 | -33.0 | 1.1 | 0.9 |
| Phase noise Profile from MTK | -33.0 | -33.7 | -33.4 | 0.7 | 0.4 |
| CP-OFDM with phase noise， 29GHz, 120kHz, 64RB， |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1 from TR38.803 | -28.1 | -28.5 | -28.5 | 0.4 | 0.4 |
| Example 2 from TR38.803 | -23.6 | -25.3 | -25.2 | 1.7 | 1.6 |
| Phase noise Profile from Qualcomm | -31.9 | -33.1 | -33.0 | 1.2 | 1.1 |
| Phase noise Profile from MTK | -32.4 | -33.0 | -33.0 | 0.6 | 0.6 |

Table  5.2.5.3-2 EVM floor and benefits with different PTRS configurations for CP-OFDM at 39GHz

| CP-OFDM with phase noise， 39GHz, 120kHz, 4RB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1: VIVO parameter | -27.8 | -27.9 | -27.2 | 0.1 | -0.6 |
| Example 1:Anritsu parameter | -28.0 | -28.2 | -27.5 | 0.2 | -0.5 |
| Example 2 from TR38.803 | -21.2 | -22.4 | -22.1 | 1.2 | 0.9 |
| Phase noise Profile from Qualcomm; Min(Example1-VIVO, Example2) | -30.2 | -31.0 | -30.5 | 0.8 | 0.3 |
| Phase noise Profile from Qualcomm; Min(Example1- Anritsu, Example2) | -30.3 | -31.1 | -30.6 | 0.8 | 0.3 |
| Phase noise Profile from MTK | -32.0 | -32.5 | -31.8 | 0.5 | -0.2 |
| CP-OFDM with phase noise， 39GHz, 120kHz, 16RB |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1:VIVO parameter | -25.6 | -25.9 | -25.6 | 0.3 | 0.0 |
| Example 1:Anritsu parameter | -25.9 | -26.2 | -25.9 | 0.3 | 0.0 |
| Example 2 from TR38.803 | -21.3 | -22.8 | -22.5 | 1.5 | 1.2 |
| Phase noise Profile from Qualcomm; Min(Example1-VIVO, Example2) | -29.4 | -30.5 | -30.2 | 1.1 | 0.8 |
| Phase noise Profile from Qualcomm; Min(Example1- Anritsu, Example2) | -29.5 | -30.5 | -30.2 | 1.0 | 0.7 |
| Phase noise Profile from MTK | -30.1 | -30.7 | -30.4 | 0.6 | 0.3 |
| CP-OFDM with phase noise， 39GHz, 120kHz, 64RB |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| Example 1:VIVO parameter | -24.8 | -25.1 | -25.0 | 0.3 | 0.2 |
| Example 1:Anritsu parameter | -25.1 | -25.5 | -25.4 | 0.4 | 0.3 |
| Example 2 from TR38.803 | -20.9 | -22.7 | -22.7 | 1.8 | 1.8 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -29.2 | -30.3 | -30.2 | 1.1 | 1.0 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -29.3 | -30.3 | -30.3 | 1.0 | 1.0 |
| Phase noise Profile from MTK | -29.4 | -30.0 | -29.9 | 0.6 | 0.5 |

From Table 5.2.5.3-1 and table  5.2.5.3-2 for CP-OFDM, we can see although the benefits are different for different phase noise profile:

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously increased with the increase of the density of PTRS configurations.

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously increased with the number of RBs allocation. For the narrow RBs allocation, the benefits with PTRS correction is comparable with 64RBs allocation for 120kHz SCS.

For DFT-S-OFDM:

Table  5.2.5.3-3 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 29GHz

| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 4RB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -31.0 | -30.7 | -31.3 | -31.3 | -0.3 | 0.3 | 0.3 |
| Example 2 from TR38.803 | -23.7 | -27.9 | -25.9 | -23.9 | 4.2 | 2.2 | 0.2 |
| Phase noise Profile from Qualcomm | -33.0 | -33.7 | -33.9 | -33.6 | 0.7 | 0.9 | 0.6 |
| Phase noise Profile from MTK | -34.9 | -35.1 | -35.6 | -35.4 | 0.2 | 0.7 | 0.5 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 10RB |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -29.4 | -29.1 | -29.2 | -29.1 | -0.3 | -0.2 | -0.3 |
| Example 2 from TR38.803 | -23.2 | -26.7 | -24.8 | -23.1 | 3.5 | 1.6 | -0.1 |
| Phase noise Profile from Qualcomm | -32.2 | -32.6 | -32.5 | -32.2 | 0.4 | 0.3 | 0.0 |
| Phase noise Profile from MTK | -33.5 | -33.6 | -33.6 | -33.4 | 0.1 | 0.1 | -0.1 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 16RB |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -28.9 | -28.4 | -28.4 | -28.3 | -0.5 | -0.5 | -0.6 |
| Example 2 from TR38.803 | -23.1 | -26.9 | -24.9 | -23.0 | 3.8 | 1.8 | -0.1 |
| Phase noise Profile from Qualcomm | -32.0 | -32.4 | -32.1 | -31.8 | 0.4 | 0.1 | -0.2 |
| Phase noise Profile from MTK | -33.0 | -32.9 | -32.8 | -32.6 | -0.1 | -0.2 | -0.4 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 64RB |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1 from TR38.803 | -28.2 | -27.1 | -27.0 | -26.9 | -1.1 | -1.2 | -1.3 |
| Example 2 from TR38.803 | -23.1 | -27.2 | -24.8 | -22.9 | 4.1 | 1.7 | -0.2 |
| Phase noise Profile from Qualcomm | -31.8 | -32.0 | -31.6 | -31.1 | 0.2 | -0.2 | -0.7 |
| Phase noise Profile from MTK | -32.4 | -31.7 | -31.6 | -31.3 | -0.7 | -0.8 | -1.1 |

Table  5.2.5.3-4 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 39GHz

| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 4RB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections |  | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1:VIVO parameter | -27.9 |  | -27.3 | -28.0 | -28.0 | -0.6 | 0.1 | 0.1 |
| Example 1: Anritsu parameter | -28.1 |  | -27.5 | -28.2 | -28.2 | -0.6 | 0.1 | 0.1 |
| Example 2 from TR38.803 | -21.2 |  | -25.3 | -23.3 | -21.2 | 4.1 | 2.1 | 0.0 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -30.3 |  | -30.6 | -31.0 | -30.7 | 0.3 | 0.7 | 0.4 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -30.3 |  | -30.7 | -31.0 | -30.7 | 0.4 | 0.7 | 0.4 |
| Phase noise Profile from MTK | -32.1 |  | -32.1 | -32.6 | -32.5 | 0.0 | 0.5 | 0.4 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 10RB |  |  |  |  |  |  |  |  |
| Phase noise model |  | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1:VIVO parameter | -26.2 |  | -25.7 | -25.8 | -25.7 | -0.5 | -0.4 | -0.5 |
| Example 1: Anritsu parameter | -26.5 |  | -26.0 | -26.2 | -26.0 | -0.5 | -0.3 | -0.5 |
| Example 2 from TR38.803 | -20.6 |  | -24.3 | -22.3 | -20.5 | 3.7 | 1.7 | -0.1 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -29.5 |  | -29.7 | -29.6 | -29.3 | 0.2 | 0.1 | -0.2 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -29.6 |  | -29.7 | -29.6 | -29.4 | 0.1 | 0 | -0.2 |
| Phase noise Profile from MTK | -30.6 |  | -30.6 | -30.6 | -30.4 | 0.0 | 0.0 | -0.2 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 16RB |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections |  | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1: VIVO parameter | -25.7 |  | -24.9 | -25.0 | -25.0 | -0.8 | -0.7 | -0.7 |
| Example 1: Anritsu parameter | -26.0 |  | -25.3 | -25.4 | -25.2 | -0.7 | -0.6 | -0.8 |
| Example 2 from TR38.803 | -20.6 |  | -24.3 | -22.3 | -20.4 | 3.7 | 1.7 | -0.2 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -29.3 |  | -29.5 | -29.2 | -28.9 | 0.2 | -0.1 | -0.4 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -29.4 |  | -29.5 | -29.3 | -29.0 | 0.1 | -0.1 | -0.4 |
| Phase noise Profile from MTK | -30.1 |  | -29.9 | -29.8 | -29.7 | -0.2 | -0.3 | -0.4 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 64RB |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections |  | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 8, | Net benefit of PTRS N_group = 4, | Net benefit of PTRS N_group = 2, |
| Example 1: VIVO parameter | -24.8 |  | -23.7 | -23.5 | -23.4 | -1.1 | -1.3 | -1.4 |
| Example 1: Anritsu parameter | -25.2 |  | -24.0 | -24.0 | -23.9 | -1.2 | -1.2 | -1.3 |
| Example 2 from TR38.803 | -20.5 |  | -24.4 | -22.1 | -20.3 | 3.9 | 1.6 | -0.2 |
| Phase noise Profile from Qualcomm Min(Example1-VIVO, Example2) | -29.2 |  | -29.1 | -28.8 | -28.3 | -0.1 | -0.4 | -0.9 |
| Phase noise Profile from Qualcomm Min(Example1- Anritsu, Example2) | -29.2 |  | -29.2 | -28.8 | -28.3 | 0.0 | -0.4 | -0.9 |
| Phase noise Profile from MTK | -29.5 |  | -28.7 | -28.5 | -28.3 | -0.8 | -1.0 | -1.2 |

From Table 5.2.5.3-3 and table 5.2.5.3-4 for DFT-s-OFDM at 29GHzand 39GHz, we can see although the benefits are different for different phase noise profile:

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously increased with the increase of the density of PTRS configurations.

- The benefits with PTRS correction for all candidate phase noise profiles are monotonously reduced with the number of RBs allocation.

- There is obvious benefit with PTRS correction only for example 2 new phase noise profiles.

- There is no obvious benefits with PTRS correction for the RBs allocation larger than 10RBs for new phase noise profiles from Qualcomm and MTK, even some PTRS configurations have the penalty for the new phase noise profiles.

- There is obvious benefits with PTRS correction for the RBs allocation less than 10RBs for new phase noise profiles from Qualcomm and MTK, especially for (N_group = 4, N_samp = 4) PTRS configuration, i.e., 1dB and 0.7dB benefits for 29GHz for 4RBs allocation for 120kHz SCS, 0.8dB and 0.6dB for 39GHz for 4RBs allocation for 120kHz SCS.

As we know, introducing PTRS will increase the data code rate, especially for narrow RB allocation. In order to check the impact of PTRS, we further compare the BLER performance between PTRS off and PTRS on with different RB allocation for DFT-S-OFDM waveform. The simulation assumption is same with the agreed for evaluation of phase noise profiles in lasting meeting and QC phase noise profile at 29GHz is applied. The background noise is set to -35dB. As we can see from above simulation results, the BLER increase for both 4RB and 64RB allocation when PTRS is configured shown as Table 5.2.5.3-5. For 4RB allocation, although there is EVM benefit as previous analyzation, the BLER increase due to high PTRS overhead. For 64RB, the BLER increase due to the joint effect of EVM penalty and overhead of PTRS. For 10RB, maybe there is a trade-off between PTRS overhead and EVM benefit, the BLER is same for PTRS on/off. In short, except for the case that example 2 phase noise profile is applied for MPR simulation, it may not be recommended to configure PTRS.

Table  5.2.5.3-5 the BLER for different RBs allocation with/without PTRS corrections

| BLER | 4 RBs | 10 RBs | 64 RBs |
| --- | --- | --- | --- |
| Without PTRS corrections | 0.5 | 0.5 | 0.6680 |
| With PTRS correction | 0.75 | 0.5 | 0.7075 |

R4-2312686

For CP-OFDM, based on the submitted simulation results in last meeting [R4-2308805], we can see the benefits with PTRS correction are monotonously increased with the number of RBs allocation. If adopting the fixed PTRS configuration of L-PTRS = 1 K-PTRS =2, it is good for full RBs allocation in EVM test, but the overhead for partial RBs allocation need consider.

Table 5.2.5.3-6: PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

For DFT-s-OFDM, based PTRS group pattern in Table 5.2.5.3-6, we introduced more simulation cases and the simulation results for some PTRS configurations under different RB allocations are shown in Table 5.2.5.3-7 and Table 5.2.5.3-8.

Table 5.2.5.3-7 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 29GHz

| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 4RB |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -33.0 | -33.2 | -33.6 | -33.5 | -33.9 | -33.7 | 0.2 | 0.6 | 0.5 | 0.9 | 0.7 |
| Phase noise Profile from MTK | -34.9 | -35.0 | -35.4 | -35.2 | -35.6 | -35.1 | 0.1 | 0.5 | 0.3 | 0.7 | 0.2 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 8RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -32.4 | -32.1 | -32.5 | -32.5 | -32.8 | -32.9 | -0.3 | 0.1 | 0.1 | 0.4 | 0.5 |
| Phase noise Profile from MTK | -33.8 | -33.3 | -33.8 | -33.5 | -34.0 | -34.0 | -0.5 | 0.0 | -0.3 | 0.2 | 0.2 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 10RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -32.2 | -31.8 | -32.2 | -32.2 | -32.5 | -32.6 | -0.4 | 0.0 | 0.0 | 0.3 | 0.4 |
| Phase noise Profile from MTK | -33.5 | -32.8 | -33.4 | -33.0 | -33.6 | -33.6 | -0.7 | -0.1 | -0.5 | 0.1 | 0.1 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 16RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -32.0 | -31.3 | -31.8 | -31.8 | -32.1 | -32.4 | -0.7 | -0.2 | -0.2 | 0.1 | 0.4 |
| Phase noise Profile from MTK | -33.0 | -32.0 | -32.6 | -32.3 | -32.8 | -32.9 | -1.0 | -0.4 | -0.7 | -0.2 | -0.1 |
| DFT-S-OFDM with phase noise， 29GHz, 120kHz, 20RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from Qualcomm | -32.0 | -31.3 | -31.6 | -31.8 | -32.0 | -32.3 | -0.7 | -0.4 | -0.2 | 0.0 | 0.3 |
| Phase noise Profile from MTK | -32.9 | -31.8 | -32.4 | -32.1 | -32.6 | -32.6 | -1.1 | -0.5 | -0.8 | -0.3 | -0.3 |

Table 5.2.5.3-8 EVM floor and benefits with different PTRS configurations for DFT-S-OFDM at 39GHz

| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 4RB |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -32.1 | -32.1 | -32.5 | -32.2 | -32.6 | -32.1 | 0.0 | 0.4 | 0.1 | 0.5 | 0.0 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 8RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -30.9 | -30.3 | -30.8 | -30.5 | -31.0 | -31.0 | -0.6 | -0.1 | -0.4 | 0.1 | 0.1 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 10RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -30.6 | -29.8 | -30.4 | -30.0 | -30.6 | -30.6 | -0.8 | -0.2 | -0.6 | 0.0 | 0.0 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 16RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -30.1 | -29.0 | -29.7 | -29.3 | -29.8 | -29.9 | -1.1 | -0.4 | -0.8 | -0.3 | -0.2 |
| DFT-S-OFDM with phase noise， 39GHz, 120kHz, 20RB |  |  |  |  |  |  |  |  |  |  |  |
| Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS (N_group, N_samp) |  |  |  |  | Net benefit of PTRS (N_group, N_samp) |  |  |  |  |
|  |  | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) | (2, 2) | (2, 4) | (4, 2) | (4, 4) | (8, 4) |
| Phase noise Profile from MTK | -30.0 | -28.8 | -29.4 | -29.1 | -29.5 | -29.6 | -1.2 | -0.6 | -0.9 | -0.5 | -0.4 |

From above tables for DFT-s-OFDM at 29GHzand 39GHz, we can see:

- The benefits with PTRS correction are monotonously increased with the increase of the density of PTRS configurations.

- The benefits with PTRS correction are monotonously reduced with the number of RBs allocation.

- There is no obvious benefits with PTRS correction for the RBs allocation larger than 10RBs, even some PTRS configurations have the penalty.

- There is obvious benefits with PTRS correction for the RBs allocation less than 10RBs, especially for (N_group = 4, N_samp = 4) PTRS configuration.

5.2.5.5   Summary the CP-OFDM simulation results for PTRS configuration

For CP-OFDM at 29GHz:

|  |  | Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS K-PTRS=2 | EVM (dB) with PTRS K-PTRS=4 | Net benefit of PTRS K-PTRS=2 | Net benefit of PTRS K-PTRS=4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm R4-2304601 | 120kHz, 4RBs | min(example1, example2) | -33.7 | -34.4 | -34.0 | 0.7 | 0.3 |
|  | 120kHz, 16RBs |  | -33.0 | -33.9 | -33.6 | 0.9 | 0.6 |
|  | 120kHz, 64RBs |  | -32.7 | -33.5 | -33.5 | 0.8 | 0.8 |
|  | 120kHz, 256RBs |  | -32.5 | -33.4 | -33.3 | 0.9 | 0.8 |
| Xiaomi R4-2308805 | 120kHz, 4RBs | UE example 1 | -31.0 | -31.2 | -30.5 | 0.2 | -0.5 |
|  |  | UE example 2 | -23.9 | -25.1 | -24.7 | 1.2 | 0.8 |
|  |  | From Qualcomm | -32.9 | -33.9 | -33.4 | 1.0 | 0.5 |
|  |  | From MTK | -34.8 | -35.4 | -34.8 | 0.6 | 0.0 |
|  | 120kHz, 16RBs | UE example 1 | -28.9 | -29.3 | -29.0 | 0.4 | 0.1 |
|  |  | UE example 2 | -23.6 | -25.3 | -25.1 | 1.7 | 1.5 |
|  |  | From Qualcomm | -32.1 | -33.2 | -33.0 | 1.1 | 0.9 |
|  |  | From MTK | -33.0 | -33.7 | -33.4 | 0.7 | 0.4 |
|  | 120kHz, 64RBs | UE example 1 | -28.1 | -28.5 | -28.5 | 0.4 | 0.4 |
|  |  | UE example 2 | -23.6 | -25.3 | -25.2 | 1.7 | 1.6 |
|  |  | From Qualcomm | -31.9 | -33.1 | -33.0 | 1.2 | 1.1 |
|  |  | From MTK | -32.4 | -33.0 | -33.0 | 0.6 | 0.6 |
| Vivo R4-2308227 | 120kHz, 4RBs | From Qualcomm | -33.9173 | -35.0024 |  | 1.0851 |  |
|  |  | From MTK | -35.6699 | -36.7056 |  | 1.0357 |  |
|  | 120kHz, 8RBs | From Qualcomm | -33.3388 | -33.9874 |  | 0.6486 |  |
|  |  | From MTK | -34.4442 | -34.8899 |  | 0.4457 |  |
|  | 120kHz, 16RBs | From Qualcomm | -33.4905 | -33.9333 |  | 0.4428 |  |
|  |  | From MTK | -33.9908 | -34.2379 |  | 0.2471 |  |
|  | 120kHz, 32RBs | From Qualcomm | -32.5213 | -34.0762 |  | 1.5549 |  |
|  |  | From MTK | -33.2888 | -34.1761 |  | 0.8873 |  |
|  | 120kHz, 64RBs | From Qualcomm | -31.7997 | -33.0798 |  | 1.2801 |  |
|  |  | From MTK | -33.0942 | -33.3608 |  | 0.2666 |  |

5.2.5.5   Summary the DFT-s-OFDM simulation results for PTRS configuration

For DFT-s-OFDM at 29GHz:

|  |  | Phase noise model | EVM (dB) with no PTRS corrections | EVM (dB) with PTRS N_group = 8, N_samp = 4 | EVM (dB) with PTRS N_group = 4, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 4 | EVM (dB) with PTRS N_group = 2, N_samp = 2 | Net benefit of PTRS N_group = 8, N_samp = 4 | Net benefit of PTRS N_group = 4, N_samp = 4 | Net benefit of PTRS N_group = 2, N_samp = 4 | Net benefit of PTRS N_group = 2, N_samp = 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm R4-2304601 | 120kHz, 4RBs | min(example1, example2) | -33.8 |  | -35.3 |  |  |  | 1.5 |  |  |
|  | 120kHz, 16RBs |  | -32.9 | -33.6 |  |  |  | 0.7 |  |  |  |
|  | 120kHz, 64RBs |  | -32.7 | -32.7 |  |  |  | 0 |  |  |  |
|  | 120kHz, 256RBs |  | -32.5 | -32.3 |  |  |  | -0.2 |  |  |  |
| Xiaomi R4-2308805 | 120kHz, 4RBs | UE example 1 | -31.0 | -30.7 | -31.3 | -31.3 |  | -0.3 | 0.3 | 0.3 |  |
|  |  | UE example 2 | -23.7 | -27.9 | -25.9 | -23.9 |  | 4.2 | 2.2 | 0.2 |  |
|  |  | From Qualcomm | -33.0 | -33.7 | -33.9 | -33.6 | -33.2 | 0.7 | 0.9 | 0.6 | 0.2 |
|  |  | From MTK | -34.9 | -35.1 | -35.6 | -35.4 | -35.0 | 0.2 | 0.7 | 0.5 | 0.1 |
|  | 120kHz, 10RBs | UE example 1 | -29.4 | -29.1 | -29.2 | -29.1 |  | -0.3 | -0.2 | -0.3 |  |
|  |  | UE example 2 | -23.2 | -26.7 | -24.8 | -23.1 |  | 3.5 | 1.6 | -0.1 |  |
|  |  | From Qualcomm | -32.2 | -32.6 | -32.5 | -32.2 | -31.8 | 0.4 | 0.3 | 0.0 | -0.4 |
|  |  | From MTK | -33.5 | -33.6 | -33.6 | -33.4 | -32.8 | 0.1 | 0.1 | -0.1 | -0.7 |
|  | 120kHz, 16RBs | UE example 1 | -28.9 | -28.4 | -28.4 | -28.3 |  | -0.5 | -0.5 | -0.6 |  |
|  |  | UE example 2 | -23.1 | -26.9 | -24.9 | -23.0 |  | 3.8 | 1.8 | -0.1 |  |
|  |  | From Qualcomm | -32.0 | -31.3 | -31.8 | -31.8 | -31.3 | 0.4 | 0.1 | -0.2 | -0.7 |
|  |  | From MTK | -33.0 | -32.9 | -32.8 | -32.6 | -32.0 | -0.1 | -0.2 | -0.4 | -1 |
|  | 120kHz, 64RBs | UE example 1 | -28.2 | -27.1 | -27.0 | -26.9 |  | -1.1 | -1.2 | -1.3 |  |
|  |  | UE example 2 | -23.1 | -27.2 | -24.8 | -22.9 |  | 4.1 | 1.7 | -0.2 |  |
|  |  | From Qualcomm | -31.8 | -32.0 | -31.6 | -31.1 |  | 0.2 | -0.2 | -0.7 |  |
|  |  | From MTK | -32.4 | -31.7 | -31.6 | -31.3 |  | -0.7 | -0.8 | -1.1 |  |
| Vivo R4-2308227 | 120kHz, 4RBs | From Qualcomm | -34.7217 | -34.3119 |  |  |  | -0.4098 |  |  |  |
|  |  | From MTK | -36.4809 | -35.7798 |  |  |  | -0.7011 |  |  |  |
|  | 120kHz, 8RBs | From Qualcomm | -33.3038 | -32.9937 | -33.946 | -32.8345 |  | -0.3101 | 0.6422 | -0.4693 |  |
|  |  | From MTK | -34.5513 | -34.0163 | -34.7963 | -34.022 |  | -0.535 | 0.245 | -0.5293 |  |
|  | 120kHz, 16RBs | From Qualcomm | -35.5055 | -33.3672 |  |  |  | -2.1383 |  |  |  |
|  |  | From MTK | -34.1009 | -32.5789 |  |  |  | -1.522 |  |  |  |
|  | 120kHz, 32RBs | From Qualcomm | -32.6484 | -30.7554 |  |  |  | -1.893 |  |  |  |
|  |  | From MTK | -33.4813 | -31.4505 |  |  |  | -2.0308 |  |  |  |
|  | 120kHz, 64RBs | From Qualcomm | -32.8215 | -29.8556 |  |  |  | -2.9659 |  |  |  |
|  |  | From MTK | -33.2358 | -30.0408 |  |  |  | -3.195 |  |  |  |
| ZTE R4-2314619 | 120kHz, 8RBs | From MTK | -31.9 |  |  |  | -31.4 |  |  |  | -0.5 |
|  | 120kHz, 20RBs |  | -31.6 |  | -31.5 |  |  |  | -0.1 |  |  |
|  | 120kHz, 20RBs |  | -30.7 |  |  | -30.8 |  |  | 0.1 |  |  |
|  | 120kHz, 66RBs |  | -30.8 | -30.9 |  |  |  |  | 0.1 |  |  |