3GPP TSG-RAN WG4 Meeting # 109		R4-2319218

Chicago, US, November 13 – 17, 2023

Agenda item:		8.29.3

Source:		Samsung

Title:	Discussion on RRM Performance part for Rel-18 MIMO evolution for Downlink and Uplink

Document for:		Discussion

- Introduction

In last RAN4#108-bis meeting, RAN4 start the discussion on RRM performance part of Rel-18 MIMO evolution for Downlink and Uplink. In this contribution, we continue to discuss for performance part.

- Discussion

In last RAN4#108-bis meeting, TDCP measurement accuracy is shown as:

Issue 5-1-2: TDCP measurement accuracy Way forward: - Option 1: - Depends on the conclusion of feasibility study in Topic #2. - Option 2: - Not to define measurement accuracy requirements for TRS based TDCP reporting

In last RAN4#108-bis meeting, no conclusion for the feasibility study of TDCP measurement accuracy. According to the guidance, the final conclusion for the feasibility study will be concluded in Nov. meeting.

Proposal 1: By the conclusion of feasibility study, if RAN4 to agree introduce TDCP accuracy reequipments for amplitude reporting, new measurement accuracy requirement of TDCP should be specified. Otherwise, no measurement accuracy requirements for TDCP.

Issue 5-2-1: Test case list Way forward: - FFS on test case list

For the test cases design, it needs to define some test cases to verify the new core requirements.

For whether to define TDCP accuracy test cases, it depends on the conclusion of feasibility study.

According to agreement to reuse Rel-17 SRS antenna port switching. All the legacy test cases are based on 1 SRS symbol. One approach is to add new test cases for 8TX for 2 SRS symbols. But considering no RF supported for 8TX, it is also fine to no new test case for the UE test in this release.

For two TA feature, the core part requirements are for MRTD/MTTD/Uplink transmit timing, we think the test case can be defined for uplink transmit timing for two cells (with timing difference) to verify the requirements.

For active downlink/UL TCI state switching delay, the test cases for MAC-CE based are specified in Rel-17 as:

EN-DC + FR2 + Serving Cell + Joint TCI + not in the active list

EN-DC + FR2 + Serving Cell + UL TCI + PL-RS is not maintained

EN-DC + FR2 + Cell with different PCI + DL TCI + not in the active list

NR SA + FR2 + Serving Cell + Joint TCI + not in the active list

NR SA + FR2 + Serving Cell + UL TCI + PL-RS is not maintained

NR SA + FR2 + Cell with different PCI + DL TCI + not in the active list

In Rel-18, if combined the multi-TRP, the situation is more complexity. To reduce the test burden, we want to not repeat every similar test as Rel-17, but only pick up the difference requirements for the test purpose.

Whether to cover the tests for both NR and EN-DC, we don’t have strong view. If no test concern from companies, RAN4 can add test cases for both NR and EN-DC.

TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + one is known; one is unknown

TC-x: NR FR2, intra-cell (Serving cell ) sDCI + UL TCI + dual TCI state switching + both two are known

TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + two are unknown

TC-x: NR FR2, inter-cell mDCI + DL TCI + both two are known, RTD<CP

TC-x: NR FR2, inter-cell mDCI + UL TCI + both two are known, RTD>CP

Proposal 2: It is proposed to add test cases as below table:

| Test purpose | Test cases |
| --- | --- |
| [TDCP accuracy test] | - Depends on the conclusion of TDCP |
| Interruptions at SRS antenna port switching | - No new test case. |
| Uplink transmit timing for two cells to support two TA | - NR SA in FR1 - NR SA in FR2 |
| m-TRP MAC-CE based TCI state switch delay | TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + one is known; one is unknown TC-x: NR FR2, intra-cell (Serving cell ) sDCI + UL TCI + dual TCI state switching + both two are known TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + two are unknown TC-x: NR FR2, inter-cell mDCI + DL TCI + both two are known, RTD<CP TC-x: NR FR2, inter-cell mDCI + UL TCI + both two are known, RTD>CP - If no test concern from companies, RAN4 can add test cases for both NR and EN-DC. |

- Conclusion

In this contribution, we continue the discussion what is the impact on RRM performance part and provide some proposals are:

Proposal 1: By the conclusion of feasibility study, if RAN4 to agree introduce TDCP accuracy reequipments for amplitude reporting, new measurement accuracy requirement of TDCP should be specified. Otherwise, no measurement accuracy requirements for TDCP.

Proposal 2: It is proposed to add test cases as below table:

| Test purpose | Test cases |
| --- | --- |
| [TDCP accuracy test] | - Depends on the conclusion of TDCP |
| Interruptions at SRS antenna port switching | - No new test case. |
| Uplink transmit timing for two cells to support two TA | - NR SA in FR1 - NR SA in FR2 |
| m-TRP MAC-CE based TCI state switch delay | TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + one is known; one is unknown TC-x: NR FR2, intra-cell (Serving cell ) sDCI + UL TCI + dual TCI state switching + both two are known TC-x: NR FR2, intra-cell (Serving cell ) sDCI + DL TCI + dual TCI state switching + two are unknown TC-x: NR FR2, inter-cell mDCI + DL TCI + both two are known, RTD<CP TC-x: NR FR2, inter-cell mDCI + UL TCI + both two are known, RTD>CP - If no test concern from companies, RAN4 can add test cases for both NR and EN-DC. |

- Reference

[1] R4-2317371, WF on R18 NR MIMO RRM requirement, Samsung