3GPP TSG-RAN WG4 Meeting # 109				           R4-2319262

Chicago, US, November 13 – 17, 2023

Agenda Item:	8.22.2.6

Source: 	CATT

Title:	Updated simulation assumptions for CPP measurement

Document for:	Approval

- Introduction

In this contribution, the link level simulation assumption for DL RSCPD is captured. Companies are encouraged to provide simulation results based on the agreed performance metric in section 3.

- Simulation assumptions

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell 3>, where cell 1 is the reference cell |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TDD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz - FDD, TDD | - 40 GHz - 60 kHz, 120 kHz - TDD |
| Propagation conditions | AWGN, TDL-A (30 ns delay spread, 5Hz), Two-tap channel defined in 38.101-4 Annex B.2.4, (a = 1, τd=0.45 µs and fD=5 Hz) |  |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | For RSCPD: (-6, -13, -13)dB for NSample = 4; (-3, -6, -6)dB for NSample = 1. (-3, -13, -13)dB for NSample = 4; (0, -6, -6)dB for NSample = 1. |  |
| Number of UE receive antennas | 2 |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 2 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: Companies are encouraged to check suitability of the parameters and the values. |  |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  | 60, FR1 | 24 | 4 | 64 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1, 4 |  |  |  |
| PRS periodicity | 40ms, 200ms |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |
| NOTE 1: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. NOTE 2: Companies are encouraged to check suitability of the parameters and the values. |  |  |  |  |

- Performance metric

At least the following performance characteristics are to be provided for DL RSCPD:

- RSCPD error CDFs for the 2 neighbour cells

- 5%-ile and 95%-ile of the RSCPD errors

In the above,

- RSCPD error = estimated RSCPD – ideal RSCPD (based on perfect channel knowledge).

- The unit of the RSCPD measurement result is degree.

- References

- R4-2009127, “Link-level simulation assumptions for UE Rx-Tx time difference measurements”, Qualcomm.

- R4-2211061, “Simulation assumption for PRS-RSRPP”, Huawei.

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”, Huawei.

- R4-2310078 Simulation assumptions for CPP measurement, CATT