3GPP TSG-RAN WG4 Meeting # 110	R4-2400526

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.1.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#109 meeting, a WF related to 8Rx UE requirements was agreed [1]. In this contribution, we provide our views for the remaining issues.

- Discussion

Issue 2-2: How to align the ideal results alignment - Proposals - Option 1: Remove the farthest outlier from the average results (the methodology used from NR BS Rel-15) - Option 2: Set the max allowed span to 3dB Issue 2-3: Margin to be added on top of the averaged impairment results for requirements derivation - Proposal - Rank2 and Rank8 - Add 0.8dB margin for 64QAM (the methodology used from NR UE Rel-15) - Rank4 - Option 1: Add 0.8dB margin for 64QAM (the methodology used from NR UE Rel-15) - Option 2: Add 1.5dB margin for 64QAM - Option 2: Define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively - Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15) - Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results (e.g., 1.2dB); i.e., When UE supports maxNumberMIMO-LayersPDSCH=4, an additional x dB margin is added to the SNR requirement

According to simulation results collection in the last meeting [2], the spans for alignment results are still quite large (over 2.5 dB). To deal the issue of large span in the simulation results alignment. We propose to set the max allowed span to 3dB.

Proposal 1: To deal the issue of large span in the simulation results alignment. We propose to set the max allowed span to 3dB.

As for rank4, the simulation results are quite diverse as the it may come from the UE implementation about 8Rx processing. Therefore, we propose to define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively.

Proposal 2: Define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively

- Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15)

- Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results. When UE supports maxNumberMIMO-LayersPDSCH=4, an additional 1.5 dB margin (on top of 0.8dB margin in set A) is added to the SNR requirement

Issue 3-1: Antenna correlation for carrier with Rank 2 in 8Rx CA test - Proposal - Option 1: Revisit Rank 2 to TDLA30-10 Low - Option 2: Keep TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) that is same as Rank 2 single carrier test Note: Interested companies can provide simulation results with both channel conditions TDLA30-10 Low and TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) for the next RAN4#110 meeting

For rank2 CA requirements, we provide our results for TDLA30-10 Low in [3]. We slightly prefer to follow legacy CA requirements using ULA low channel to define requirements.

Proposal 3: We slightly prefer to follow legacy CA requirements using ULA low channel to define rank2 CA requirements.

Issue 3-2: Number of HARQ process for 8Rx CA test - Proposal HARQ process number; CCs with the same duplex mode & SCS with Pcell; CCs with different duplex mode / SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; 4; 8. TDD PCell; Option 1: 8 Option 2: For CC with Rank 2: 10 For CC with Rank 8: 8 Other options not precluded; 8. FDD 15 kHz + FDD 15 kHz CA; FDD PCell; 4; N/A. TDD 30 kHz + TDD 30 kHz CA; TDD PCell; Option 1: 8 Option 2: For CC with Rank 2: 10 For CC with Rank 8: 8 Other options not precluded; N/A. Issue 3-3: K1 value for 8Rx CA test - Proposal The number of slots between PDSCH and corresponding HARQ-ACK information; CCs with the same duplex mode and SCS with Pcell; CCs with different duplex mode and/or SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; {2}; {2}. TDD PCell; Option 1: For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} Option 2: For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} Other options not precluded; {7,5,4,11,9}. FDD 15 kHz + FDD 15 kHz CA; FDD PCell; {2}; N/A. TDD 30 kHz + TDD 30 kHz CA; TDD PCell; Option 1: For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} Option 2: For CC with Rank2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} Other options not precluded; N/A.

For the number of HARQ process and K1 value in CA requirements, we have the following proposals.

Proposal 4: For CA requirements, we propose to use the following configuration for the number of HARQ process and K1 value.

Number of HARQ process for 8Rx CA test

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

K1 value for 8Rx CA test

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

- Conclusion

In this contribution, we provide our views for 8Rx PDSCH single carrier and CA requirements.

Proposal 1: To deal the issue of large span in the simulation results alignment. We propose to set the max allowed span to 3dB.

Proposal 2: Define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively

- Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15)

- Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results. When UE supports maxNumberMIMO-LayersPDSCH=4, an additional 1.5 dB margin (on top of 0.8dB margin in set A) is added to the SNR requirement

Proposal 3: We slightly prefer to follow legacy CA requirements using ULA low channel to define rank2 CA requirements.

Proposal 4: For CA requirements, we propose to use the following configuration for the number of HARQ process and K1 value.

Number of HARQ process for 8Rx CA test

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

K1 value for 8Rx CA test

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

- Reference

- R4-2316914, “WF for 8Rx and 4Tx performance requirements”, Huawei, HiSilicon

- R4-2319226, “Simulation results collection for 8 Rx UE demodulation requirements”, Ericsson

- R4-2400527, “Simulation results on PDSCH requirements for 8Rx UE”, MediaTek