3GPP TSG-RAN WG4 Meeting # 110	R4-2400529

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.27.2

Source:	MediaTek Inc.

Title:	Discussion on PDCCH requirements for DSS enhancement

Document for:	Discussion

- Introduction

In RAN4#109 meeting, a WF for DSS enhancement requirements was agreed [1]. In this contribution, we provide our views on remaining issues.

- Discussion

There are some remaining issues after RAN4#109.

Antenna configuration Way forward: - Option 1: 2x2, 2x4, 4x2, 4x4 - Option 2: 4x2, 4x4 only Channel bandwidth Agreement: - For FDD, consider 10MHz/15kHz Way forward: - For TDD, - Option 1: 10MHz/15kHz - Option 2: 20MHz/15kHz Channel model Agreement: - TDLA30-10 and TDLC300-100 - Discuss whether to down-select in the next meeting

For antenna configuration, as RAN4 agreed to consider 4 LTE CRS ports, we prefer to follow legacy LTE-NR coexistences test cases in TS38.101-4 to define requirements with 4Tx.

Observation 1: All legacy LTE-NR coexistence requirements use 4Tx when considering 4 CRS ports.

Proposal 1: Define PDCCH requirements in DSS enhancement WI with 4Tx only, i.e., 4x2 and 4x4 only.

For TDD channel bandwidth, we do not have strong preference for 10MHz or 20MHz. As RAN4 had used 10MHz for FDD, we propose to use 20MHz for TDD requirements.

Proposal 2: Define PDCCH requirements for TDD with 20MHz/15kHz.

Considering the test coverage and saving the test cost, we propose to define only 4 requirements for PDCCH requirements in DSS scenario.

Proposal 3: Define 4 requirements for PDCCH requirements in DSS scenarios:

- FDD, 4T2R, AL4, TDLA30-10 (in Clause 5.3.2.1.6)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | TBD | TDLA30-10 | 4x2 Low | 1 | TBD |

- TDD, 4T2R, AL8, TDLC300-100 (in Clause 5.3.2.2.6)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 | 48 | 2 | 8 | TBD | TDLC300-100 | 4x2 Low | 1 | TBD |

- FDD, 4T4R, AL4, TDLA30-10 (in Clause 5.3.3.1.5)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | TBD | TDLA30-10 | 4x4 Low | 1 | TBD |

- TDD, 4T4R, AL8, TDLC300-100 (in Clause 5.3.3.2.5)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 | 48 | 2 | 8 | TBD | TDLC300-100 | 4x4 Low | 1 | TBD |

- Conclusion

In this contribution, we provide views about the PDCCH performance requirements for DSS enhancement WI. The observations and proposals are summarized as below.

Observation 1: All legacy LTE-NR coexistence requirements use 4Tx when considering 4 CRS ports.

Proposal 1: Define PDCCH requirements in DSS enhancement WI with 4Tx only, i.e., 4x2 and 4x4 only.

Proposal 2: Define PDCCH requirements for TDD with 20MHz/15kHz.

Proposal 3: Define 4 requirements for PDCCH requirements in DSS scenarios:

- FDD, 4T2R, AL4, TDLA30-10 (in Clause 5.3.2.1.6)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | TBD | TDLA30-10 | 4x2 Low | 1 | TBD |

- TDD, 4T2R, AL8, TDLC300-100 (in Clause 5.3.2.2.6)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 | 48 | 2 | 8 | TBD | TDLC300-100 | 4x2 Low | 1 | TBD |

- FDD, 4T4R, AL4, TDLA30-10 (in Clause 5.3.3.1.5)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | TBD | TDLA30-10 | 4x4 Low | 1 | TBD |

- TDD, 4T4R, AL8, TDLC300-100 (in Clause 5.3.3.2.5)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 | 48 | 2 | 8 | TBD | TDLC300-100 | 4x4 Low | 1 | TBD |

- Reference

- R4-2321139, “WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements”, Ericsson