3GPP TSG-RAN4 Meeting #110	R4-2400541

# Athens, Greece, Feb 26 2024 – Mar 1 2024

Source: 	Qualcomm Incorporated

Title: 	View on RRM performance requirements for FR1+FR1 NR-DC

Agenda item:	8.4.4

Document for:	Discussion

# Introduction

During the last meeting, RAN4 spilt test-cases for FR1+FR1 RRM performance requirement. In this paper, we share our view regarding test scope and parameters.

# Discussion

- Agree on the TC list and CR slitting as below.

| TC# | The RRM requirement for test case | Detailed Scope | Volunteer Company |
| --- | --- | --- | --- |
| TC1 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC | FFS: Only consider one of parallel processing and sequential processing for test cases of HO with PSCell to reduce the number of test cases. FFS: for TC2, considering the testability issue discussed in maintenance part, this test case may not be tested | Qualcomm |
| TC2 | HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC |  | Huawei |
| TC3 | HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC |  | OPPO |
| TC5 | PSCell addition and release delay of unknown FR1 PSCell |  | Nokia |
| TC6 | PSCell activation and deactivation for FR1+FR1 NR-DC | Define both RACH-based and RACH-less based PSCell activation/deactivation in one TC | Ericsson |
| [TC7] | Conditional PSCell addition and release delay of NR PSCell in FR1 | Conditional PSCell addition and release delay of NR PSCell in FR1(without SSB index measurement) | Vivo |

General test configuration for HO with PSCell test cases.

Before discussing detail of test case configuration, we share our view about general configuration for HO with PSCell test cases. From our understanding, RRM requirement based on sequential processing or parallel processing are already introduced during R17 HO with PSCell. Therefore, RAN4 does not need to introduce for both sequential processing and parallel processing which can be verified during other test cases. In order to reduce test time and number of test cases, parallel processing is considered for FR1+FR1. RRM requirement with sequential processing can be verified during R17 HO with PSCell test cases. Similarly, RAN4 does not need to define both unknown and known target cell scenario for FR1+FR1 NR-DC.

Proposal : To reduce test duration and number of test cases, RAN4 introduce

- HO with PSCell test cases only for parallel processing scenario.

- Target cell status : known target PCell and target PSCell.

Proposal : RAN4 introduce FR2 test case and the test case is not tested until the testability issue is resolved.

# Conclusions

Proposal : To reduce test duration and number of test cases, RAN4 introduce

- HO with PSCell test cases only for parallel processing scenario.

- Target cell status : known target PCell and target PSCell.

Proposal : RAN4 introduce FR2 test case and the test case is not tested until the testability issue is resolved.

# References

[1] R4-2321586, WF on NR eFeRRM (part2), OPPO, RAN #109