3GPP TSG-RAN WG4 Meeting # 110	R4-2400575

# Athens, Greece, February 26 – March 1, 2024

Source:	LG Electronics

Title:	Discussion on UE demodulation performance for NR SL evolution

Agenda item:	8.22.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on UE demodulation performance for NR SL evolution based on the approved WF[1] and according to the work plan [2], we need to allocate the work of CR. So, this contribution provide initial template for CR work split.

Discussion

Sub-topic 1-1 NR sidelink CA scenario

- Agreed scenarios - PSSCH performance requirements - Support single carrier bandwidth: 10MHz, 20MHz, 30MHz, 40MHz (Note: Additional simulation work for PSSCH performance is 10/30/40 since we already have 20MHz requirements in Rel-16.) - PSCCH decoding capability test - PSFCH decoding capability test

The PSSCH performance is depends on the number of sub-channel configuration rather than bandwidth. From the configuration of the 20MHz requirements in Rel-16, the number of sub-channel is 2, and the sub-channel size is 10 RBs. If the sub-channel configuration is same, the performances of other bandwidth are expected to be same. So, we propose to define only one representative case 10+10 MHz or 20+20 MHz. If other bandwith combination requirement is necessary, just the description can be added like “other bandwidth combinations may also apply to the requirements below.”.

Proposal 1: Define only one representative case 10+10 MHz or 20+20 MHz. If other bandwith combination requirement is necessary, just add a description as “other bandwidth combinations may also apply to the requirements below”.

Table 11.1.2.1.1-2: Minimum performance

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-1.1 | 20 / 30 | QPSK, 0.30 | TDLA30-2700 | 10% | 3.4 |
| 2 | R.PSSCH.2-1.2 | 20 / 30 | 16QAM, 0.37 | TDLA30-1400 |  | 8.8 |
| 3 | R.PSSCH.2-1.3 | 20 / 30 | 64QAM, 0.43 | TDLA30-180 |  | 14.8 |

Table 11.1.2A.1.1-1: Minimum performance for CA configuration

| Test number | CA Bandwidth combination (MHz) | Modulation format and code rate | Minimum performance requirement |
| --- | --- | --- | --- |
| 1 | 20+20 | 16QAM, 0.37 | Test 2 as defined in Table11.1.2.1.1-2 per CC |

Proposal 2: Regarding PSCCH decoding capability test, it would be better to define maximum bandwidth combination case. So, 30+40 MHz scenario can be considered. The legacy minimum requirement can be baseline and the minimum performance can be updated as follow:

Table 11.1.8A.1.1-1: Test Parameters for SL CA PSCCH decoding capability test

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Member ID (Note 1) |  |  |  | 0 |  |
| Sidelink UE i, 0 ≤ i ≤ X (Note 5) | Sidelink Transmissions |  |  | PSCCH + PSSCH |  |
|  | Timing offset (Note 2) |  | s | 0 |  |
|  | Frequency offset (Note 3) |  | Hz | 0 |  |
|  | Synchronization source |  |  | GNSS |  |
|  | Propagation Channel |  |  | Static propagation condition without external noise |  |
|  | Antenna configuration |  |  | 1x2 Low |  |
|  | PSSCH RMC (Note 7) |  |  | R.PSSCH.2-1.4 |  |
|  | PSCCH RMC (Note 4) |  |  | R.PSCCH.2-1.1 |  |
|  | Source ID |  |  | 0 |  |
|  | PSFCH periodicity |  | Slots | 1 |  |
|  | MinTimeGapPSFCH |  | Slots | 2 |  |
|  | Channel bandwidth |  | MHz | 30 | 40 |
|  | X |  |  | 6 | 9 |
|  | Pscch-RxSidelink |  |  | value1 | value1 |
|  | PSFCH Resource (Note 6) | RB index |  | 10*i |  |
|  |  | CS pair index |  | 0 |  |
| Note 1: Member ID is an identifier uniquely identifying a member. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. Note 4: OCC index for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission as per in Clause 8.4.1.3.2 of TS 38.211[9]. Note 5: Each UE occupies one sub-channel so that all sub-channels are filled. Note 6: The mapping procedure of PSSCH resource and PSFCH resource is specified in Clause 16.3 of TS 38.213 [11]. Note 7: Follow the RMC configuration except channel bandwidth. |  |  |  |  |  |

Table 11.1.8A.1.1-2: Single carrier performance with different bandwidth for multiple CA configurations

| Test Number | Bandwidth (MHz) / Subcarrier spacing(kHz) | PSCCH Reference channel | Propagation Channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) |
| 1 | 30 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |
| 2 | 40 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |

Table 11.1.8A.1.1-3: Minimum performance for CA configuration

| Test number | CA Bandwidth combination (MHz) | Minimum performance requirement |
| --- | --- | --- |
| 1 | 30+40 | As defined in Table 11.1.8A.1.1-2 per CC |

Proposal 3: Similar to PSCCH decoding capability test, PSFCH decoding capability test also 30+40 MHz scenario can be considered. The legacy minimum requirement can be baseline and the minimum performance can be updated as follow:

Table 11.1.9A.1.1-1: Test Parameters for SL CA PSFCH decoding capability test

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Channel Bandwidth |  | MHz | 30 | 40 |
| HARQ-ACK information |  |  | ACK or NACK |  |
| Source ID of tested UE |  |  | 0 |  |
| Sidelink UE i, 0 ≤ i ≤ N-1(Note 3) | Sidelink transmissions for |  | PSFCH |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |  |
|  | Antenna configuration |  | 1x2 Low |  |
|  | Member ID(Note 4) |  | i |  |
|  | PSFCH resource allocation(Note 5) |  | N UEs transmit PSFCHs one by one on each RB with CS pair index 0. i.e. UE 0 transmits PSFCH on RB 0, UE 1 transmits PSFCH on RB 1,…, UE (N-1) transmits PSFCH on RB N-1 |  |
|  | PSFCH periodicity | Slots | 1 |  |
| Note 1: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 2: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. Note 3: N equals to the number of PSFCH(s) resources that UE can receive in a slot as specified in [TBD] Note 4: Member ID is an identifier uniquely identifying a member Note 5: All PSFCHs in a slot are corresponding to one PSSCH that occupies all sub channels. |  |  |  |  |

Table 11.1.9A.1.1-2: Single carrier performance with different bandwidth for multiple CA configurations

| Test Number | Bandwidth (MHz) / Subcarrier spacing(kHz) | Propagation Channel | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Probability of success detection slot with ACK only | Probability of success detection slot with NACK or DTX |
| 1 | 30 / 30 | Static propagation condition without external noise | 99 | 99 |
| 2 | 40 / 30 | Static propagation condition without external noise | 99 | 99 |
| Note 1: The probability of success detection slot with ACK only is the probability that the corresponding PSSCH is not retransmitted when Option A is selected. Note 2: The probability of success detection slot with NACK or DTX is the probability that the corresponding PSSCH is retransmitted when Option B or option C is selected. |  |  |  |  |

Table 11.1.9A.1.1-3: Minimum performance for CA configuration

| Test number | CA Bandwidth combination (MHz) | Minimum performance requirement |
| --- | --- | --- |
| 1 | 30+40 | As defined in Table 11.1.9A.1.1-2 per CC |

Sub-topic 1-2 NR sidelink unlicensed band scenario

- Issue 1-2-1: Whether to introduce new requirement for PSSCH/PSCCH in SL-U - Issue 1-2-2: Whether to introduce new requirement for PSFCH in SL-U Way Forward: - PSSCH for interlacing and PSFCH for combining tests needed. - FFS: whether to introduce new requirement for PSCCH.

For SL-U scenario, the interlacing impact should be evaluated. But we don’t think both PSSCH and PSCCH need to be evaluated. So, only PSSCH for interlacing and PSFCH for comining tests needed. The requirement for PSCCH is not necessary.

Proposal 4: The requirement for PSCCH is not necessary. Only PSSCH for interlacing and PSFCH for combining tests can be considered in SL-U.

- Issue 1-2-5: Necessary principles to consider for LBT model Way Forward: - Option 1: RAN4 to consider following principle for LBT model: (HW) - Due to the utilization of HARQ-ACK feedback, LBT failure probability shall be set to 1 (pLBT=0) to guarantee that HARQ-ACK feedback is not impacted by the LBT failure. - It's typical to configure gap between two consecutive COTs to give TE more time to perform LBT, one potential way is to set the start position of PSSCH transmission in the first slot of each COT to #7 - CPE extension should be configured for the first AGC symbol of each SL slot within the COT to make the gap between two consecutive slots smaller than 16us - The COT duration should be designed to guarantee that PSFCH is always transmitted in the slot with 14 symbol allocation. - 35 SCI2 information bits are assumed during Rel-16 V2X test. However, to convey the COT information, SCI2 information bits is expected to be more which should be re-discussed. - Option 2: RAN4 to discuss whether LBT should be considered for defining SL-U demodulation performance requirements. And if so, whether the model in sub-clause B.5.1 in the specification can be reused.

- Issue 1-2-6: Starting point for LBT model Way Forward: - Option 1: RAN4 to consider following LBT model as starting point. (HW) - TE performs LBT to initial a COT with LBT failure probability equaling to 0 (pLBT=1) and share this COT with tested UE - The start symbol of first slot in each COT is #7 - The COT duration is randomly selected from {2,4,8} slots - COT information is conveyed in SCI stage 2. - CPE extension is configured for the first AGC symbol of each SL slot within the COT - Tested UE uses the sharing COT to transmit PSFCH by via type 2 channel access - Option 2: LBT should be modeled in SL-U test. If LBT is agreed, NR-U test parameters can be used as reference. (LGE) - Test parameters for LBT have LBT failure probability (PLBT) and if LBT failure occurred SL does not transmit for SL transmission period. - The length of first slot of the SL Tx burst can be from 6 to 12 OFDM symbols except AGC and TxRx switching symbols. Which can be pre-configured by test case scenario. - COT duration can be randomly selected from a set. E.g. {2, 4, 6, 7} slots. - Option 3: RAN4 to discuss whether LBT should be considered for defining SL-U demodulation performance requirements. And if so, whether the model in sub-clause B.5.1 in the specification can be reused. (Nokia)

Regarding the LBT model and principles, generally we agree with HW’s suggestion. And we would like to discuss more for the detail values.

- RAN4 to consider following principle for LBT model:

  - Due to the utilization of HARQ-ACK feedback, LBT failure probability shall be set to 1 (pLBT=0) to guarantee that HARQ-ACK feedback is not impacted by the LBT failure.

  - It's typical to configure gap between two consecutive COTs to give TE more time to perform LBT, one potential way is to set the start position of PSSCH transmission in the first slot of each COT to #7

  - CPE extension should be configured for the first AGC symbol of each SL slot within the COT to make the gap between two consecutive slots smaller than 16us

  - The COT duration should be designed to guarantee that PSFCH is always transmitted in the slot with 14 symbol allocation.

  - 35 SCI2 information bits are assumed during Rel-16 V2X test. However, to convey the COT information, SCI2 information bits is expected to be more which should be re-discussed.

- RAN4 to consider following LBT model as starting point.

  - TE performs LBT to initial a COT with  LBT failure probability equaling to 0 (pLBT=1) and share this COT with tested UE

  - The start symbol of first slot in each COT is #7

  - The COT duration is randomly selected from {2,4,8} slots

  - COT information is conveyed in SCI stage 2.

  - CPE extension is configured for the first AGC symbol of each SL slot within the COT

  - Tested UE uses the sharing COT to transmit PSFCH by via type 2 channel access

- Issue 1-2-7: Test configurations for PSSCH of SL-U Way Forward: - Option 1: RAN4 to consider following test configuration for PSSCH requirements definition: (HW) - Configure 1 PSSCH occasion for each PSSCH - MCS:16QAM, 0.37 - Propagation: TDLA30-1500 - PSFCH resource period: 4 - MinTimeGap: 3 - PSSCH DMRS pattern: 3 symbols for slot without PSFCH transmission and 2 symbols for slot with PSFCH transmission. - Option 2: SL-U demodulation performance can reuse the existing SL test parameters as much as possible. (LGE) - For transmission mode, reuse the existing NR-U transmission model as specified in TS38.101-4 B.5 as much as possible. - Option 3: Existing test parameters and transmission mode from NR-U can be reused. (Nokia)

Proposal 5: We can discuss more detail about test configurations for PSSCH of SL-U. The initial format is as follow:

Table 11.1.2B.1.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| SL transmission model |  |  | As specified in B.X |
| SL model parameters | SL transmission duration values | Slots | {2,4,6,7} or {2,4,8} |
|  | Occupied OFDM symbols in slot other than the first slot of the transmission duration | Symbols | 12 (Except AGC, TxRx switching symbols) |
|  | Occupied OFDM symbols in the first slot of the transmission duration | Symbols | {6,7, …,12} |
|  | SL transmission period or Gap between COTs | ms or slots | 5 ms or 3 slots |
|  | LBT failure probability (pLBT) |  | 0.25 |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern (Note 1) |  | {2,3} |
|  | Number of interlace |  | 1 with RB index 0, 5, 10, …, 50 |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 3) | Hz | +650 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| PSFCH resource period |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.2B.1.1-2: Minimum performance for SL-U

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-2.1 | 20 / 30 | 16QAM, 0.37 | TDLA30-1500 | 10 % | TBD |

- Table A.6.2.2-2: PSSCH Reference Measurement Channel for SL-U

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-2.1 |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 11 |
| CP-OFDM symbols for slot with PSFCH (Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH (Note 2) |  |  | 12 |
| Modulation order |  |  | 16QAM |
| MCS index |  |  | 11 |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 15 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 68 |
|  | α |  | 1 |
|  | βoffset |  | 2 |
| Overhaed for TBS determination |  |  | 0 |
| Transport Block Size for slot with PSFCH |  | Bits | 808 |
| Transport Block Size for slot without PSFCH |  | Bits | 1416 |
| Transport block CRC |  |  | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 2232 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 3792 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. Note 2: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and guard symbols. |  |  |  |

- Issue 1-2-8: Test configurations for PSCCH of SL-U Way Forward: - Option 1: RAN4 to consider the parameters in Table 2-3 and 2-4 for PSCCH requirements definition. (HW) - Table 2-3: Common Parameters Parameter; Unit; Test 1. Active cell(s); None. PSCCH payloads; bit; 26. Sidelink UE 1; Sidelink Transmissions; PSCCH+PSSCH. Timing offset (Note 1); s; CP/2-12*64*Tc. Frequency offset (Note 2); Hz; +600. Synchronization; GNSS or GNSS-equivalent. Antenna configuration; 1x2 Low. PSSCH RMC; TBD. NOTE 1: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. NOTE 2: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. NOTE 3: OCC index i for PSCCH DMRS is randomly selected from {0, 1, 2} for each PSCCH transmission.. - Table 2-4: Test parameters Test number; PSCCH Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Propagation condition; Reference value. Probability of missed PSCCH (%); SNR (dB) of PSCCH. 1; TBD; 20 / 30; TDLA30-1500; 1; TBD.

Proposal 6: We think that the requirement of PSCCH of SL-U is not necessary. So, the PSCCH test configuration also is not necessary.

- Issue 1-2-9; Test configurations for PSFCH of SL-U Way Forward: - Option 1: RAN4 to further discuss how to design the LBT model and test setup for PSFCH performance test. (HW)

Proposal 7: We would like to discuss more detail configuration of PSFCH of SL-U. The initial proposal is as follow:

Table 11.1.5A.1.1.1-1: Test Parameters

| Parameter | unit | Test 1 |
| --- | --- | --- |
| K3 dedicated PRB | RB | 5 |
| The number of PSFCH symbols (Note 1) | symbol | 2 |
| Number of information bits | bit | 1 |
| Synchronization source |  | GNSS |
| Timing offset (Note 2) | s | CP/2-12*64*Tc |
| Frequency offset (Note 3) | Hz | 600 |
| PSFCH resource period | Slots | 1 |
| Antenna configuration |  | 1x2 Low |
| Note 1: First symbol is included. First symbol is used for AGC and not used for demodulation. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |

The ACK missed detection probability shall not exceed 1% at the SNR given in table 11.1.5A.1.1.1-2.

The DTX to ACK probability shall not exceed 1%.

Table 11.1.5A.1.1.1-2: Minimum requirements

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | ACK missed detection probability (%) | SNR (dB) |
| 1 | 20 / 30 | TDLA30-180 | 1 | TBD |

According to the work plan, during RAN4#110 meeting we need to discuss about the CR work split.

Meeting; Works. RAN4#108-bis (Oct.2023); Performance part - Discussion and approvement on work plan - Discussion on work scope and the list of performance test cases. RAN4#109 (Nov.2023); Performance part - Discussion and finalization on work scope and the list of performance test cases - Discussion on initial link simulation assumptions. RAN4#110 (Feb.2024); Performance part - Collection of initial simulation results - Update simulation assumptions for final performance requirements - Discussion and finalization on CR work split. RAN4#110-bis (Apr.2024); Performance part - Collection of final simulation results - Finalization performance requirements based on collected simulation results - Submit draft CRs. RAN4#111 (May.2024); Performance part - Discussion and conclusion on remaining issues - Finalization draft CRs and approvement on Big CR based on the draft CRs - Completion Sidelink demodulation performance requirements.

CR work split for SL demodulation performance.

Table 1: Work split on demodulation performance requirements for R18 Sidelink evoluation

| No. | Requirements for | Detail | New or impacted section in TS38.133 | Volunteer Company |
| --- | --- | --- | --- | --- |
| 1 | PSSCH demodulation requirements for SL-CA | PSSCH demodulation requirements for SL-CA based on single carrier performance. | 11.1.2X |  |
| 2 | PSCCH decoding capability test for SL-CA | PSCCH decoding capability test for SL-CA at maximum bandwidth combination. | 11.1.8X |  |
| 3 | PSFCH decoding capability test for SL-CA | PSFCH decoding capability test for SL-CA at maximum bandwidth combination. | 11.1.9X |  |
| 4 | PSSCH demodulation requirements for SL-U | PSSCH demodulation requirements for SL-U. The discussed LBT model can be defined in B.5 Also, the reference measurement channel for SL-U PSSCH should be defined in A.6.2 | 11.1.2X, A.6.2, B.5 |  |
| 5 | PSFCH demodulation requirements for SL-U | PSFCH demodulation requirements for SL-U. | 11.1.5X |  |

Conclusion

In this contribution, we provide our views on work scope and test cases for demodulation performance of SL evolution based on the WF [1].

Proposal 1: Define only one representative case 10+10 MHz or 20+20 MHz. If other bandwith combination requirement is necessary, just the description can be added like “other bandwidth combinations may also apply to the requirements below.”.

Table 11.1.2.1.1-2: Minimum performance

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-1.1 | 20 / 30 | QPSK, 0.30 | TDLA30-2700 | 10% | 3.4 |
| 2 | R.PSSCH.2-1.2 | 20 / 30 | 16QAM, 0.37 | TDLA30-1400 |  | 8.8 |
| 3 | R.PSSCH.2-1.3 | 20 / 30 | 64QAM, 0.43 | TDLA30-180 |  | 14.8 |

Table 11.1.2A.1.1-1: Minimum performance for CA configuration

| Test number | CA Bandwidth combination (MHz) | Modulation format and code rate | Minimum performance requirement |
| --- | --- | --- | --- |
| 1 | 20+20 | 16QAM, 0.37 | Test 2 as defined in Table11.1.2.1.1-2 per CC |

Proposal 2: Regarding PSCCH decoding capability test, it would be better to define maximum bandwidth combination case. So, 30+40 MHz scenario can be considered. The legacy minimum requirement can be baseline and the minimum performance can be updated as follow:

Table 11.1.8A.1.1-1: Test Parameters for SL CA PSCCH decoding capability test

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Member ID (Note 1) |  |  |  | 0 |  |
| Sidelink UE i, 0 ≤ i ≤ X (Note 5) | Sidelink Transmissions |  |  | PSCCH + PSSCH |  |
|  | Timing offset (Note 2) |  | s | 0 |  |
|  | Frequency offset (Note 3) |  | Hz | 0 |  |
|  | Synchronization source |  |  | GNSS |  |
|  | Propagation Channel |  |  | Static propagation condition without external noise |  |
|  | Antenna configuration |  |  | 1x2 Low |  |
|  | PSSCH RMC (Note 7) |  |  | R.PSSCH.2-1.4 |  |
|  | PSCCH RMC (Note 4) |  |  | R.PSCCH.2-1.1 |  |
|  | Source ID |  |  | 0 |  |
|  | PSFCH periodicity |  | Slots | 1 |  |
|  | MinTimeGapPSFCH |  | Slots | 2 |  |
|  | Channel bandwidth |  | MHz | 30 | 40 |
|  | X |  |  | 6 | 9 |
|  | Pscch-RxSidelink |  |  | value1 | value1 |
|  | PSFCH Resource (Note 6) | RB index |  | 10*i |  |
|  |  | CS pair index |  | 0 |  |
| Note 1: Member ID is an identifier uniquely identifying a member. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. Note 4: OCC index for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission as per in Clause 8.4.1.3.2 of TS 38.211[9]. Note 5: Each UE occupies one sub-channel so that all sub-channels are filled. Note 6: The mapping procedure of PSSCH resource and PSFCH resource is specified in Clause 16.3 of TS 38.213 [11]. Note 7: Follow the RMC configuration except channel bandwidth. |  |  |  |  |  |

Table 11.1.8A.1.1-2: Single carrier performance with different bandwidth for multiple CA configurations

| Test Number | Bandwidth (MHz) / Subcarrier spacing(kHz) | PSCCH Reference channel | Propagation Channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) |
| 1 | 30 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |
| 2 | 40 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |

Table 11.1.8A.1.1-3: Minimum performance for CA configuration

| Test number | CA Bandwidth combination (MHz) | Minimum performance requirement |
| --- | --- | --- |
| 1 | 30+40 | As defined in Table 11.1.8A.1.1-2 per CC |

Proposal 3: Similar to PSCCH decoding capability test, PSFCH decoding capability test also 30+40 MHz scenario can be considered. The legacy minimum requirement can be baseline and the minimum performance can be updated as follow:

Table 11.1.9A.1.1-1: Test Parameters for SL CA PSFCH decoding capability test

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Channel Bandwidth |  | MHz | 30 | 40 |
| HARQ-ACK information |  |  | ACK or NACK |  |
| Source ID of tested UE |  |  | 0 |  |
| Sidelink UE i, 0 ≤ i ≤ N-1(Note 3) | Sidelink transmissions for |  | PSFCH |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |  |
|  | Antenna configuration |  | 1x2 Low |  |
|  | Member ID(Note 4) |  | i |  |
|  | PSFCH resource allocation(Note 5) |  | N UEs transmit PSFCHs one by one on each RB with CS pair index 0. i.e. UE 0 transmits PSFCH on RB 0, UE 1 transmits PSFCH on RB 1,…, UE (N-1) transmits PSFCH on RB N-1 |  |
|  | PSFCH periodicity | Slots | 1 |  |
| Note 1: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 2: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. Note 3: N equals to the number of PSFCH(s) resources that UE can receive in a slot as specified in Clause 4.2.16.1.6 of TS 38.306[14]( IE psfch-RxNumber)) . Note 4: Member ID is an identifier uniquely identifying a member Note 5: All PSFCHs in a slot are corresponding to one PSSCH that occupies all sub channels. |  |  |  |  |

Table 11.1.9A.1.1-2: Single carrier performance with different bandwidth for multiple CA configurations

| Test Number | Bandwidth (MHz) / Subcarrier spacing(kHz) | Propagation Channel | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Probability of success detection slot with ACK only | Probability of success detection slot with NACK or DTX |
| 1 | 30 / 30 | Static propagation condition without external noise | 99 | 99 |
| 2 | 40 / 30 | Static propagation condition without external noise | 99 | 99 |
| Note 1: The probability of success detection slot with ACK only is the probability that the corresponding PSSCH is not retransmitted when Option A is selected. Note 2: The probability of success detection slot with NACK or DTX is the probability that the corresponding PSSCH is retransmitted when Option B or option C is selected. |  |  |  |  |

Table 11.1.9A.1.1-3: Minimum performance for CA configuration

| Test number | CA Bandwidth combination (MHz) | Minimum performance requirement |
| --- | --- | --- |
| 1 | 30+40 | As defined in Table 11.1.9A.1.1-2 per CC |

Proposal 4: The requirement for PSCCH is not necessary. Only PSSCH for interlacing and PSFCH for combining tests can be considered in SL-U.

Proposal 5: We can discuss more detail about test configurations for PSSCH of SL-U. The initial format is as follow:

Table 11.1.2B.1.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| SL transmission model |  |  | As specified in B.X |
| SL model parameters | SL transmission duration values | Slots | {2,4,6,7} or {2,4,8} |
|  | Occupied OFDM symbols in slot other than the first slot of the transmission duration | Symbols | 12 (Except AGC, TxRx switching symbols) |
|  | Occupied OFDM symbols in the first slot of the transmission duration | Symbols | {6,7, …,12} |
|  | SL transmission period or Gap between COTs | ms or slots | 5 or 3 slots |
|  | LBT failure probability (pLBT) |  | 0.25 |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern (Note 1) |  | {2,3} |
|  | Number of interlace |  | 1 with RB index 0, 5, 10, …, 50 |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 3) | Hz | +650 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| PSFCH resource period |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.2B.1.1-2: Minimum performance for SL-U

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-2.1 | 20 / 30 | 16QAM, 0.37 | TDLA30-1500 | 10 % | TBD |

- Table A.6.2.2-2: PSSCH Reference Channel for SL-U

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-2.1 |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 11 |
| CP-OFDM symbols for slot with PSFCH (Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH (Note 2) |  |  | 12 |
| Modulation order |  |  | 16QAM |
| MCS index |  |  | 11 |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 15 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 68 |
|  | α |  | 1 |
|  | βoffset |  | 2 |
| Overhaed for TBS determination |  |  | 0 |
| Transport Block Size for slot with PSFCH |  | Bits | 808 |
| Transport Block Size for slot without PSFCH |  | Bits | 1416 |
| Transport block CRC |  |  | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 2232 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 3792 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. Note 2: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and guard symbols. |  |  |  |

Proposal 6: We think that the requirement of PSCCH of SL-U is not necessary. So, the PSCCH test configuration also is not necessary.

Proposal 7: We would like to discuss more detail configuration of PSFCH of SL-U. The initial proposal is as follow:

Table 11.1.5A.1.1.1-1: Test Parameters

| Parameter | unit | Test 1 |
| --- | --- | --- |
| K3 dedicated PRB | RB | 5 |
| The number of PSFCH symbols (Note 1) | symbol | 2 |
| Number of information bits | bit | 1 |
| Synchronization source |  | GNSS |
| Timing offset (Note 2) | s | CP/2-12*64*Tc |
| Frequency offset (Note 3) | Hz | 600 |
| PSFCH resource period | Slots | 1 |
| Antenna configuration |  | 1x2 Low |
| Note 1: First symbol is included. First symbol is used for AGC and not used for demodulation. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |

The ACK missed detection probability shall not exceed 1% at the SNR given in table 11.1.5A.1.1.1-2.

The DTX to ACK probability shall not exceed 1%.

Table 11.1.5A.1.1.1-2: Minimum requirements

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | ACK missed detection probability (%) | SNR (dB) |
| 1 | 20 / 30 | TDLA30-180 | 1 | TBD |

CR work split for SL demodulation performance.

Table 1: Work split on demodulation performance requirements for R18 Sidelink evoluation

| No. | Requirements for | Detail | New or impacted section in TS38.133 | Volunteer Company |
| --- | --- | --- | --- | --- |
| 1 | PSSCH demodulation requirements for SL-CA | PSSCH demodulation requirements for SL-CA based on single carrier performance. | 11.1.2X |  |
| 2 | PSCCH decoding capability test for SL-CA | PSCCH decoding capability test for SL-CA at maximum bandwidth combination. | 11.1.8X |  |
| 3 | PSFCH decoding capability test for SL-CA | PSFCH decoding capability test for SL-CA at maximum bandwidth combination. | 11.1.9X |  |
| 4 | PSSCH demodulation requirements for SL-U | PSSCH demodulation requirements for SL-U. The discussed LBT model can be defined in B.5 Also, the reference measurement channel for SL-U PSSCH should be defined in A.6.2 | 11.1.2X, A.6.2, B.5 |  |
| 5 | PSFCH demodulation requirements for SL-U | PSFCH demodulation requirements for SL-U. | 11.1.5X |  |

Reference

[1] R4-2321131, “WF on UE demodulation performance for NR SL evolution,” LG Electronics.

[2] R4-2315146, “Work plan for Demodulation Performance of Rel-18 NR Sidelink Evolution,” LG Electronics, OPPO.

[3] 3GPP TS38.101-4 v18.2.0, “UE radio transmission and reception; Part4: Performance requirements (Release 18),” December, 2023.