# 3GPP TSG-RAN WG4 Meeting # 110				R4-2401107

# Athens, GR, 26 Feb. – 01 Mar., 2024

Agenda item:	10

Source: 	NTT DOCOMO, INC.

Title: 	Rel-18 RAN4 UE feature list for NR_MC_enh

Document for:	Approval

- Introduction

This section provides the RAN4 UE feature list for Rel-18 NR_MC_enh. The components in the table are described considering RAN2 TS 38.306. The same contribution with additional explanations are submitted in AI 8.15.1 for Multi-carrier enhancement according to Chairman’s guidance [1].

[1] R4-2401106, Views on RAN2 LS and UE feature list for Rel-18 Tx switching, NTT DOCOMO, INC.

- Proposal

Proposal: RAN4#110 discusses the following table as a baseline for the UE feature list for Rel-18 multi-carrier enhancements WI.

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | Switching period for dynamic UL Tx switching across 3 bands in case of inter-band CA, SUL for single TAG | 1. Indicate support of dynamic UL Tx switching across 3 bands for inter-band UL CA, or SUL for single TAG. 2. Indicate the supported switching period for dynamic UL Tx switching across 3 bands for inter-band UL CA, or SUL for single TAG. |  | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 2 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-2 | Switching period for dynamic UL Tx switching across 4 bands in case of inter-band CA, SUL for single TAG | 1. Indicate support of dynamic UL Tx switching across 4 bands for inter-band UL CA, or SUL for single TAG. 2. Indicate the supported switching period for dynamic UL Tx switching across 4 bands for inter-band UL CA, or SUL for single TAG. |  | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 2 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | [38-3] | Switching period for dynamic UL Tx switching across 2 bands in case of inter-band CA, SUL for dual TAG | 1. Indicate support of dynamic UL Tx switching across 2 bands for inter-band UL CA, or SUL for dual TAG. 2. Indicate the supported switching period for dynamic UL Tx switching across 2 bands for inter-band UL CA, or SUL for dual TAG. |  | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 2 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-4 | Switching period for dynamic UL Tx switching across 3 bands in case of inter-band CA, SUL for dual TAG | 1. Indicate support of dynamic UL Tx switching across 3 bands for inter-band UL CA, or SUL for dual TAG. 2. Indicate the supported switching period for dynamic UL Tx switching across 3 bands for inter-band UL CA, or SUL for dual TAG. |  | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 2 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-5 | Switching period for dynamic UL Tx switching across 4 bands in case of inter-band CA, SUL for dual TAG | 1. Indicate support of dynamic UL Tx switching across 4 bands for inter-band UL CA, or SUL for dual TAG. 2. Indicate the supported switching period for dynamic UL Tx switching across 4 bands for inter-band UL CA, or SUL for dual TAG. |  | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 2 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-6 | Application of DL interruptions due to dynamic UL Tx switching | 1. Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern SUL+TDD | 38-1, 38-2, 38-3, 38-4, 35-5 | Yes | N/A | UL Tx switching where DL interruption is needed cannot be supported. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. | Optional with capability signaling |
| 38. NR_MC_enh | 38-7 | Switching Period for unaffected Band for Dual UL | 1. Indicate for a given band pair {band X and band Y}, whether/how the switching period is to be applied on band X, Y, Z, when a UL Tx switching is triggered from band pair {band X and band Z} to band pair {band Y and band Z}, as defined in 38.101-1. If absent for band Z, the UE is not required to transmit on any UL bands during the switching period reported for the band pair of band X and band Y, as defined in 38.101-1 2. Indicate the support of uplink transmission on band Z and is not required to transmit on band X and Y during the switching period reported for the band pair of band X and band Y, as specified in 38.101-1. 3. Indicate additionally the supported switching period for unaffected band for dual UL. | 38-1, 38-4 | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 3 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-8 | Additional switching Period for Dual UL | 1. Indicate additionally the supported Tx switching period for switching between a band pair and another band pair or another band, when Rel-18 UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18. If the capability is not reported, the switching period reported in switchingPeriodFor2T-r18 or switchingPeriodFor1T-r18 applies, as specified in TS 38.214 and TS 38.101-1. | 38-1, 38-2, 38-4, 35-5 | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 1 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-9 | Improved switching period for four-band switching case | 1. Support the capability that the switching period can be improved to min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} assuming UE’s preferred (switched-from, switched-to) band pairs for parallel UL transmission switching for a band combination consisting of four different bands. | 38-1, 35-5 | Yes | N/A | Network can only assume the maximum switch period | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Detailed information can refer to the LS to RAN2 in R4-2317609 | Optional with capability signalling |
| 38. NR_MC_enh | 38-10 | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among 3 or 4 bands | 1. Apply UL-MIMO coherence for the 2Tx-capable UL band(s). Rel-17 signalling on UL-MIMO coherence capability for 2Tx-2Tx switching is reused | 38-1, 38-2, 38-3, 38-4, 35-5 | Yes | N/A | UL-MIMO coherence cannot be supported for UL Tx switching across more than 2 bands. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Detailed information can refer to the LS to RAN2 in R4-2217741. | Optional with capability signalling |