3GPP TSG-RAN WG4 # 110	R4-2401163

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.13.1.2

Source:	MediaTek Inc.

Title:	Simulation results of Advanced receiver to cancel inter-user interference for MU-MIMO

Document for:	Discussion

# Introduction

In RAN#101, the status report of Rel-18 WI of NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined [1].  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. This work is split into two phases where the first phase studies the performance gain, reference receiver assumption, interference modelling, testability, required signalling overhead, as well as impact on other WGs. The initial receiver candidates are E-MMSE-IRC and R-ML. In the second phase it is expected to specify PDSCH demodulation requirements under MU-MIMO scenario with advanced receiver.

# Simulation parameters and results

In this chapter we are going to discuss simulation parameters in Chapter 2.1, simulation results in Chapter 2.2 and simulation analysis in Chapter 2.3.

# 2.1 Simulation parameters

In this chapter we are going to show updated simulation results with short analysis.

We have provided simulation results of agreed FDD and TDD scenarios with 2Rx and 4Rx. Also, we are assuming the same scrambling ID for DMRS sequence for all UEs. We have simulated only single co-scheduled UE scenarios for this meeting. Also, we have assumed full bandwidth allocation of all UEs and the same QCL assumption in all DMRS ports. The evaluation metric is SNR at 70% of maximum throughput.

We have assumed co-scheduled UE modulation order information as known and detected by UE.

In this document we compare performance of 3 receiver types as agreed in WF [2], namely

- R-ML with genie modulation order information (Rel-18 advanced receiver)

- R-ML with modulation order blind detection (Rel-18 advanced receiver)

- MMSE-IRC (Rel-17 reference receiver)

# 2.2 Simulation results

In previous meeting we agreed extensive simulation plan for this meeting. All agreed tests are run, and simulation results are shown in Table 2-1. We provide 3 simulation results for all test cases. Genie MO R-ML is using know modulation order information, provided with DCI signalling, for R-ML receiver. MOBD R-ML is using blind detection of modulation order information for R-ML receiver. MMSE-IRC is following agreed Rel-17 reference receiver implementation. Test cases marked with star and red color are optional cases for interested companies.

| Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Genie MO R-ML | MOBD R-ML | MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/21 | FDD 15kHz SCS | 10MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 14.4 | 16.8 | 24.1 |
| 2/22 |  |  |  |  |  |  | ULA Low |  |  | 11.4 | 12.5 | 16.1 |
| 3/23 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.0 | 6.5 | 7.5 |
| 4/24 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 13.0 | 16.0 | 27.9 |
| 25 |  |  |  | 2T2R |  | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 22.6 | 24.2 | N/A |
| 26 |  |  |  |  |  |  | ULA Low |  |  | 17.1 | 17.6 | 31.0 |
| 27 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.3 | 10.4 | 10.9 |
| 28 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 22.1 | 24.2 | 38.7 |
| 5*/29* |  |  |  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | 13.6 | 15.5 | 20.7 |
| 6*/30* |  |  |  | 2T4R |  |  |  |  |  | 12.2 | 15.1 | 22.9 |
| 31* |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM | 20.7 | 22.0 | 29.2 |
| 32* |  |  |  | 2T4R |  |  |  |  |  | 20.0 | 21.7 | 32.4 |
| 7/33 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK | 10.9 | 11.8 | 12.8 |
| 8/34 |  |  |  |  |  |  | XP medium |  |  | 12.1 | 13.3 | 14.5 |
| 9/35 |  |  |  |  |  |  | ULA Low | MCS 17 | 16QAM | 16.9 | 17.1 | 17.2 |
| 10/36 |  |  |  |  |  |  | XP medium |  |  | 18.5 | 18.9 | 19.2 |
| 11/37 | TDD 30kHz SCS | 40MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 15.3 | 17.4 | 26.9 |
| 12/38 |  |  |  |  |  |  | ULA Low |  |  | 11.8 | 12.6 | 17.2 |
| 13/39 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.3 | 6.7 | 7.7 |
| 14/40 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 14.1 | 16.9 | 38.1 |
| 41 |  |  |  | 2T2R |  | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 23.1 | 24.6 | N/A |
| 42 |  |  |  |  |  |  | ULA Low |  |  | 17.3 | 17.8 | 33.3 |
| 43 |  |  |  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.6 | 10.8 | 11.3 |
| 44 |  |  |  |  |  | TDLC300-100 | ULA medium |  |  | 23.2 | 25.3 | N/A |
| 15*/45* |  |  |  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | 14.3 | 15.8 | 23.0 |
| 16*/46* |  |  |  | 2T4R |  |  |  |  |  | 13.1 | 15.2 | 32.9 |
| 47* |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM | 21.1 | 22.3 | 30.7 |
| 48* |  |  |  | 2T4R |  |  |  |  |  | 20.8 | 22.6 | 39.9 |
| 17/49 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK | 11.2 | 12.0 | 13.1 |
| 18/50 |  |  |  |  |  |  | XP medium |  |  | 12.5 | 13.5 | 14.9 |
| 19/51 |  |  |  |  |  |  | ULA Low | MCS 17 | 16QAM | 17.2 | 17.4 | 17.8 |
| 20/52 |  |  |  |  |  |  | XP medium |  |  | 19.0 | 19.3 | 19.8 |

Table 2-1: Simulation results of MU-MIMO scenarios with different receiver assumptions.

# 2.3 Simulation analysis

We can do several observations from these simulations. These are repeated also in discussion document.

Observations of Rank 1+1 2T2R tests

Observation #1: On Rank 1+1 tests with 2T2R MCS17 for target UE leads to too high SNR requirements in our view.

Observation #2: On Rank 1+1 tests with 2T2R and MCS13 for target UE our simulations show the following channels as feasible tests: TDLC300-100 ULA medium with both precoder options, and TDLC300-100 ULA low.

Observations of Rank 1+1 2T4R tests

Observation #3: On Rank 1+1 tests with 2T4R TDLA30-10 ULA low does not give enough gain over MMSE-IRC.

Observation #4: On Rank 1+1 tests with 2T4R MCS17 for target UE leads to too high SNR requirements in our view.

Observation #5: On Rank 1+1 tests with 2T4R and MCS13 for target UE our simulations show the following channels as feasible tests: TDLC300-100 ULA medium with both precoder options.

Observations of Rank 2+2 4T4R tests

Observation #6: On Rank 2+2 tests with 4T4R and MCS17 for target UE does not give enough gain over MMSE-IRC.

# Conclusion

In this paper we provided simulation results with analysis and observations on the advanced receiver to cancel inter-user interference for MU-MIMO. The following observations are made:

Observations of Rank 1+1 2T2R tests

Observation #1: On Rank 1+1 tests with 2T2R MCS17 for target UE leads to too high SNR requirements in our view.

Observation #2: On Rank 1+1 tests with 2T2R and MCS13 for target UE our simulations show the following channels as feasible tests: TDLC300-100 ULA medium with both precoder options, and TDLC300-100 ULA low.

Observations of Rank 1+1 2T4R tests

Observation #3: On Rank 1+1 tests with 2T4R TDLA30-10 ULA low does not give enough gain over MMSE-IRC.

Observation #4: On Rank 1+1 tests with 2T4R MCS17 for target UE leads to too high SNR requirements in our view.

Observation #5: On Rank 1+1 tests with 2T4R and MCS13 for target UE our simulations show the following channels as feasible tests: TDLC300-100 ULA medium with both precoder options.

Observations of Rank 2+2 4T4R tests

Observation #6: On Rank 2+2 tests with 4T4R and MCS17 for target UE does not give enough gain over MMSE-IRC.

# Reference

- RP-233584, “Status report for NR demodulation requirement evolution”, China Telecom

- R4-2321114, “WF on advanced receiver for MU-MIMO scenario”, China Telecom