3GPP TSG RAN WG4 Meeting # 110 	R4-2401167

# Athens, Greece, February 26 – March 1, 2024

# Source:	LG Electronics, OPPO, [HUAWEI, VOLKSWAGEN,…]

# Title:	New WID on further NR sidelink evolution

# Document for:	Information

Agenda Item:	15

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:      New WID on further NR sidelink evolution

Acronym:	[NR_SL_enh3]

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:  Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| NR_SL_enh2 | 940097 | NR sidelink evolution | Rel-18 RAN work item on NR sidelink evolution |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

3	Justification

In Rel-18, RAN4 specified the following NR sidelink requirements for single carrier and CA operation.

- NR sidelink single carrier in unlicensed spectrum

  - Power class 5

  - 5 NS values

  - 1 Tx

- NR sidelink inter-band con-current operation

  - Example band combination such as n78 with power class 3 + n46 with power class 5

- NR sidelink CA in n47

  - Power class 3

  - Intra-band contiguous CA

For NR sidelink in unlicensed spectrum (SL-U), only power class 5 for 1Tx with 5 NS values was introduced in Rel-18 due to limited time. For NR Uu in unlicensed spectrum (NR-U), power class 5 was introduced in Rel-17 and support for power class 3 was added in Rel-18 to enhance the uplink coverage. There is similar strong need for coverage improvements in NR sidelink and the Rel-18 work for NR-U power class 3 serves as solid basis for introduction of  power class 3 support also for SL-U. When it comes to NS values, total of 17 NS values are supported for NR-U while, only 5 out of 17 NS values for NR-U were defined in Rel-18 SL-U. In order to achieve the global footprint, the remaining 12 NS values should also be specified. For the number of Tx antenna for NR SL-U, 2 Tx should be supported for sidelink MIMO and Tx diversity.

For NR sidelink CA, automotive industry shows strong interest towards power class 2 contiguous and non-contiguous intra band CA to account for the fragmented utilization of different channels of 10MHz and 20MHz in n47 and to achieve sufficient communication range. However, as can be observed above, the power class 2 was not covered in the Rel-18 NR sidelink CA due to the time constraints even though the power class 2 is supported for single carrier operation in n47. Considering the request from the automotive industry, completeness of specification and other activities on the high power UE in 3GPP, intra-band non-contiguous CA along with power class 2 support should be specified in Rel-19.

For inter-band con-current operation, only example band combination was specified for NR Uu with power class 3 + SL-U with power class 5 in Rel-18. If additional band combinations are requested with same power class configuration, the related requirement could be added to the corresponding basket WI. However, if different power class configurations are requested in either of the band, the corresponding general RF requirement with the example band combination needs to be first specified in the core work item.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

- Specify the following requirements for NR sidelink CA in n47 [RAN4]

- Intra-band non-contiguous CA including 10MHz + 10MHz and 10MHz  + 20MHz with power class 2 and power class 3

- Intra-band contiguous CA with power class 2

- Specify the following requirements for single carrier NR sidelink in unlicensed spectrum [RAN4]

- 1 Tx with power class 3

- Tx diversity and MIMO with power class 5 and power class 3

- [Remaining NS values for power class 5]

- Specify the following requirements for inter-band con-current operation based on the request [RAN4]

- NR Uu band with power class 3 + SL-U band with Power class 3

- Specify the corresponding RRM core requirements for the above features [RAN4]

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the following requirements [RAN4]

- UE RRM performance requirements for the specified features listed in the objectives of core part if needed

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | RF Core Requirement | RAN#109 | Core Part |
| 38.133 | RRM Core Requirement | RAN#109 | Core Part |
| 38.133 | RRM Performance Requirement | RAN#111 | Performance Part |
| 38.786 | Further RF Core Requirement | RAN#109 | Core Part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

7	Work item leadership

RAN WG4

8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

LG Electronics

[BOSCH]

[Continental]

[Huawei]

[KT]

[LG Uplus]

META

Oppo

[Samsung]

[SKT]

[Volkswagen]