3GPP TSG-RAN WG4 Meeting #110	R4-2401228

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.14.3.4

Source: 	Qualcomm Incorporated

Title: 	Simulation results for PRS BW aggregation

Document for:	Information

# Introduction

RAN4 agreed on simulation assumptions for PRS BW aggregation [1]. In addition to the agreed simulation assumptions, in the results submitted herein we assumed a gap between adjacent PFLs as proposed in [2].

This paper includes simulation results for the following measurements:

- UE Rx-Tx time difference

- RSTD

# Simulation results for UE Rx-Tx time difference

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | 15 | 15 |
|  | 132 | 30 | 32 | 2 | 4 | 7 | 8 |
|  | 272 |  | 16 | 2 | 4 | 4 | 4 |
|  | 64 | 60 | 32 | 2 | 4 | 7 | 8 |
|  | 132 |  | 16 | 2 | 4 | 4 | 4 |
| FR2 | 64 | 60 | 32 | 2 | 4 | 7 | 8 |
|  | 132 |  | 16 | 2 | 4 | 4 | 4 |
|  | 64 | 120 | 16 | 2 | 4 | 4 | 4 |
|  | 128 |  | 8 | 2 | 4 | 2 | 2 |

Table 1: UE Rx-Tx simulation results for 2 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | 15 | 15 |
|  | 132 | 30 | 32 | 4 | 4 | 8 | 7 |
|  | 272 |  | 16 | 4 | 4 | 4 | 4 |
|  | 64 | 60 | 32 | 4 | 4 | 8 | 8 |
|  | 132 |  | 16 | 4 | 4 | 4 | 4 |
| FR2 | 64 | 60 | 32 | 4 | 4 | 8 | 8 |
|  | 132 |  | 16 | 4 | 4 | 4 | 4 |
|  | 64 | 120 | 16 | 4 | 4 | 4 | 4 |
|  | 128 |  | 8 | 4 | 4 | 2 | 2 |

Table 2: UE Rx-Tx simulation results for 2 PFLs in AWGN with 4 samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | 13 | 15 |
|  | 132 | 30 | 32 | 2 | 4 | 4 | 4 |
|  | 272 |  | 16 | 2 | 4 | 2 | 3 |
|  | 64 | 60 | 32 | 2 | 4 | 3 | 4 |
|  | 132 |  | 16 | 2 | 4 | 2 | 3 |
| FR2 | 64 | 60 | 32 | 2 | 4 | 3 | 4 |
|  | 132 |  | 16 | 2 | 4 | 2 | 3 |
|  | 64 | 120 | 16 | 2 | 4 | 2 | 2 |
|  | 128 |  | 8 | 2 | 4 | 1 | 1 |

Table 3: UE Rx-Tx simulation results for 3 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | 15 | 15 |
|  | 132 | 30 | 32 | 4 | 4 | 4 | 4 |
|  | 272 |  | 16 | 4 | 4 | 3 | 3 |
|  | 64 | 60 | 32 | 4 | 4 | 4 | 4 |
|  | 132 |  | 16 | 4 | 4 | 2 | 2 |
| FR2 | 64 | 60 | 32 | 4 | 4 | 4 | 4 |
|  | 132 |  | 16 | 4 | 4 | 2 | 2 |
|  | 64 | 120 | 16 | 4 | 4 | 2 | 2 |
|  | 128 |  | 8 | 4 | 4 | 2 | 1 |

Table 4: UE Rx-Tx simulation results for 3 PFLs in AWGN with 4 samples.

# Simulation results for RSTD

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | ±18 |
|  | 132 | 30 | 32 | 2 | 4 | ±9 |
|  | 272 |  | 16 | 2 | 4 | ±5 |
|  | 64 | 60 | 32 | 2 | 4 | ±10 |
|  | 132 |  | 16 | 2 | 4 | ±5 |
| FR2 | 64 | 60 | 32 | 2 | 4 | ±10 |
|  | 132 |  | 16 | 2 | 4 | ±5 |
|  | 64 | 120 | 16 | 2 | 4 | ±5 |
|  | 128 |  | 8 | 2 | 4 | ±3 |

Table 5: RSTD simulation results for 2 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | ±18 |
|  | 132 | 30 | 32 | 4 | 4 | ±10 |
|  | 272 |  | 16 | 4 | 4 | ±6 |
|  | 64 | 60 | 32 | 4 | 4 | ±9 |
|  | 132 |  | 16 | 4 | 4 | ±5 |
| FR2 | 64 | 60 | 32 | 4 | 4 | ±9 |
|  | 132 |  | 16 | 4 | 4 | ±5 |
|  | 64 | 120 | 16 | 4 | 4 | ±5 |
|  | 128 |  | 8 | 4 | 4 | ±3 |

Table 6: RSTD simulation results for 2 PFLs in AWGN with 4 samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | ±16 |
|  | 132 | 30 | 32 | 2 | 4 | ±4 |
|  | 272 |  | 16 | 2 | 4 | ±3 |
|  | 64 | 60 | 32 | 2 | 4 | ±5 |
|  | 132 |  | 16 | 2 | 4 | ±3 |
| FR2 | 64 | 60 | 32 | 2 | 4 | ±5 |
|  | 132 |  | 16 | 2 | 4 | ±3 |
|  | 64 | 120 | 16 | 2 | 4 | ±2 |
|  | 128 |  | 8 | 2 | 4 | ±1 |

Table 7: RSTD simulation results for 3 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | ±13 |
|  | 132 | 30 | 32 | 4 | 4 | ±5 |
|  | 272 |  | 16 | 4 | 4 | ±3 |
|  | 64 | 60 | 32 | 4 | 4 | ±5 |
|  | 132 |  | 16 | 4 | 4 | ±2 |
| FR2 | 64 | 60 | 32 | 4 | 4 | ±5 |
|  | 132 |  | 16 | 4 | 4 | ±2 |
|  | 64 | 120 | 16 | 4 | 4 | ±3 |
|  | 128 |  | 8 | 4 | 4 | ±1 |

Table 8: RSTD simulation results for 3 PFLs in AWGN with 4 samples.

# References

[1] R4-2321461, Simulation assumptions for PRS measurement accuracy for PRS/SRS BW aggregation, RAN4#109

[2] R4-2401227, On RRM core maintenance for PRS/SRS BW aggregation, Qualcomm Inc., RAN4#110