# 3GPP TSG-RAN WG4 Meeting # 110                                                             R4-2401273

# Athens, Greece, Feb. 26th – Mar. 1st , 2024

# Source:	ZTE Corporation

# Title:	TP for TR38.718-02-01_CA_n40A-n79C

# Agenda Item:	7.10.2

# Document for:	Approval

- Introduction

The CA_n40A-n79C_BCS4 and 5 were requested and included in the WID, this contribution provides a text proposal to introduce CA_n40A-n79C in TR38.718-02-01.

The fallback have already been defined in the spec.

In the current spec, UL CA_n79C was included via draft CR in previous meeting. However, the technical analysis to check the MSD issues are missing considering the n40 and n79 support simultaneous Rx/Tx operation. Thus in the TP, we also provide the co-existence study on whether UL CA_n79C would falls into band n40 DL Rx.

- Reference

- [1] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

- Text Proposal

----- Start of TP -----

  - 5.x		CA_n40-n79

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n40-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n40-n79 | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4000 MHz | – | 5000 MHz | TDD |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n40-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n79C | CA_n79C CA_n40A-n79C | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

      - 5.x.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n40 or single band n79, it was already analyzed in the fallback configurations of DL CA_n40A-n79A with UL single carrier, and it can be applied for DL CA_n40A-n79C with UL single carrier.

      - 5.x.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n79C should be analyzed for CA_n40A-n79C, which can be found in table Table 5.x.1.3.1-1:

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 40 | 40 | 200 | 4400 | 5000 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 4400 | 4480 | 4600 | 5000 | 4920 | 4800 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 80 | 200 | 8880 | 9920 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 4200 | 5200 | 13280 | 14920 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 160 | 400 | 8600 | 10200 | 17680 | 19920 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 4000 | 5400 | 13000 | 15200 | 22080 | 24920 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 240 | 600 | 8400 | 10400 | 17400 | 20200 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 3800 | 5600 | 12800 | 15400 | 21800 | 25200 |

It can be seen that there are no co-existence issues caused by CA_n79C impact band n40 DL.

      - 5.x.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n40-n79 are already defined in the spec TS38.101-1.

      - 5.x.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined. Thus there are no changes for the current specification.

      - 5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.x.2		Specific for 2 bands UL CA

      - 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n40A-n79C | 23 | +2/-3 |

      - 5.x.2.2		UE co-existence studies

Due to both band n40 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n40A-n79C since the Tx and Rx for either n40 or n79 will not be operation at the same time.

      - 5.x.2.3		REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

----- End of TP -----