3GPP TSG-RAN4 Meeting #110	R4-2401455

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4142 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Perf) CR to 38.133 Rel-17 Cat-F for GNSS margin removed from the test requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2024-2-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The value of TGNSS_margin for AT command approach shall take the maximum allowed GNSS 2-D position error in 3GPP TS 37.571-1 into consideration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The value of TGNSS_margin was revised to 20.48*64Tc assuming that the position error is 100m. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value of TGNSS_margin will remain undetermined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.3.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion in R4-2401454. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***************************** Start of changes **************************************

A.14.3.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up PCell according to parameters given in Table A.14.3.1.1.1-2.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN–TGNSS_margin)  of the first detected path of DL SSB.

a.	The NTA_offset value (in Tc units) is 25600

b.	The NTA,adjcommon value is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation.

c.	The NTA,adjUE value is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters.

d.	The Te_NTN values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1C.2-1

e.	The TGNSS_margin is the margin for the GNSS position definition error considered in the core requirement, which needs to be substracted for the test requirement, due to the usuage of AT commands in the test. TGNSS_margin=327,6820.48*64×Tc

3)	If the NTN parameters are configured as GSO scenario, the test system shall adjust the timing of the DL path by values given in Table A.14.3.1.1.2-1. If the NTN parameters are configured as NGSO scenario, the test system shall adjust the timing of the DL path according to the serving-satellite-ephemeris-related higher-layers parameters.

Table A.14.3.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1C.2 Table 7.1C.2.1-1 until the UE transmit timing offset is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN–TGNSS_margin)  respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN–TGNSS_margin)  of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

# ***************************** End of changes **************************************