3GPP TSG-RAN WG4 Meeting #110	R4-2401703

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Discussion on demodulation performance requirements for IoT NTN enhancement

Source: 	Huawei, HiSilicon

Agenda item:	9.4.5

Document for:	Discussion

Background

During RAN4#109 meeting, WF [1] on IoT NTN Demod was approved. In this contribution, we share our views about IoT NTN enhancements demodulation performance.

Discussion

Whether to define PDSCH requirements with HARQ disabled

- Agreement: - Continue the discussion in the next meeting.

In Rel-18, the IoT NTN is enhanced to support disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates. However, from our understanding, this feature has no impact on demodulation since only HARQ feedback is different comparing to the legacy procedure. The existing Rel-17 IoT NTN demodulation performance requirements can already verify IoT UE baseband processing under NTN scenario. Additional test setup is needed to test the performance with disabled HARQ feedback that will increase the test complexity. To reduce the test effort, we propose to not define any demodulation performance requirements for IoT NTN enhancements.

Do not define any demodulation performance requirements for IoT NTN enhancements.

Proposals

In this contribution, we discuss on demodulation performance requirements for IoT NTN enhancements. Our observations and proposals are:

- Do not define any demodulation performance requirements for IoT NTN enhancements.

Reference

R4-2321145, WF on [109][332] IoT_NTN_Demod, RAN4#109, MediaTek