# 3GPP TSG-RAN WG4 Meeting #110                                                                R4-2401812

# Athens, GR, 26th  Feb – 01st  Mar, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.786 | CR | 002 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SL_enh2-Core) Big CR to TR 38.786 UE NR sidelink evolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-3-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This big CR is to capture the endorsed draftCR for SL-U feature on RAN4#110 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Belwo endorsed draftCRs are captured: R4-2401531 (NR_SL_enh2-Core) Maintenance CR on requirements of sidelink evolution for TR 38.786 vivo R4-2401465 CR to TR 38.786 NS_28 and NS_30 A-MPR for SL-U LG Electronics R4-2401811 (NR_SL_enh2-Core) CR to TR 38.786 MPR results for PSFCH OPPO R4-2403681 (NR_SL_enh2-Core) CR on updated the Pemax of Sidelink CA for TR 38.786 vivo |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreement of correction is not captured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5, 6, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<start of change>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CC	Component Carriers

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

C-IMD        	Counter Intermodulation Distortion

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DMRS	Demodulation Reference Signal

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

FR2	Frequency Range 2

HD	Half Duplex

IM	Implementation Margin

ITS	Intelligent Transportation System

LTE	Long Term Evolution

LBT	Listen before Talk

MPR	Maximum Power Reduction

NF	Noise Figure

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

ProSe	Proximity-based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSFCH        	Physical Sidelink Feedback CHannel

PSSCH	Physical Sidelink Shared Channel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

S-SSB         	Sidelink Synchronization Signal Block

SEM	Spectrum emission mask

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

<Unchanged Parts Omitted>

## 5.1	Operating bands

### 5.1.1	Operating bands for single carrier operation in unlicensed band

NR Sidelink is designed to operate in the unlicensed operating bands in FR1 defined in Table 5.1.1-1.

Table 5.1.1-1. NR SL-U operating bands in FR1

| NR SL-U operating band | Sidelink (SL) Transmission operating band | Sidelink (SL) Reception operating band | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |
| n461 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | HD | PC5 |
| n961 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | HD | PC5 |
| n1021 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | HD | PC5 |
| NOTE 1: Direct connection between client devices and between vehicular devices in the shared spectrum bands or portions of the shared spectrum bands is subject to country-specific conditions and can be prohibited per region-specific regulatory rules, e.g., in USA and Canada. |  |  |  |  |

<Unchanged Parts Omitted>

### 5.2.3	Channel bandwidth for NR SL CA operation

For NR SL CA operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.3-1.

Table 5.2.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| SL_n47B | SL_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | 20 | 20,30 |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

### 6.1.2	UE maximum output power reduction for NR SL-U

6.1.2.1.3.3	OPPO’s simulation results

For the PSFCH MPR, the simulation cases are listed as below table 1.

Table 1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 30 |
|  | 2 | CP-OFDM | 40 | 50RB0 | 15 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 30 |
|  | 4 | CP-OFDM | 60 | 105RB0 | 30 |
| Wide band operation Interlaced RB allocation | 5 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 6 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 7 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 8 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 9 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 10 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 11 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 12 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 13 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 14 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 15 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 16 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 17 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 18 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 19 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 20 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 2 MPR simulation result for PSFCH single CC

| Single CC interlace | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| QPSK | 10.24 | 13.82 | 12.98 | 15.60 |

Table 3 MPR simulation result for PSFCH Wideband Interlace

| Wideband Interlace | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 9.41 | 9.98 | 13.16 | 9.33 | 12.49 | 9.41 | 14.28 | 9.98 |
| Wideband Interlace | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| QPSK | 13.07 | 9.82 | 12.98 | 14.86 | 9.73 | 12.73 | 14.52 | 9.65 |

From table 2 and table 3 above, it can be observed that for larger channel bandwidth that larger MPR is needed. There is related to the P-APR of multiple repeated sub-channels which is similar also to the S-SSB MPR simulation result.

To also account for some implementation margin, it can be observed that the MPR simulation result of OPPO’s are aligned with the MPR requirement defined in the RAN4#109 meeting.

### 6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

#### 6.1.3.1	A-MPR for SL-U with NS_28

##### 6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.1.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.1.1.1-1: Simulation results for NS_28 Allocation Full and partial allocations.

| NS_28 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤8.0 | ≤8.1 | ≤7.7 | ≤8.0 | ≤7.1 | ≤7.8 | ≤6.9 | ≤7.8 |
| 16 QAM | ≤8.0 | ≤8.3 | ≤7.4 | ≤8.3 | ≤7.3 | ≤8.1 | ≤6.9 | ≤8.1 |
| 64 QAM | ≤8.1 | ≤8.7 | ≤7.4 | ≤8.7 | ≤7.3 | ≤8.2 | ≤7.0 | ≤8.2 |
| 256 QAM | ≤8.2 | ≤8.8 | ≤7.4 | ≤8.8 | ≤7.3 | ≤8.2 | ≤7.1 | ≤8.1 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.1.1.2	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.1.2-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.1.2-1: NS_28-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5480) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 5.47 | 5.89 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 5.89 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 5.90 | 5.47 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.90 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5500) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5490) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.06 | 5.47 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.06 | 5.47 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.06 | 5.48 | 5.47 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.48 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5510) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.53 | 3.91 | 2.81 | 0.39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 3.90 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 3.91 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5550) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5500) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 4.67 | 5.07 | 5.47 | 5.90 | 5.06 | 5.48 | 2.13 | 0.11 |
|  | '16QAM' | 4.67 | 5.07 | 5.47 | 5.89 | 5.06 | 5.47 | 2.13 | 0.10 |
|  | '64QAM' | 4.67 | 5.07 | 5.47 | 5.90 | 5.06 | 5.47 | 3.15 | 2.12 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.89 | 5.47 | 5.47 | 5.47 | 5.05 |
| '60MHz' (5520) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.67 | 2.14 | 0.40 | 3.53 | 4.28 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.29 | 2.14 | 0.40 | 3.53 | 4.28 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.67 | 3.17 | 2.15 | 3.53 | 4.28 | 3.15 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5540) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.82 | 3.17 | 2.15 | 3.17 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5600) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 2.15 | 3.16 | 2.47 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5510) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 4.28 | 4.67 | 5.47 | 5.47 | 5.06 | 5.47 | 4.67 | 5.06 | 2.46 | 0.10 | 2.46 | 3.16 |  |  |  |  |  |  |
|  | '16QAM' | 4.28 | 4.67 | 5.47 | 5.89 | 5.06 | 5.47 | 4.67 | 5.07 | 2.46 | 0.09 | 2.46 | 3.16 |  |  |  |  |  |  |
|  | '64QAM' | 4.28 | 4.67 | 5.47 | 5.47 | 5.06 | 5.07 | 4.67 | 5.06 | 3.16 | 2.12 | 3.16 | 3.15 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.90 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (5530)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 2.14 | 0.40 | 3.53 | 4.28 | 3.90 | 4.28 | 2.46 | 0.09 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 4.28 | 2.15 | 0.39 | 3.53 | 4.28 | 3.90 | 4.28 | 2.46 | 0.36 | 2.13 | 0.63 |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 4.28 | 3.16 | 2.14 | 3.53 | 4.28 | 3.90 | 4.67 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (5610)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.15 | 2.47 | 0.39 | 2.47 | 0.40 | 2.46 | 0.10 | 2.13 | 0.36 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.09 | 2.13 | 0.64 |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.06 | 5.46 | 5.06 |

Table 6.1.3.1.1.2-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.1.2-2: NS_28-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 5480 40MHz : 5490/5510 60MHz : 5500/5520 80MHz : 5510/5530 |  |  |  | 20MHz : 5500 40MHz : 5550 60MHz : 5540/5600 80MHz : 5610 |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 5.48 | 5.90 | 2.46 | 3.16 | 2.81 | 2.82 | 2.46 | 0.36 |
|  | 16 QAM | 5.48 | 5.90 | 2.46 | 3.16 | 2.81 | 2.82 | 2.46 | 0.64 |
|  | 64 QAM | 5.47 | 5.90 | 3.16 | 3.15 | 3.17 | 2.82 | 3.16 | 2.46 |
|  | 256 QAM | 5.48 | 5.90 | 5.47 | 5.06 | 5.48 | 5.08 | 5.88 | 5.06 |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.1.2-2 (left side A-MPR in Table 6.1.3.1.1.2-2) because the distance from the boundary of regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5480 MHz = A-MPR of 5160/5340/5700 MHz

- 40MHz

: A-MPR of 5490 MHz = A-MPR of 5170/5330 MHz

: A-MPR of 5510 MHz = A-MPR of 5190/5310 MHz

- 60MHz

: A-MPR of 5500 MHz = A-MPR of 5180/5320 MHz

: A-MPR of 5520 MHz = A-MPR of 5200/5300/5680 MHz

- 80MHz

: A-MPR of 5510 MHz = A-MPR of 5190/5310 MHz

: A-MPR of 5530 MHz = A-MPR of 5210/5290 MHz

Considering implementation margin and the centre frequencies on the right side of A-MPR in Table Table 6.1.3.1.1.2-2, such as, 5500MHz for CBW 20MHz, 5550MHz for CBW 40MHz, 5540/5600MHz for CBW 60MHz, and 5610MHz for CBW 80MHz, the SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1 can be reused as A-MPR requirement.

As a result, considering implementation margin and the centre frequency, Table 6.1.3.1.1.2-3 can be proposed for SL-U NS_28 PSSCH/PSCCH A-MPR.

Table 6.1.3.1.1.2-3. NS_28 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 5.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 4.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, 5520, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

##### 6.1.3.1.2	A-MPR for S-SSB transmission

6.1.3.1.2.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.2.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.2.1-1: NS_28-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5160) | 10.07 | 8.52 | 7.33 | 7.83 | 6.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5180) | 8.75 | 7.32 | 5.53 | 6.45 | 5.71 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5200) | 8.82 | 7.14 | 5.60 | 6.34 | 5.62 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5700) | 8.80 | 7.17 | 7.81 | 6.46 | 5.54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5170) | 10.98 | 9.88 | 7.77 | 9.86 | 8.73 | 7.43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5190) | 9.35 | 8.21 | 6.09 | 8.70 | 7.23 | 5.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5230) | 9.17 | 8.31 | 6.01 | 8.80 | 7.21 | 5.70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5670) | 9.11 | 8.13 | 5.99 | 8.58 | 7.28 | 5.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5180) | 9.55 | 8.22 | 8.97 | 7.11 | 8.06 | 6.83 | 6.37 | 5.05 | 8.24 | 7.43 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5200) | 8.30 | 7.10 | 7.27 | 6.23 | 6.52 | 5.53 | 6.26 | 5.10 | 8.33 | 7.36 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5220) | 7.95 | 7.03 | 7.28 | 6.17 | 6.51 | 5.65 | 6.46 | 5.04 | 8.46 | 7.15 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5680) | 8.95 | 8.16 | 7.09 | 6.14 | 6.34 | 5.55 | 6.41 | 5.07 | 8.13 | 7.27 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5190) | 10.09 | 7.97 | 9.31 | 7.64 | 8.65 | 7.20 | 7.63 | 6.92 | 7.34 | 5.79 | 6.31 | 4.97 | 8.96 | 7.35 | 8.23 | 7.20 | 8.15 | 7.52 |  |  |
| '80MHz' (5210) | 8.69 | 7.40 | 7.31 | 6.27 | 7.14 | 6.18 | 6.36 | 5.40 | 7.48 | 5.90 | 6.35 | 5.03 | 8.40 | 7.35 | 8.12 | 7.09 | 8.33 | 6.98 |  |  |
| '80MHz' (5610) | 7.48 | 6.82 | 7.47 | 6.35 | 7.14 | 6.19 | 6.30 | 5.52 | 7.51 | 6.02 | 6.48 | 4.84 | 8.66 | 7.36 | 8.28 | 6.96 | 7.13 | 6.85 |  |  |

Table 6.1.3.1.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.2.1-2: NS_28-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation /Centre frequency of CBW (MHz)/ (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz : 5160 40MHz : 5170/5190/5310 60MHz :5180/5200/5220/5520/ 5540/ 5680 80MHz : 5190/5210/5530/5610 |  |  |  | 20MHz : 5180/5200/5700 40MHz : 5230/5510/5670 |  |  |  |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 10.98 | 8.22 | 7.51 | 6.02 | 9.17 | 7.81 | - | - |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.2.1-2 (left side A-MPR in Table 6.1.3.1.2.1-2) because the distance from the boundary of the regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5160 MHz = A-MPR of 5340/5480 MHz

: A-MPR of 5180 MHz = A-MPR of 5320 MHz

- 40MHz

: A-MPR of 5170 MHz = A-MPR of 5330/5490 MHz

: A-MPR of 5190MHz = A-MPR of 5310/5510 MHz

- 60MHz

: A-MPR of 5180 MHz = A-MPR of 5320/5500 MHz

: A-MPR of 5200 MHz = A-MPR of 5300/5520MHz

: A-MPR of 5220 MHz = A-MPR of 5280/5540MHz

- 80MHz

: A-MPR of 5190 MHz = A-MPR of 5310/5510 MHz

: A-MPR of 5210 MHz = A-MPR of 5290/5530 MHz

Considering implementation margin, Table 6.1.3.1.2.1-3 shows proposal for SL-U NS_28 S-SSB A-MPR.

Table 6.1.3.1.2.1-3: NS_28 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |
| --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer/Inner RB sets |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |  |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 | Table 6.2E.2F-5(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |

##### 6.1.3.1.3	A-MPR for PSFCH transmission

6.1.3.1.3.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.3.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.3.1-1: NS_28-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’(5160) | 9.39 | 9.11 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5180) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5200) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5700) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5170) | 10.59 | 9.37 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5190) | 9.08 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5230) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5510) | 8.00 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5670) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |
| ‘60MHz’(5180) | 11.17 | 10.29 | 9.10 | 7.00 | 9.71 |  |  |  |  |  |  |  |
| ‘60MHz’(5200) | 9.83 | 8.65 | 7.30 | 7.00 | 9.71 |  |  |  |  |  |  |  |
| ‘60MHz’(5220) | 9.69 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| ‘60MHz’(5520) | 9.69 | 8.65 | 7.30 | 7.00 | 9.71 |  |  |  |  |  |  |  |
| ‘60MHz’(5540) | 9.69 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| ‘60MHz’(5680) | 10.53 | 8.65 | 7.30 | 7.00 | 9.73 |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |
| '80MHz'(5190) | 11.61 | 10.91 | 10.25 | 9.00 | 9.96 | 7.02 | 10.59 | 9.69 | 9.66 |  |  |  |
| '80MHz'(5210) | 11.16 | 10.05 | 8.71 | 7.43 | 9.96 | 7.02 | 10.17 | 9.69 | 9.66 |  |  |  |
| '80MHz'(5530) | 10.43 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.17 | 9.69 | 9.66 |  |  |  |
| '80MHz'(5610) | 10.43 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.17 | 9.26 | 8.73 |  |  |  |

Table 6.1.3.1.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.3.1-2: NS_28-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz : 5160 40MHz : 5170/5190/5310 60MHz :5180/5200/5220/5520/ 5540/ 5680 80MHz : 5190/5210/5530/5610 |  | 20MHz : 5180/5200 40MHz : 5230/5510/5670 |  |
|  | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 12.23 | 9.96 | 10.2 | - |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.3.1-2 (left side A-MPR in Table 6.1.3.1.3.1-2) because the distance from the boundary of regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5160 MHz = A-MPR of 5340/5480 MHz

: A-MPR of 5180 MHz = A-MPR of 5320 MHz

- 40MHz

: A-MPR of 5170 MHz = A-MPR of 5330/5490 MHz

: A-MPR of 5190MHz = A-MPR of 5310/5510 MHz

- 60MHz

: A-MPR of 5180 MHz = A-MPR of 5320/5500 MHz

: A-MPR of 5200 MHz = A-MPR of 5300/5520MHz

: A-MPR of 5220 MHz = A-MPR of 5280/5540MHz

- 80MHz

: A-MPR of 5190 MHz = A-MPR of 5310/5510 MHz

: A-MPR of 5210 MHz = A-MPR of 5290/5530 MHz

Considering implementation margin, Table 6.1.3.1.3.1-3 shows proposal for SL-U NS_28 PSFCH A-MPR.

Table 6.1.3.1.3.1-3: NS_28 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz hannels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, 5530, and 5610 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |

#### 6.1.3.2	A-MPR for SL-U with NS_29

##### 6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.2.2	A-MPR for S-SSB transmission

##### 6.1.3.2.3	A-MPR for PSFCH transmission

#### 6.1.3.3	A-MPR for SL-U with NS_30

##### 6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.3.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.3.1.1-1: Simulation results for NS_30 Allocation Note 2 Full and partial allocations.

| NS_30 A-MPR Allocation Note2 | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤10.7 | ≤10.9 | ≤10.4 | ≤11.0 | ≤9.3 | ≤10.3 | ≤9.1 | ≤10.4 |
| 16 QAM | ≤10.8 | ≤9.0 | ≤10.1 | ≤11.1 | ≤9.6 | ≤10.5 | ≤9.2 | ≤10.5 |
| 64 QAM | ≤10.9 | ≤11.4 | ≤10.1 | ≤11.4 | ≤9.6 | ≤10.6 | ≤9.3 | ≤10.6 |
| 256 QAM | ≤11.0 | ≤11.5 | ≤10.0 | ≤11.5 | ≤9.7 | ≤10.6 | ≤9.4 | ≤10.5 |

Table 6.1.3.3.1.1-2: Simulation results for NS_30 Allocation Note 3 Full and partial allocations.

| NS_30 A-MPR Allocation Note3 | 20M | 20M | 40M | 40M |
| --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial |
| QPSK | ≤8.3 | ≤8.4 | ≤5.9 | ≤6.2 |
| 16 QAM | ≤8.1 | ≤8.7 | ≤5.8 | ≤6.3 |
| 64 QAM | ≤8.3 | ≤9.0 | ≤5.8 | ≤6.3 |
| 256 QAM | ≤8.4 | ≤8.9 | ≤5.8 | ≤6.3 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.3.1.2	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.3.1.2-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.3.1.2-1: NS_30-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5160) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.51 | 8.52 | 8.97 | 9.44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 8.50 | 8.52 | 8.96 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 8.51 | 8.51 | 8.96 | 9.44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 8.51 | 8.97 | 8.96 | 9.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5180) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 2.83 | 3.90 | 4.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 2.82 | 3.54 | 4.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.83 | 3.91 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5240) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.17 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.17 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5260) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5280) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.15 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5480) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |
|  | 'QPSK' | 7.17 | 7.62 | 8.06 | 8.07 |  |  |  |  |
|  | '16QAM' | 7.17 | 7.63 | 8.06 | 8.07 |  |  |  |  |
|  | '64QAM' | 7.17 | 7.62 | 8.06 | 8.07 |  |  |  |  |
|  | '256QAM' | 7.17 | 7.62 | 8.06 | 8.07 |  |  |  |  |
| '20MHz' (5550) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5680) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.17 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5700) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5170) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.60 | 8.06 | 8.50 | 8.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.60 | 8.06 | 8.50 | 8.52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.60 | 8.06 | 8.50 | 8.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.60 | 8.06 | 8.50 | 8.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5190) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 6.30 | 6.74 | 2.81 | 3.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 6.30 | 6.73 | 2.81 | 3.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 6.30 | 6.74 | 2.81 | 3.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 6.30 | 6.74 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5230) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 |  |  |  |  |
| '40MHz' (5490) | Scenario # | #3 | #9 | #13 | #30 |
|  | 'QPSK' | 7.17 | 7.61 | 7.17 | 7.61 |
|  | '16QAM' | 7.16 | 7.61 | 7.17 | 7.61 |
|  | '64QAM' | 7.16 | 7.61 | 7.17 | 7.62 |
|  | '256QAM' | 7.16 | 7.61 | 7.17 | 7.62 |
| '40MHz' (5510) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 4.66 | 5.07 | 2.47 | 1.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.07 | 2.81 | 1.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.47 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5550) | Scenario # | #3 | #9 | #13 | #30 |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |
| '60MHz' (5180) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 8.05 | 8.06 | 8.96 | 9.90 | 8.50 | 8.96 | 2.46 | 2.46 |
|  | '16QAM' | 8.05 | 8.05 | 8.96 | 9.90 | 8.50 | 8.97 | 2.46 | 2.79 |
|  | '64QAM' | 8.05 | 8.06 | 8.96 | 9.43 | 8.50 | 9.42 | 3.16 | 2.80 |
|  | '256QAM' | 8.05 | 8.06 | 8.96 | 9.89 | 8.50 | 8.97 | 5.47 | 5.06 |
| '60MHz' (5200) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 6.73 | 7.17 | 3.90 | 4.28 | 6.73 | 7.17 | 2.13 | 0.10 |
|  | '16QAM' | 6.73 | 7.17 | 3.90 | 4.29 | 6.30 | 7.17 | 2.13 | 0.36 |
|  | '64QAM' | 6.73 | 7.17 | 3.90 | 4.67 | 6.31 | 7.17 | 3.16 | 2.12 |
|  | '256QAM' | 6.73 | 7.17 | 5.47 | 5.07 | 6.30 | 7.17 | 5.88 | 5.05 |
| '60MHz' (5220) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 5.06 | 5.88 | 2.14 | 2.15 | 2.47 | 2.47 | 2.13 | 0.10 |
|  | '16QAM' | 5.06 | 5.89 | 2.14 | 2.15 | 2.47 | 2.48 | 2.13 | 0.36 |
|  | '64QAM' | 5.06 | 5.89 | 3.16 | 2.15 | 3.17 | 2.48 | 3.16 | 2.13 |
|  | '256QAM' | 5.47 | 5.89 | 5.48 | 5.07 | 5.47 | 5.07 | 5.88 | 5.06 |
| '60MHz' (5280) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 4.67 | 5.48 | 2.14 | 0.14 | 2.47 | 0.66 | 2.13 | 0.36 |
|  | '16QAM' | 4.67 | 5.48 | 2.14 | 0.40 | 2.47 | 0.65 | 2.13 | 0.35 |
|  | '64QAM' | 4.67 | 5.47 | 3.17 | 2.16 | 2.81 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.47 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.06 |
| '60MHz' (5500) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 7.17 | 7.61 | 8.05 | 8.51 | 7.61 | 8.06 | 2.13 | 1.19 |
|  | '16QAM' | 7.17 | 7.61 | 8.05 | 8.51 | 7.60 | 8.06 | 2.13 | 1.49 |
|  | '64QAM' | 7.17 | 7.61 | 8.05 | 8.51 | 7.61 | 8.06 | 3.16 | 2.12 |
|  | '256QAM' | 7.17 | 7.61 | 8.05 | 8.51 | 7.61 | 8.06 | 5.47 | 5.05 |
| '60MHz' (5520) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.88 | 6.31 | 2.47 | 3.17 | 4.66 | 5.07 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.88 | 6.74 | 2.48 | 3.17 | 4.67 | 5.07 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.88 | 6.74 | 3.17 | 3.17 | 4.66 | 5.07 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 6.31 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5540) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 3.17 | 3.90 | 2.14 | 0.39 | 2.47 | 1.22 | 2.13 | 0.10 |
|  | '16QAM' | 3.17 | 3.90 | 2.14 | 0.40 | 2.47 | 1.22 | 2.13 | 0.36 |
|  | '64QAM' | 3.17 | 3.90 | 3.17 | 2.14 | 3.17 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.06 |
| '60MHz' (5680) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 3.17 | 3.90 | 2.14 | 0.15 | 2.47 | 0.66 | 2.13 | 0.10 |
|  | '16QAM' | 3.17 | 3.90 | 2.15 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |
|  | '64QAM' | 3.17 | 3.91 | 3.16 | 2.15 | 3.17 | 2.48 | 3.16 | 2.13 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 |
| '80MHz' (5190) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |
|  | 'QPSK' | 7.60 | 7.61 | 8.96 | 8.98 | 8.50 | 8.96 | 8.05 | 8.05 | 2.46 | 3.15 | 5.88 | 6.30 |  |  |  |
|  | '16QAM' | 7.60 | 7.61 | 8.51 | 9.90 | 8.50 | 8.96 | 8.05 | 8.06 | 2.80 | 3.15 | 5.88 | 6.30 |  |  |  |
|  | '64QAM' | 7.60 | 7.61 | 8.96 | 9.89 | 8.96 | 8.96 | 8.05 | 8.05 | 3.16 | 3.15 | 5.88 | 6.30 |  |  |  |
|  | '256QAM' | 7.61 | 7.61 | 8.51 | 9.90 | 8.50 | 8.96 | 8.05 | 8.05 | 5.46 | 5.06 | 5.88 | 6.30 |  |  |  |
| '80MHz' (5210) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 6.73 | 7.17 | 3.90 | 4.29 | 6.30 | 7.17 | 6.73 | 7.17 | 2.46 | 0.10 | 2.13 | 2.13 |
|  | '16QAM' | 6.73 | 7.17 | 3.90 | 4.28 | 6.30 | 7.17 | 6.73 | 7.17 | 2.46 | 0.10 | 2.13 | 2.13 |
|  | '64QAM' | 6.73 | 7.17 | 3.53 | 4.29 | 6.31 | 7.17 | 6.73 | 7.17 | 3.15 | 2.12 | 2.81 | 2.47 |
|  | '256QAM' | 6.73 | 7.17 | 5.47 | 5.07 | 6.30 | 7.17 | 6.73 | 7.17 | 5.47 | 5.06 | 5.47 | 5.06 |
| '80MHz' (5510) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 7.16 | 7.17 | 8.05 | 8.07 | 7.60 | 8.06 | 7.17 | 7.61 | 2.46 | 1.80 | 4.27 | 4.66 |
|  | '16QAM' | 7.16 | 7.17 | 8.06 | 8.07 | 7.61 | 7.61 | 7.17 | 7.61 | 2.46 | 1.80 | 4.66 | 4.66 |
|  | '64QAM' | 7.17 | 7.17 | 8.06 | 8.07 | 7.61 | 8.05 | 7.16 | 7.61 | 3.15 | 2.12 | 4.27 | 4.66 |
|  | '256QAM' | 7.17 | 7.17 | 8.05 | 8.06 | 7.61 | 8.05 | 7.17 | 7.61 | 5.46 | 5.05 | 5.47 | 5.06 |
| '80MHz' (5530) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 6.30 | 6.73 | 2.14 | 2.15 | 4.67 | 5.06 | 5.88 | 6.30 | 2.46 | 0.10 | 2.13 | 0.91 |
|  | '16QAM' | 6.30 | 6.73 | 2.14 | 2.15 | 4.67 | 5.07 | 5.88 | 6.31 | 2.46 | 0.09 | 2.13 | 0.91 |
|  | '64QAM' | 6.30 | 6.73 | 3.17 | 2.15 | 4.67 | 5.07 | 5.88 | 6.31 | 3.15 | 2.13 | 2.80 | 2.46 |
|  | '256QAM' | 6.30 | 6.73 | 5.47 | 5.08 | 5.47 | 5.07 | 5.88 | 6.31 | 5.47 | 5.06 | 5.47 | 5.06 |
| '80MHz' (5610) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.14 | 2.47 | 0.40 | 2.48 | 0.40 | 2.46 | 0.10 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.47 | 0.66 | 2.46 | 0.36 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.47 | 3.17 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |

Table 6.1.3.3.1.2-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.3.1.2-2 : NS_30-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 5160/5480 40MHz : 5170/5190/5490 60MHz : 5180/5200/5500/5520 80MHz : 5190/5210/5510/5530 |  |  |  | 20MHz : 5180 40MHz : 5510 60MHz : 5220/5280/5540/5680 80MHz : |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 12.74 | 14.21 | 5.88 | 6.30 | 5.06 | 5.88 | 2.13 | 0.36 |
|  | 16 QAM | 12.74 | 14.21 | 5.88 | 6.30 | 5.06 | 5.89 | 2.13 | 0.36 |
|  | 64 QAM | 12.26 | 14.21 | 5.88 | 6.30 | 5.06 | 5.89 | 3.16 | 2.13 |
|  | 256 QAM | 12.74 | 14.21 | 5.88 | 6.30 | 5.48 | 5.89 | 5.88 | 5.06 |

The following centre frequencies for each CBW can be assumed to have same A-MPR results in Table 6.1.3.3.1.2-1 (left side A-MPR in Table 6.1.3.3.1.2-1) because the distance from ‘-41dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5160 MHz = A-MPR of 5340/5480MHz

- 40MHz

: A-MPR of 5170MHz = A-MPR of 5330/5490 MHz

: A-MPR of 5190 MHz = A-MPR of 5310 MHz

- 60MHz

: A-MPR of 5180MHz = A-MPR of 5320/5500 MHz

: A-MPR of 5200 MHz = A-MPR of 5300/5520MHz

- 80MHz

: A-MPR of 5190 MHz = A-MPR of 5310/5510 MHz

: A-MPR of 5210 MHz = A-MPR of 5290/5530 MHz

The following centre frequencies for each CBW can be assumed to have same A-MPR results in Table 6.1.3.3.1.2-1 (right side A-MPR in Table 6.1.3.3.1.2-1) because the distance from ‘-41dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5180 MHz = A-MPR of 5320 MHz

- 60MHz

: A-MPR of 5220 MHz = A-MPR of 5280/5540MHz

For other centre frequencies from Table 6.1.3.3.1.2-1, considering implementation margin, SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1 can be reused as A-MPR requirement.

As a result, considering implementation margin and the centre frequencies, Table 6.1.3.3.1.2-3 shows proposal for SL-U NS_30 PSSCH/PSCCH A-MPR.

Table 6.1.3.3.1.2-3 : NS_30 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation3 |  |  |  | RB Allocation4 |  |  |  | RB Allocation (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 12.0 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 3.0 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 10.0 | ≤ 12.5 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 3.0 | ≤ 2.0 |  |
|  | 64 QAM | ≤ 10.0 | ≤ 13.0 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 3.0 |  |
|  | 256 QAM | ≤ 10.5 | ≤ 13.5 | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5510 MHz, and 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5220, 5280, 5540, and 5680 MHz. NOTE 5: Applicable for all valid channels other than those enumerated under NOTE 3 and NOTE 4. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.3.2	A-MPR for S-SSB transmission

6.1.3.3.2.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.3.2.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.3.2.1-1: NS_30-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5160) | 27.99 | 26.67 | 24.24 | 21.76 | 21.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5180) | 8.89 | 7.27 | 5.66 | 6.45 | 5.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5200) | 8.78 | 7.16 | 5.63 | 6.46 | 5.72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5320) | 8.94 | 7.33 | 5.53 | 6.27 | 5.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5340) | 32.80 | 32.11 | 24.25 | 26.33 | 24.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5480) | 18.31 | 20.19 | 24.26 | 9.90 | 9.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5700) | 8.74 | 7.25 | 6.72 | 6.58 | 5.59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5170) | 29.81 | 29.49 | 24.42 | 28.15 | 26.71 | 24.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5190) | 20.42 | 11.45 | 8.69 | 8.92 | 7.24 | 5.41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5230) | 9.24 | 8.21 | 6.04 | 8.77 | 7.31 | 5.60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5310) | 14.82 | 11.44 | 8.89 | 8.69 | 7.25 | 5.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5330) | 32.39 | 31.37 | 25.18 | 8.77 | 7.34 | 5.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5490) | 28.78 | 28.52 | 23.93 | 17.48 | 19.67 | 8.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5510) | 9.16 | 8.46 | 8.59 | 8.70 | 7.41 | 5.42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5670) | 9.20 | 8.22 | 5.98 | 8.87 | 7.32 | 5.56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5180) | 27.98 | 24.92 | 25.31 | 21.72 | 21.26 | 21.14 | 6.37 | 4.96 | 22.67 | 21.01 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5200) | 12.28 | 9.44 | 10.61 | 8.26 | 6.71 | 5.64 | 6.38 | 5.15 | 23.63 | 21.62 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5220) | 9.02 | 7.08 | 7.16 | 5.88 | 6.44 | 5.64 | 6.36 | 5.06 | 10.43 | 7.80 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5280) | 7.94 | 6.97 | 7.10 | 6.37 | 6.37 | 5.64 | 6.27 | 5.11 | 9.87 | 7.97 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5500) | 27.47 | 25.22 | 24.72 | 20.96 | 12.48 | 21.35 | 6.56 | 4.94 | 23.84 | 21.79 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5520) | 10.80 | 9.54 | 7.16 | 8.35 | 6.36 | 5.65 | 6.40 | 5.18 | 23.83 | 9.17 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5540) | 7.90 | 7.07 | 7.09 | 6.20 | 6.57 | 5.60 | 6.38 | 4.86 | 8.23 | 8.04 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5680) | 8.22 | 7.65 | 7.18 | 6.21 | 6.56 | 5.85 | 6.47 | 4.95 | 8.34 | 7.05 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5190) | 28.34 | 25.27 | 27.83 | 22.70 | 24.64 | 21.38 | 21.32 | 20.77 | 9.37 | 8.07 | 6.42 | 5.24 | 25.12 | 21.36 | 23.17 | 21.78 | 23.19 | 21.42 |  |  |
| '80MHz' (5210) | 22.59 | 10.27 | 11.81 | 8.91 | 10.80 | 8.06 | 6.39 | 5.50 | 7.53 | 5.90 | 6.21 | 4.95 | 23.46 | 9.53 | 10.81 | 21.13 | 10.80 | 8.99 |  |  |
| '80MHz' (5290) | 22.15 | 10.27 | 7.76 | 6.27 | 7.12 | 6.40 | 6.49 | 5.91 | 7.42 | 5.94 | 6.26 | 4.96 | 11.75 | 8.90 | 9.43 | 7.79 | 10.69 | 9.19 |  |  |
| '80MHz' (5310) | 28.76 | 23.76 | 11.59 | 9.12 | 7.11 | 6.28 | 6.28 | 5.62 | 10.82 | 8.28 | 6.44 | 5.07 | 26.26 | 21.37 | 23.82 | 9.06 | 26.44 | 21.21 |  |  |
| '80MHz' (5510) | 26.13 | 25.05 | 25.56 | 22.97 | 23.55 | 21.27 | 11.81 | 20.54 | 7.55 | 8.02 | 6.30 | 4.95 | 23.95 | 9.98 | 10.83 | 21.44 | 10.49 | 21.20 |  |  |
| '80MHz' (5530) | 11.69 | 10.36 | 10.77 | 8.63 | 7.21 | 8.07 | 6.65 | 5.65 | 7.44 | 5.88 | 6.20 | 4.94 | 23.81 | 9.43 | 10.98 | 20.95 | 10.79 | 9.18 |  |  |
| '80MHz' (5610) | 7.63 | 6.78 | 7.25 | 6.33 | 7.33 | 6.07 | 6.39 | 5.64 | 7.70 | 5.92 | 6.47 | 5.03 | 8.55 | 7.45 | 8.12 | 6.92 | 7.07 | 7.02 |  |  |

Table 6.1.3.3.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.3.2.1-2: NS_30-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation /Centre frequency of CBW (MHz)/ (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz : 5160/5340/5480 40MHz : 5170/5190/5310/5330/5490 60MHz : 5180/5200/5500/5520 80MHz : 5190/5210/5290/5310/5510/5530 |  |  |  | 20MHz : 5180/5200/5320/5700 40MHz : 5230/5510/5670 60MHz : 5220/5280/5540/5680 80MHz : 5610 |  |  |  |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 32.80 | 25.27 | 10.82 | 8.28 | 10.43 | 8.59 | 7.70 | 5.92 |

Considering implementation margin, Table 6.1.3.3.2.1-3shows proposal for SL-U NS_30 S-SSB A-MPR.

Table 6.1.3.3.2.1-3:  NS_30 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 35.5 | ≤ 28.0 | ≤ 13.5 | ≤ 11.0 | ≤ 13.0 | ≤ 11.0 | ≤ 10.5 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |

##### 6.1.3.3.3	A-MPR for PSFCH transmission

6.1.3.3.3.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.3.3.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.3.3.1-1: NS_30-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’(5160) | 26.72 | 25.66 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5180) | 7.38 | 9.33 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5200) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5320) | 7.43 | 8.82 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5340) | 32.89 | 30.83 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5480) | 21.05 | 16.83 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5700) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5170) | 29.78 | 26.69 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5190) | 12.58 | 7.85 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5230) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5310) | 12.67 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5330) | 33.21 | 8.86 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5490) | 28.26 | 21.27 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5510) | 12.27 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5670) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |
| ‘60MHz’(5180) | 31.10 | 28.95 | 25.70 | 7.69 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5200) | 23.10 | 18.41 | 9.63 | 7.00 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5220) | 12.51 | 8.65 | 7.33 | 7.00 | 11.24 |  |  |  |  |  |  |  |
| ‘60MHz’(5280) | 12.91 | 8.65 | 7.30 | 7.00 | 11.35 |  |  |  |  |  |  |  |
| ‘60MHz’(5500) | 30.21 | 26.85 | 16.92 | 7.00 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5520) | 12.98 | 8.65 | 8.45 | 7.00 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5540) | 9.69 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| ‘60MHz’(5680) | 9.87 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |
| '80MHz'(5190) | 31.31 | 30.67 | 28.58 | 24.90 | 13.18 | 7.92 | 28.39 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5210) | 27.86 | 13.81 | 12.75 | 9.29 | 9.96 | 7.02 | 27.41 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5290) | 27.41 | 12.25 | 8.43 | 7.43 | 9.96 | 7.02 | 24.90 | 11.30 | 26.55 |  |  |  |
| '80MHz'(5310) | 31.77 | 24.59 | 8.90 | 8.27 | 13.20 | 7.02 | 29.01 | 26.41 | 28.29 |  |  |  |
| '80MHz'(5510) | 30.04 | 29.18 | 26.57 | 17.16 | 12.65 | 7.02 | 27.41 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5530) | 26.05 | 13.81 | 10.92 | 7.94 | 9.96 | 7.02 | 27.22 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5610) | 10.43 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.17 | 9.26 | 8.73 |  |  |  |

Table 6.1.3.3.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.3.3.1-2: NS_30-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz : 5160/5340/5480 40MHz :5170/5190/5310/5330/ 5490/5510 60MHz :5180/5200/5500/5520 80MHz : 5190/5210/5290/5310/ 5510/5530 |  | 20MHz : 5180/5200/5320/5700 40MHz : 5230/5510/5670 60MHz : 5220/5280/5540/5680 80MHz : 5610 |  |
|  | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 33.21 | 13.20 | 12.91 | 9.96 |

Considering implementation margin, Table 6.1.3.3.3.1-3 shows proposal for SL-U NS_30 PSFCH A-MPR.

Table 6.1.3.3.3.1-3 : NS_30 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | RB Allocation2 |  | RB Allocation3 |  |
|  | Outer RB set configuration | Inner RB set configuration | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 36.0 | ≤ 16.0 | ≤ 15.5 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

### 6.2.2	UE maximum output power reduction for inter-band con-current operation

It has been agreed to refer to the existing requirement for NR Uu and refer to the SL-U requirement if specified.

For the inter-band con-current operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in TS 38.101-1 clause 6.2.2 apply for NR Uu operation in licensed band, and the MPR requirements in clause 6.1.2 apply for NR sidelink operation in unlicensed band.

### 6.2.3	UE additional maximum output power reduction for inter-band con-current operation

It has been agreed to refer to the existing requirement for NR Uu and refer to the SL-U requirement if specified.

For the inter-band con-current operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in TS 38.101-1 clause 6.2.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in clause 6.1.2 apply for NR sidelink operation in unlicensed band.

### 6.2.5	Minimum output power for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.3.1 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.1.5 shall apply for NR sidelink operation in unlicensed band.

### 6.2.6	Transmit OFF power for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.3.2 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.1.6 shall apply for NR sidelink operation in unlicensed band

### 6.2.8	Power control for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.3.4 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.1.8 shall apply for NR sidelink operation in unlicensed band

### 6.2.10	Spectrum emission mask for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the general/additional SEM requirements specified in TS 38.101-1 clause 6.5.2 shall apply for NR Uu operation in licensed band and the general/additional SEM requirements specified in clause 6.1.10 shall apply for NR sidelink operation in unlicensed band.

### 6.2.11	ACLR requirements for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the ACLR requirement specified in clause TS 38.101-1 6.5.2.4 shall apply for NR Uu operation in licensed band and the ACLR requirement specified in clause 6.1.11 shall apply for NR sidelink operation in unlicensed band.

### 6.2.14	Transmit intermodulation for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.5.4 shall apply for NR Uu operation in licensed band and the requirements specified in clause 6.1.14 shall apply for NR sidelink operation in unlicensed band.

<Unchanged Parts Omitted>

#### 6.3.9.3	Carrier Leakage

For SL intra-band contiguous CA, the carrier leakage requirement as specified in clause 6.4E.2.3 in TS38.101-1 shall be applied per component carrier when only one SL transmission carrier is activated in a time.

<Unchanged Parts Omitted>

### 6.3.4	Configured transmitted power for NR SL CA operation

For intra-band contiguous SL CA operation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.3.2 and subclause 6.3.3 respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤  PCMAX,  ≤  PCMAX_H

For SL transmission of intra-band contiguous CA when same slot pattern is used in all aggregated component carriers.

PCMAX_L = MIN{10 log10 ∑ pEMAX,C - TC, PPowerClass, SL_CA – MAX(MAX(MPR,  A-MPR) + ΔTIB,c+TC, P-MPR ), PRegulatory, PEMAX, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,C , PPowerClass, SL_CA, PRegulatory, PEMAX, CA }

where

-	For the total transmitted power PCMAX,PSSCH/PSCCH, pEMAX, C is the value given by the IE sl-maxTransPower for each component carrier and PEMAX,CA is the value given by the  IE sl-maxTransPower-CA for maximum transmitted power of SL CA, defined by TS 38.331;

-	PPowerClass, SL_CA is the maximum UE power specified in Table 6.3.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.3.2 and subclause 6.3.3 respectively;

-	TIB,c  and P-MPR are specified in clause 6.2.4 in TS38.101-1;

-	TC is the highest value TC,c among all component carriers c in the subframe over both timeslots. TC,c = 1.5 dB when NOTE 3 in Table 6.2.1-1 in TS38.101-1 applies, otherwise TC,c = 0 dB;

-	PRegulatory= 10 - Gpost connector dBm when V2X UE is within the protected zone in ETSI TS 102 792 of CEN DSRC tolling system and operating in Band n47; PRegulatory= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For intra-band SL CA operation, when at least one different numerology/slot pattern is used in aggregated cells, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

The measured configured maximum output power PUMAX,c for sidelink CA operation, when at least one slot has a different transmission numerology or slot pattern, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

### 7.1.1	Reference sensitivity power level for NR SL-U

Background information for NR V2X reference sensitivity requirement has been captured in the TR 38.886 as captured below and can be reused for NR SL-U

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +NFV2X+ IM – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: Implementation Margin 2.5 dB is assumed. When the number of RB size equal to or less than 24RBs, 0.5dB additional relaxation is allowed.

-	SNRV2X:Target SNR for NR V2X is -0.5 dB

-	Diversity gain: 3dB

For NR-U, the following equations in table below are used to calculate the REFSENS requirement and it can be further re-used in NR SL-U in bands n46, n96 and n102, while taking account that for NR-U target SNR of -1dB was used and therefore the REFSENS values in table below are 0.5dB higher than with NR SL-U target SNR of 0.5dB.

Table 7.1.1-1 REFSENS for NR-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -90.1 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -89.2 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 of TS 38.101-1. |  |  |  |  |

For the NR-U REFESENS requirement, the RB configuration for larger than 20MHz is considered as wideband operation and hence all intra-cell guard bands between sub-bands should be filled. In this case, the full RB allocation of NR is used to define the reference sensitivity requirement. However, for V2X, specific RB allocation using the sub-channels based on the RAN2 specification TS 38.331 is used as captured below:

Following the TS 38.331 definitions Table 7.1.1-2 shows the RB configuration for NR SL-U reference sensitivity requirement.

Table 7.1.1-2 NR SL-U RB configurations for REFSENS

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | 135 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | 135 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | 135 |  |

With the agreed SNR and RB configuration as above, the REFSENS requirement for NR SL-U is  shown in Table 7.1.1-3.

Table 7.1.1-3 REFSENS for SL-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |