3GPP TSG-RAN WG4 Meeting #110	R4-2402406

# Athens, Greece, February 26 – March 1, 2024

Source:			Nokia, Nokia Shanghai Bell, Skyworks Solutions Inc.

Title:			NS_17 for Band n28 3 MHz operation

Agenda item:	8.10.2

Document for:	Approval

# Introduction

3 MHz operation in E-UTRA band 28 for a block 715-718 / 770-773 MHz allocated recently in Japan has been introduced to TS 36.101 [1-6]. Japanese regulations [7-10] require the protection of DTV band (470-710 MHz) at the level of -26.2 dBm/6 MHz, which is specified in the network signaling value NS_17 but was previously only specified for 5 MHz or wider channel bandwidth. Since A-MPR is not required for NS_17, it was possible to reuse the same NS_17 for 3 MHz channel bandwidth.

Further, 3 MHz channel bandwidth has recently been introduced to NR specifications [11,12], however, 3 MHz operation in NR band n28 for Japan is not properly specified yet. A CR [13] was proposed in RAN4#109 to align the NR spec with the E-UTRA spec on NS_17. However, the CR was not agreed as there was no confirmation if NS_17 can be reused without A-MPR for NR waveforms.

In this paper, we present NR A-MPR simulations for NS-17 for 3 MHz operation in Japan and propose a wayforward.

# Discussion

In a similar way to what we have simulated previously for E-UTRA in [1], we have run the A-MPR simulations for a 3 MHz NR carrier at 716.5 MHz for NS_17 emission requirement. We have simulated NR waveforms (WOLA-based CP-OFDM and SC-FDMA) for all modulations with contiguous allocations using our UE transmitter model which has been used to derive various A-MPR values in RAN4.

The result is that no A-MPR (or RB restriction) is needed also for the NR like our LTE simulations [1]. In Figure 1, you can find simulated spectrum for SC-FDMA on full allocation with QPSK, which is the worst case (due to smaller MPR than CP-OFDM or higher modulations) among all the simulated cases. We have observed 6.8 dB margin for NS_17, which indicates that the NR UEs can fulfill regulations easily for the 3 MHz operation with a sufficient margin.

We have also attached a figure of 1 PRB allocation at the lower channel edge, where we have observed plenty of margin (34.3 dB) for NS_17.

In both cases, general emission mask looks more stringent requirement than NS_17. (Indeed, our model has zero margin for the SEM.)

Observation 1: NS_17 can be extended without A-MPR or RB restriction for NR band n28 3 MHz operation in Japan.

Although it is not planned yet to deploy NR 3 MHz channel bandwidth in Japan, it is recommended to align the NS_17 in NR specifications with LTE as 3 MHz channel bandwidth at this stage when we just introduced 3 MHz channel bandwidth in the NR. This can avoid a later introduction of CR to modify NS_17 in case NR 3 MHz is deployed in Japan.

Proposal 1: It is proposed to add 3 MHz channel bandwidth for NS_17 in the NR spec.

Figure 1: Transmitter spectrum for full RB allocation

Figure 2: Transmitter spectrum for the edge RB allocation

# Summary

We have run A-MPR simulations for NS_17 for the 3 MHz operation in NR band n28 in Japan. It is observed that NS_17 emission requirement can be fulfilled without A-MPR or RB restriction.

Observation 1: NS_17 emission requirement can be fulfilled without A-MPR or RB restriction for NR band n28 3 MHz operation in Japan.

Proposal 1: It is proposed to add 3 MHz channel bandwidth for NS_17 in the NR spec.

# Reference

[1] R4-2312529 [TEI15] Band 28 3 MHz operation for Japan [Japan_3MHz_n28], Nokia, Nokia Shanghai Bell, Rakuten Mobile, Inc, RAN4#108

[2] R4-2312530 [TEI15] CR TS 36.101: Band 28 3 MHz operation for Japan [Japan_3MHz_n28], Nokia, Rakuten Mobile, Inc, RAN4#108

[3] R4-2312531 [TEI15] CR TS 36.101: Band 28 3 MHz operation for Japan [Japan_3MHz_n28], Nokia, Rakuten Mobile, Inc, RAN4#108

[4] R4-2312532 [TEI15] CR TS 36.101: Band 28 3 MHz operation for Japan [Japan_3MHz_n28], Nokia, Rakuten Mobile, Inc, RAN4#108

[5] R4-2312533 [TEI15] CR TS 36.101: Band 28 3 MHz operation for Japan [Japan_3MHz_n28], Nokia, Rakuten Mobile, Inc, RAN4#108

[6] R4-2312533 [TEI15] CR TS 36.101: Band 28 3 MHz operation for Japan [Japan_3MHz_n28], Nokia, Rakuten Mobile, Inc, RAN4#108

[7] R4-2300500 Discussion on new frequency allocation in Japan, Rakuten Mobile, Inc, KDDI Corporation, NTT DOCOMO, INC., Softbank Corp., RAN4#106

[8] https://www.soumu.go.jp/menu_news/s-news/01kiban14_02000598.html

[9] https://www.soumu.go.jp/menu_news/s-news/01kiban09_02000480.html

[10] https://www.soumu.go.jp/menu_news/s-news/01kiban09_02000484.html

[11] RP-234062 Summary for WI NR support for dedicated spectrum less than 5MHz for FR1, Nokia, RAN#102

[12] RP-232749 Status report for WI NR support for dedicated spectrum less than 5MHz for FR1, Nokia, RAN#102

[13] R4-2319808 CR to R18 TS38.101-1 to Correct NS17 and NS18 emission requirement table, Skyworks Solutions Inc., RAN4#109

[14] R4-2402407 NS_17 correction on Band n28 3 MHz operation in Japan, Nokia, RAN4#110