3GPP TSG-RAN WG4 Meeting #111	R4-2407015

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	7.8.7.1

Source:	MediaTek Inc.

Title:	Simulation results on UE demodulation requirements for less than 5MHz

Document for:	Information

# Introduction

In this contribution, we provide simulation results on punctured PBCH and PDCCH for less than 5MHz WI.

# Simulation Results

      - 2.1 Simulation results for punctured PBCH

Based on agreements in RAN4#110, we provide simulation results for PBCH in Figure 1 and 2.

Issue 1-4-1: PBCH requirement in non-HST conditions Agreement: Use the following parameters for PBCH requirement in normal conditions. Interested companies are encouraged to bring simulation results. Duplex; Bandwidth (MHz) / Subcarrier spacing (kHz); SSB/PBCH index; Propagation condition; Antenna configuration and correlation matrix; Reference value. Pm-bch (%); SNR (dB). FDD; 3 / 15; Unknown; TDLC300-100; 1 x 2 Low,; 1; TBD. 1x4 Low; 1; TBD.

| Figure 1. PBCH performance for 1T2R | Figure 2. PBCH performance for 1T4R |
| --- | --- |

The SNR values achieving 1% Pm-bch are summarized in Table 1.

Table 1. SNR values achieving 1% Pm-bch

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | Alignment SNR (dB) | Impairment SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1x2 Low | 1 | -5.4 | -2.9 |
|  |  |  |  | 1x4 Low | 1 | -7.5 | -5.0 |

      - 2.2 Simulation results for punctured PDCCH

RAN4 agreed to define PDCCH requirements for 3MHz with 15 PRBs with antenna configuration 1x2 and 2x4 [1].

Agreement: - Define two test cases for punctured PDCCH in less than 5 MHz CBW with 1x2 and 2x4 antenna configurations: Number of Tx; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLA30-10; 1x2 Low; 1; [TBA]. 2; 3; 15; 3; 4; [R.PDCCH. 1-2.x FDD]; TDLC300-100; 2x4 Low; 1; [TBA].

Simulation curves for BLER versus SNR are shown in Figure 3 and 4 with the assumption that partial CCE is punctured at the transmitter side.

| Figure 3. punctured PDCCH performance for 1T2R | Figure 4. punctured PDCCH performance for 2T4R |
| --- | --- |

In Table 2, we summarize the SNR values achieving 1% BLER.

Table 2. SNR values achieving 1% BLER

| CBW (MHz) | CORESET RB | Payload size | AL | Propagation condition | Antenna configuration | SNR @ 1% BLER (alignment) | SNR @ 1% BLER (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 35 | 4 | TDLA30-10 | 1x2 Low | 4.3 | 6.8 |
| 3 | 15 | 35 | 4 | TDLC300-100 | 2x4 Low | -4.1 | -1.6 |

# Conclusion

In this contribution, we provide simulation results, including alignment and impairment results, on punctured PBCH and PDCCH for less than 5MHz WI.

# Reference

- R4-2406044, “Way Forward for [110bis][323] NR_FR1_lessthan_5MHz_BW_demod”, Nokia, Nokia Shanghai Bell