3GPP RAN WG4 Meeting #111		R4-2407093

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	10.1.1.2.3

Source:	Apple

Title:	Views on increasing UE transmission high power limit

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their initial views on the respective features above where the discussions were concluded with an approved WF on “HPUE for UL CA and EN-DC” [2]. In this contribution, we share our further views on “increasing UE transmission high power limit for inter-band UL CA and EN-DC” and propose the MSD framework under this feature to follow the same principle of PC2 inter-band UL CA/EN-DC with higherPowerLimit-r17/ higherPowerLimitMRDC-r17 capability.

# Discussion

The “Increasing UE power high limit for CA and DC” feature was introduced in Rel-17 [3] with the intention to maximize the UE per-band power class capability while transmitting in two UL bands simultaneously which would have been underutilized without the feature due to the total maximum output power restriction in current 3GPP UE power class definition. In Rel-17, only PC2 UL configurations with (PC3 FDD/TDD + PC2 TDD) composition have been specified to support this feature. In Rel-18, the feature was further extended to include UL configurations with (PC3 FDD/TDD + PC5 TDD) composition.

With the introduction of PC2 FDD bands and PC1.5 inter-band UL CA and EN-DC, it opens up the opportunity for many new possible UL configurations where the “increasing UE power high limit for CA and DC” feature can also be leveraged, as summarized in Table 2-1.

| Power Class | UL Configuration |
| --- | --- |
| PC2 | PC2 FDD + PC5 TDD |
| PC2 | PC2 FDD + PC3 TDD |
| PC2 | PC2 FDD + PC3 FDD |
| PC1.5 | PC1.5 TDD + PC5 TDD |
| PC1.5 | PC1.5 TDD + PC3 TDD |
| PC1.5 | PC1.5 TDD + PC3 FDD |
| PC1.5 | PC1.5 TDD + PC2 TDD |
| PC1.5 | PC1.5 TDD + PC2 FDD |

Table 2-1 Possible new UL configurations to support “increasing UE power high limit” feature

For UL configurations consist of at least one FDD band, it may be subject to 2UL IMD interference issue. If PCMAX for each constituent UL band would be applied to evaluate MSD requirements, new MSD framework would be needed for these UL configurations. However, this will not only create more fragmentations in MSD test configurations but also increase specifications complexity substantially. To avoid the unnecessary new MSD framework to support the “increasing UE power high limit for CA and DC” feature, for PC2 with (PC3 FDD + PC2 TDD), RAN4 has already agreed that the 2UL IMD MSD test configuration for the same PC2 combination without the “increasing UE power high limit” feature would be reused. Therefore, for the possible new UL configurations to support “increasing UE power high limit” feature, we propose the MSD framework to also follow the same principle of PC2 inter-band UL combinations with “increasing UE power high limit” feature.

Observation 1: For UL configurations consist of at least one FDD band, it may be subject to 2UL IMD interference issue. If PCMAX for each constituent UL band would be applied to evaluate MSD requirements, new MSD framework would be needed for these UL configurations.

Observation 2: For PC2 with (PC3 FDD + PC2 TDD) and “increasing UE power high limit” feature, RAN4 has already agreed that the 2UL IMD MSD test configuration for the same PC2 combination without the “increasing UE power high limit” feature would be reused.

Proposal: For the possible new UL configurations to support “increasing UE power high limit” feature, the MSD framework is to follow the same principle of PC2 inter-band UL combinations with “increasing UE power high limit” feature.

Table 2-2 summarizes the proposed 2UL IMD MSD power compositions for the possible new UL configurations to support “increasing UE power high limit” feature.

| Power Class | UL Configuration | UL Power for 2UL IMD MSD |
| --- | --- | --- |
| PC2 | PC2 FDD + PC5 TDD | 20dBm + 20dBm |
| PC2 | PC2 FDD + PC3 TDD | 23dBm + 23dBm |
| PC2 | PC2 FDD + PC3 FDD | 23dBm + 23dBm |
| PC1.5 | PC1.5 TDD + PC5 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC3 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC3 FDD | 27.8dBm + 23dBm |
| PC1.5 | PC1.5 TDD + PC2 TDD | N/A |
| PC1.5 | PC1.5 TDD + PC2 FDD | 26dBm + 26dBm |

Table 2-2 Proposed 2UL IMD MSD power compositions for the possible new UL configurations

# 3	Conclusion

In this contribution, we share our further views on “increasing UE transmission high power limit for inter-band UL CA and EN-DC” and propose the MSD framework under this feature to follow the same principle of PC2 inter-band UL CA/EN-DC with higherPowerLimit-r17/ higherPowerLimitMRDC-r17 capability.

Observation 1: For UL configurations consist of at least one FDD band, it may be subject to 2UL IMD interference issue. If PCMAX for each constituent UL band would be applied to evaluate MSD requirements, new MSD framework would be needed for these UL configurations.

Observation 2: For PC2 with (PC3 FDD + PC2 TDD) and “increasing UE power high limit” feature, RAN4 has already agreed that the 2UL IMD MSD test configuration for the same PC2 combination without the “increasing UE power high limit” feature would be reused.

Proposal: For the possible new UL configurations to support “increasing UE power high limit” feature, the MSD framework is to follow the same principle of PC2 inter-band UL combinations with “increasing UE power high limit” feature.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2406583 “WF on HPUE UE for UL CA and EN-DC”, Samsung, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024

RP-212163 “New WID: Increasing UE power high limit for CA and DC”, China Telecom, Qualcomm, 3GPP TSG-RAN Meeting #93-e, September 13th – 17th, 2021