3GPP TSG-RAN4 Meeting #111   	R4-2407252

# Fukuoka City, Fukuoka, Japan, 20th - 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-5 on eNTN demod requirements for PDCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 has agreed to define requirements for PDCCH demod for FR2 NTN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced requirements for FR2 NTN for PDCCH demod |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no requirements for PDCCH demod for FR2 NTN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 11.2.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 11.2.3-1 are valid for all PDCCH tests unless otherwise stated.

Table 11.2.3-1: Common test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 160 |
|  | CSI-RS offset |  | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 8 CSI-RS resource 2: 9 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 2: The high layer parameter precoderGranularity equals to sameAsREG-bundle as defined in clause 7.4.1.3 of TS 38.211 [9] |  |  |  |  |

#### 11.2.3.1	1RX requirements

The parameters specified in Table 11.2.3.1-1 are valid for all PDCCH requirements unless otherwise stated.

Table 11.2.3.1-1: Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| CCE to REG mapping type |  | Interleaved |
| REG bundle size |  | 2 |
| Interleaver size |  | 3 |
| Shift index |  | 0 |

##### 11.2.3.1.1	Minimum requirements with 1Tx Antenna

For the parameters specified in Table 11.2.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 11.2.3.1.1-1. The downlink physical setup is in accordance with Annex C.5.1.

Table 11.2.3.1.1-1: Minimum performance requirements with 1 Tx Antenna

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | 200 | 132 | 1 | 8 | R.PDCCH.x1 | NTN-TDLC5-1200 | 1x1 Low | 1 | TBA |
| 1-2 | 200 | 132 | 2 | 16 | R.PDCCH.x2 | NTN-TDLC5-1200 | 1x1 Low | 1 | TBA |