3GPP TSG-RAN WG4 Meeting #111                                                                                                           R4-2407474

Fukuoka, JP, May 20th – May 24th, 2024

Source: 	CATT

Title: 	Discussion on SAN extreme testing conditions for NTN SAN

Agenda item:	7.16.4

Document for:	Approval

- Introduction

In RAN4#110-bis meeting, a WF [1] on SAN extreme testing conditions for SAN was agreed with remaining issues on SAN extreme testing condition, which require further discussion.

This contribution, we provide our views on SAN extreme testing conditions for NTN SAN.

- Discussion

SAN extreme testing conditions

As per the WF [1], the remaining open issue concerning SAN extreme testing conditions is shown as below:

Way forward 1: Consider the following options (with other not precluded) for SAN extreme testing conditions decision next meeting: - Option 1: Do not consider extreme conditions testing; - Option 2: Consider to introduce new dedicated set of optional manufacturer declarations in TS 38.181, table 4.6-1, and Annex B as below (Track Changes): Condition; Minimum; Maximum. Barometric pressure; D.106; D.107. Relative humidity; D.108; D.109. Vibration; D.110; D.111. Additional conditions; D.112; D.112. NOTE: Additional extreme test conditions might be decided outside 3GPP, according to the specific mission requirements, together with operator and manufacturer customizations.. Way forward 2: If Option 2 in Way Forward 1 is agreed, agree that extreme conditions testing for NTN SAN is a in band-agnostic topic, i.e. not to re-open this discussion for new NTN frequency range or band(s), e.g. Ku, etc.

Since there were no requirements for extreme conditions, and testability in extreme condition is also uncertain, for option 1 and option 2 in Way forward 1, so option 1(Do not consider extreme conditions testing)  in Way forward 1 is reasonable.  We prefer option 1 in Way forward1 and propose it. We don't need to define any requirements for extreme conditions.

Proposal 1: Option1 (Do not consider extreme conditions testing) in Way Forward 1 is agreed.

The Table B.3.1-1 of TS 38.181 for SAN extreme testing condition is copied as below:

Table B.3.1-1: Limits of conditions for extreme test environment for SPRF

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | Declared by manufacturer | Declared by manufacturer |
| Relative humidity | Declared by manufacturer | Declared by manufacturer |
| Vibration | Declared by manufacturer | Declared by manufacturer |
| Additional conditions | Declared by manufacturer | Declared by manufacturer |
| NOTE: Additional extreme test conditions might be decided outside 3GPP, according to the specific mission requirements, together with operator and manufacturer customizations. |  |  |

According to the Table B.3.1-1 of TS 38.181 as shown as above, since the declaration identifier is missing, it is reasonable to add declaration identifier for declaration.  If we want Option 2 in Way Forward 1, it is better to add declaration identifier using DE.x in Table B.3.1-1 in Annex of TS 38.181 instead of in Manufacturers declarations’ Table 4.6-1 of TS 38.181 as below. And a CR [2] is provided to add declaration identifier using DE.x in Table B.3.1-1 in Annex of TS 38.181

Proposal 2: If Option 2 in Way Forward 1 is agreed, add declaration identifier using DE.x in the appendix, not in the Manufacturers declarations’ Table 4.6-1.

From our understanding, it is reasonable that that extreme conditions testing for NTN SAN is band-agnostic.

- Conclusion

This contribution provides analysis on SAN extreme testing conditions for NTN SAN. The following proposals and observations are concluded as follows:

Proposal 1: Option1 (Do not consider extreme conditions testing) in Way Forward 1 is agreed.

Proposal 2: If Option 2 in Way Forward 1 is agreed, add declaration identifier using DE.x in the appendix, not in the Manufacturers declarations’ Table 4.6-1.

A CR [2] is provided to add declaration identifier using DE.x in Table B.3.1-1 in Annex of TS 38.181 for approval.

- References

- R4-2406002, Way Forward for [110bis][306] NR_NTN_enh_Part2 Extreme conditions and MU, Huawei,HiSilicon, RAN4#110bis

- R4-2407475 , (NR_NTN_enh-Perf)CR for TS 38.181, Add Declaration identifier for SAN extreme testing condition declaration, CATT, RAN4#111.