3GPP TSG-RAN WG4 Meeting # 111	R4-2407536

# Fukuoka, Japan,  May 20th - 24th , 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR 38101-1 Clarification of MIMO and TxD capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | R18_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the introductiion of 3Tx, an UL configuration supporting 3Tx operation may involve different power classes, TDD/FDD, UL-MIMO etc. Currently several notes are added to clarify these capabilities. However, these notes can be simplified based on the following facts: - For a band combination consisting of one FDD band and one TDD band, UL-MIMO mode is only applied to the TDD band, and the FDD band supports only PC3. - For a band combination consisting of two TDD bands, UL-MIMO mode can be applied to either of the TDD bands. - Power class for the TDD band with UL-MIMO mode can be either the band combination power class or lower: If the band combination power class is PC2, then the power class of the TDD band with UL-MIMO can be either PC3 or PC2. If the band combination power class is PC1.5, then the power class of the TDD band with UL-MIMO is PC1.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are made on top of the endorsed draft CR R4-2405180: - Revise Note 2/3/4 according to the above facts. - Remove Note x,y. - Remove all superscripts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | More concise and clear capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.3.1, 6.2L.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of Change 1/2>

#### 6.2H.3.1	UE maximum output power for inter-band UL CA with UL MIMO

For inter-band UL CA with UL MIMO in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2H.3.1-1. The period of measurement shall be at least one sub frame (1 ms). The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2 and 6.2D.1-3 for 2-layer configuration and ULFPTx configuration respectively for the component carrier configured with UL MIMO.

Table 6.2H.3.1-1: UE Power Class for inter-band UL CA with UL MIMO in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n5A-n77A | 29 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n8A-n78A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n41A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n77A | 29 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n26A-n78A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n41A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n66A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n71A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n77A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: For a band combination comprising one FDD band and one TDD band, UL-MIMO mode is exclusively applied to the TDD band. NOTE 3: For a band combination consisting of two TDD bands, UL-MIMO mode can be applied to either of the TDD bands. NOTE 4: For a PC2 band combination, the power class of the component TDD band in UL-MIMO mode can be either PC3 or PC2. For a PC1.5 band combination, the power class of the component TDD band in UL-MIMO mode is PC1.5. For a band combination comprising one FDD band and one TDD band, the FDD band supports only PC3. NOTE 5: Power class 3 is default power class unless otherwise stated. NOTE 6: FWA form factor is targeted unless otherwise stated. |  |  |  |  |  |  |

< End of Change 1/2>

< Start of Change 2/2>

#### 6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2L.3.1-1. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2L.3.1-1: UE Power Class for inter-band UL CA with Tx Diversity in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n5A-n77A | 29 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n8A-n78A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n41A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n77A | 29 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n26A-n78A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n41A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n66A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n71A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| CA_n41A-n77A |  |  | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: For a band combination comprising one FDD band and one TDD band, UL-MIMO mode is exclusively applied to the TDD band. NOTE 3: For a band combination consisting of two TDD bands, UL-MIMO mode can be applied to either of the TDD bands. NOTE 4: Power class 3 is the default power class unless otherwise stated. For a PC2 band combination, the power class of the component TDD band in UL-MIMO mode can be either PC3 or PC2. For a PC1.5 band combination, the power class of the component TDD band in UL-MIMO mode is PC1.5. For a band combination comprising one FDD band and one TDD band, the FDD band supports only PC3. NOTE 5: FWA form factor is targeted unless otherwise stated. |  |  |  |  |  |  |

< End of Change 2/2>