3GPP TSG-RAN WG4 Meeting #111	R4-2407582

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title:	UE RF specification technical corrections for CA and MIMO

Source:	Qualcomm Incorporated

Agenda item:	12.1.1.1

Release:	Rel-18

Document for:	Approval

# 1.	Introduction

RAN has enabled RAN4 to discuss specification improvement for the sake of future [1]. In this paper we discuss some of the language that is used in the current specification that causes confusion and problem and some other that may cause different interpretations.

It should be noted that the technical specifications are used by people that have no exposure to the underlying discussion creating those specifications and as such should be self explanatory. In many cases as we outline in this paper, the specification is understandable only with the presumption  that the reader has information from the process that resulted in to this language.

# 2. 	Discussion

## 2.1	Ambiguous wording on applicability of requirements

Continuing the discussion in [3] and focusing on CA sections, we discuss and propose possible corrections for applicability of the requirements. We discuss inter-band, intra-band contiguous and intra-band non-contiguous separately but common to all three cases, all requirements should follow the how the ambiguous wording will be corrected. Note that the discussion under Power_Limit_CA_DC is about power class applicability that is enclosed in this discussion.

### 2.1.1	Inter-band CA

Inter-band CA in LTE was defined with static power control i.e. the maximum power for each cell is limited by assuming maximum power on the other cell/band.

For NR, a dynamic power sharing was introduced where in the absence of UL grant on the other cell/band allows UE to use power up to the defined power class on the other band. This implies that the UE has knowledge of the each transmission occasions between the two bands. This is also how the Ran1 specification with the power control for example clause 7.5 in 38.213 is written, it refers to “respective transmission occasions” as shown below.

Ran4 specifications for inter-band carrier aggregation use a reference “assigned”. With this information, the inter-band CA requirements should be applicable when there are overlapping transmission occasions in two bands.

RAN4 specifications therefore are using a language that leave room for interpretation.

What may have been a source of confusion is that the inter-band CA band combination acronym is referred to as band configuration when exact number of component carriers are included in the acronym, as it is in the column title for Table 6.2A.1.3-1. However, this is just a name, in section 5 in Table 5.2A.2.1-1 inter-band CA combination is referred to as “NR CA band” without the letters and further in section 5.5 they are referred as configurations e.g. in Table 5.5A.3.1-1a.

Proposal 1: Update all uplink inter-band CA requirements to align with RAN1 specifications to apply when there are simultaneous transmission occasions on two bands.

### 2.1.2	Intra-band contiguous

Intra-band contiguous CA is mildly clearer situation. The MPR section refers to allocation which alone might imply that RAN4 intra-band CA requirement should apply only when both CCs are allocated, similarly to the inter-band CA.

However, further in the MPR section contiguous intra-band CA allocation is defined also when one CC has no allocation (LCRB,c=0) requirements:

RAN4 RF experts who followed the MPR derivation, may also remember the issue that MPR was developped for one DC location (carrier leakage) alone and 3GPP has no method to indicate a DC location based on allocation. Therefore it can be considered that the intra-band contiguos CA requirements apply when UE is configured and activated for two uplink CCs on same band that are withing nomilan channel spacing from each other.

Proposal 2: Intra-band contiguous CA requirements apply when UE is confgired/activated for two uplink CCs

### 2.1.3	Intra-band non-contiguous

Intra-band contiguos is slighlty more complicated that its contiguous counterpart. In MPR section for dualPA=1, there is a reference that when allocation is only on one CC, the MPR that applies is the single CC MPR.

This implies that the intra-band non-contiguos CA requirements in RAN4 specifications hould apply only when both CCs allocated. It is intuitive since the second carrier leakage that comes from second PA is not present in the antenna connector without allocated UL grant on that CC. However, for dualPA=0, meaning one DC location, the specification continues to specify new MPR greater than the one defined in the single CC MPR sections:

This then implies that requirements should apply based on configuration/activation.

Proposal 3: Intra-band non-contiguous CA requirements apply based on allocation when dualPA=1 and based on configuration/activation when dualPA=0.

In summary, if any of the proposals 1,2,3 are agreeable, Ran4 should proceed to discussing how the language in the specifications should be updated.

Proposal 4: RAN4 to update specification language according to proposal 1,2 and 3.

This language update would then also solve the problem of when which power class indication applies.

## 2.2	dual TX vs TXD vs PC1.5 vs 2Tx vs UL-MIMO

### 2.2.1	dualTX

Specification uses the terms in the heading but does not define what dualTX or 2Tx is. If reader was aware of discussions on the corresponding Wis, one maybe aware of the meaning but for a reader outside, these maybe unclear. The problem arises when a requirement applicability depends on these terms.

A UE with dual Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211].

In Table 6.2.1-1 note5 states for PC1.5

NOTE 5: Achieved via dual Tx

Which means TxD, in other word transmit antenna port virtualization.

In practice, dual TX means UE with two antenna connectors and TxD capability means it uses those for single port transmission but the “UE with two antenna connectors” in the above UL MIMO clause means UE that supports 2-layer (or 4-layer) UL MIMO and has two logical antenna port. The usage and relation of dual TX with the TxD should be clarified.

Proposal 5: DualTX reference in conjunction with PC1.5 should be updated to refer to TxD

Proposal 6: Dual TX in conjunction with UL MIMO should be updated to refer to the UE supporting two antenna ports.

For the proposal 6, the exact language should be studies, since in some cases this means UE support maxNumberSRS-Ports-PerResource >1 and in some cases it means maxNumberMIMO-LayersCB-PUSCH > 1 .

### 2.2.2	Spatial multiplexing scheme

For example in 6.2D.1:

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme,

Further, in 6.2D.2 same clause uses all the terms.

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2- 1for PC3, Table 6.2D.2-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2- TPMIGroup-r16 for the band and Table 6.2.2-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ulFullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 with dual Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4 Tx respectively.

It is not very clear what is a “UE in closed-loop spatial multiplexing scheme”. As per writers understanding, it is referring to UL MIMO, but under what circumstances the requirements in this case apply. How does UE enter it to the “scheme”? There is a set of capabilities and then a set of configurations and scheduling that happens.

It is common understand that when UE is configured with 2-logical TX ports but how does the configuration happen exactly should be described here so that the requirement is exactly defined.

Proposal 7: RAN4 to study how the UL MIMO section requirements should be explicitly defined to rely on terminology aligned with other work groups

# Conclusion

We discussed technical ambiguities in CA clauses and made proposals for clarifications:

Proposal 1: Update all uplink inter-band CA requirements to align with RAN1 specifications to apply when there are simultaneous transmission occasions on two bands.

Proposal 2: Intra-band contiguous CA requirements apply when UE is confgired/activated for two uplink CCs

Proposal 3: Intra-band non-contiguous CA requirements apply based on allocation when dualPA=1 and based on configuration/activation when dualPA=0.

Proposal 4: RAN4 to update specification language according to proposal 1,2 and 3.

We also discussed dualTX use and made proposals:

Proposal 5: DualTX reference in conjunction with PC1.5 should be updated to refer to TxD

Proposal 6: Dual TX in conjunction with UL MIMO should be updated to refer to the UE supporting two antenna ports.

And lastly we discussed “spatial multiplexing” and proposed

Proposal 7: RAN4 to study how the UL MIMO section requirements should be explicitly defined to rely on terminology aligned with other work groups

# Reference

[1]	RP-240782, “Moderator's summary on RAN4 spec quality”, RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

[2]	R4-2406709, “WF on UERF_Spec_Improvement” Qualcomm, 3GPP TSG-RAN WG4 Meeting #110Bis, Changsha, China, 15th – 19th April, 2024

[3]	R4-2405890, “UE RF specification improvement areas”, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, 15th – 19th April, 2024