3GPP TSG-RAN WG4 Meeting #111	R4-2407710

# Fukuoka, Japan, May 20-24, 2024

Source:	T-Mobile USA

Title:	TP for TR38.850: FDD PC2 for n25 with DL CA_n25A-n41A

Agenda item:	6.18.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 n25 for DL CA_n25A-n41A.

There is currently no MSD for PC3 CA_n25A-n41A but At RAN4#110 it was suggested that there might be UL4/DL3 Harmonic Mixing MSD and cross-band isolation MSD for n25 PC2. This issue was further discussed at RAN4#110bis. This contribution considers Harmonic Mixing MSD and cross-band isolation MSD for CA_n25A-n41A with UL n25 PC2.

- Discussion

In [1] we proposed that since there is usually MB-HB diplexing, and the filtering margin along with the PA TX filter response should be enough to mitigate the effect of power class 2 for UL4. Because there is no MSD for PC3, we proposed 0.1 dB MSD PC2. For cross-band isolation MSD, we assumed 0.2 dB MSD for PC3.

During RAN4#110bis [1] was postponed, awaiting additional input. This TP is a placeholder for agreement on the MSD at RAN4#111.

- Reference

[1] R4-2404382 “TP for TR38.850: FDD PC2 for n25 with DL CA_n25A-n41A,” T-Mobile USA

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n25A-n41A

5.x.1	UE maximum output power

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier10 |  | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A |  | n418,9 n25A8 CA_n25A-n41A8, 13,14 |  | n25 |  | 5, 10, 15, 20 |  | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |  |
|  |  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 |  | See n25 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

### 5.x.2	Harmonic Mixing Reference sensitivity requirements

#### 5.x.2.0	General

For PC3, CA_ n25A-n41A has no harmonic MSD for UL n25. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n25 without TxD

MSD for PC2 in n41 was discussed at RAN4#111 and the following was agreed to:

Table 7.3A.4-4a: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 10 | [TBD] | NOTE 11 | UL4/DL3 |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | [TBD] | NOTE 11 | UL4/DL3 |

#### 5.x.2.2	Harmonic Mixing reference sensitivity requirements with PC2 on n25 with TxD

[TBD].

### 5.x.3	Cross-band isolation reference sensitivity requirements

#### 5.x.3.0	General

For PC3, CA_ n25A-n41A has no cross band isolation MSD for UL n25. This section proposes MSD for PC2 FDD.

#### 5.x.3.1	Reference sensitivity requirements with PC2 on n25 without TxD

MSD for PC2 in n41 was discussed at RAN4#111 and the following was agreed to:

Table 7.3A.6-1a: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | [TBD] | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |

#### 5.x.3.2	Cross-band isolation reference sensitivity requirements with PC2 on n25 with TxD

[TBD]