3GPP TSG-RAN WG4 Meeting #111	R4-2407896

# Fukuoka, Japan, 20th - 24th, May, 2024

Title:	Discussion on XR UE OTA test

Source:	Samsung

Agenda item:	10.4.2.1

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, the LS [1, R4-2406085] on collaboration for XR head phantom was sent to CTIA and now CTIA is already working on it. The head phantom scenario was agreed as prioritized scenario and FFS free space scenario [2, R4-2406086]:

Issue 2-2-2: Phantom-based and Free space XR test scenarios Way forward: - Consider Head phantom scenario (1st priority) and FFS Free space for head-worn XR devices - For Free Space, the positioning guideline of XR device should be studied in RAN4. The corresponding XR UE mechanical mode description should also be defined

In this contribution, we share our views on the free space scenario for head-worn XR devices and some other open issues as well.

2. 	Discussion

For head-worn XR devices, the usage scenario should be the head phantom scenario. The free space test of head-worn XR devices may be needed in development stage, but should not be adopted in conformance testing. Take watch supporting cellular communication as comparison, there is only arm phantom test method and no free space test method for watch product in standard. Hear-worn XR as wearable devices, it should follow the practice of smart watch and only adopt the phantom based test in conformance testing.

RAN4 study on the free space testing of hear-worn XR devices should be focused on providing guidance to the industry rather than specifying test method for conformance.

Proposal 1:	free space testing of hear-worn XR devices is not specified as test method for conformance

It was also agreed to further discuss the XR device type for test method. As summarized in TR26.928, the form factors include various types as shown in following figure.

Figure 1: XR Form Factors (Figure 4.8-1 of TR26.928)

Generally speaking there are two categories in form factor, one is eyeglasses, and the other is helmet. It can also be noticed that for some XR form factors in above figure the “5G Technology” is located outside of XR devices externally, e.g. XR5G-V1/V2/V3 and XR5G-A1/A2/A5, also indicated in the table below.

Table 1: XR Device Types (Table 4.8-1 of TR26.928)

| XR Type Number | XR Device Type Name | Tethering Examples | 5G Uu Modem | XR Engine Localization | Power Supply | Typical Max Avail Power |
| --- | --- | --- | --- | --- | --- | --- |
| XR5G-P1 | Phone | n/a | XR device | XR device or split | Internal | 3-5 W |
| XR5G-V1 | Simple VR Viewer wired tethering | USB-C | External | External | External | 2-5 W |
| XR5G-V2 | Simple VR Viewer wireless tethering | 802.11ad/y, 5G sidelink, etc. | External | External | Internal | 2-3 W |
| XR5G-V3 | Smart VR Viewer wireless tethering | 802.11ad/y, 5G sidelink, etc. | External | XR device or Split | Internal | 2-3 W |
| XR5G-V4 | VR HMD Standalone | n/a | XR device | XR device or Split | Internal | 3-7 W |
| XR5G-A1 | Simple AR Wearable Glass wired tethering | USB-C | External | External | External | 1-3 W |
| XR5G-A2 | Simple AR Wearable Glass wireless tethering | 802.11ad/y, 5G sidelink. etc. | External | External | Internal | 0.5 – 2 W |
| XR5G-A3 | Smart AR HMD see-through standalone | n/a | XR device | XR device or Split | Internal | 3-7 W |
| XR5G-A4 | AR Wearable Glass standalone | n/a | XR device | XR device or Split | Internal | 2 - 4 W |
| XR5G-A5 | Smart AR Wearable Glass wireless tethering | 802.11ad/y, 5G sidelink. etc. | External | XR device or Split | Internal | 0.5 – 2 W |

It is questionable to test the NR OTA performance of a XR devices without 5G inside. It is proposed to discuss if head-worn XR devices with external 5G are in scope or not.

Proposal 2:	RAN4 to discuss if head-worn XR devices with external 5G (e.g. XR5G-V1/V2/V3, XR5G-A1/A2/A5) are in scope or not.

3. 	Conclusion

Proposal 1:	free space testing of hear-worn XR devices is not specified as test method for conformance

Proposal 2:	RAN4 to discuss if head-worn XR devices with external 5G (e.g. XR5G-V1/V2/V3, XR5G-A1/A2/A5) are in scope or not.

4. 	References

- R4-2406085    “LS on Head Phantoms for XR devices OTA testing”, 3GPP RAN4

- R4-2406086    “Way Forward for [110bis][338] TRP_TRS_MIMO_OTA”, vivo, CAICT