3GPP TSG-RAN WG4 Meeting # 111                                                                 R4-2407951

Fukuoka, Japan, 20 May – 24 May, 2024

Agenda item:	10.8.2.1

Source: 	CMCC

Title: 	(NR_ATG_enh-Core) Discussion on UE RF requirements for ATG with intra-band contiguous CA

Document for:	Discussion

# 1. Introduction

RAN#103 meeting approved New WID on Enhancements for Air-to-ground network for NR in Rel-19. RF core requirements for intra-band co-located and inter-band co-located DL CA shall be specified in this WID. In RAN4#110bis meeting, WF [1] was approved. In this contribution, we’d like to provide our views on UE RF requirements for ATG with intra-band contiguous CA.

# 2. Discussion

## 2.1. BCS for CA_n79C

To ensure the diversity of ATG CA BCS as much as possible, the BCS in TS 38.101-1 for TN UE could be reused for ATG CA_n79C.

Proposal 1: Use the following BCS for CA_n79C.

Table 1: BCS for CA_n79C

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 and 5 |

## 2.2. Rx requirements for intra-band CA

Last meeting WF listed the Rx requirements that may be affected by ATG with intra-band CA. The following table shows our analysis.

Table 2. Summary of the proposals for ATG UE requirements with DL intra-band CA

| Requirement | Applicability |
| --- | --- |
| Reference sensitivity | During last meeting we have reached the agreement that - Only consider contiguous DL CA case for ATG UE intra-band CA. - The example band combination for intra-band contiguous CA is CA_n79C. REFSENS requirements for TN UE is reused for ATG UE without CA. Since the agreement that we only consider contiguous intra-band CA, the principle in TS 38.101-1 sub-clause 7.3A.2.1 for TDD band could be reused. Proposal 2: Requirements of TDD contiguous intra-band CA in TS 38.101-1 sub-clause 7.3A.2.1 could be reused for ATG UE with intra-band CA. |
| Maximum input level | Max input level for ATG UE had been discussion in R18 ATG, and the requirements had been specified in TS 38.101-1 sub-clause 7.4J-1. Since there are two antenna types for ATG UE, the max input level for ATG UE with intra-band CA could be specified in table 3, and the principle in TS 38.101-1 sub-clause 7.4A.1 could be reused. Table 3: Maximum input level for ATG with CA_n79C Rx Parameter; Units; ATG UE Types. Omni-directional antenna; Antenna array. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; -422; -302. -443; -323. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM.. Proposal 3: The max input level requirements for ATG UE with intra-band CA could be specified in table 3, and the principle in TS 38.101-1 sub-clause 7.4A.1 could be reused. |
| ACS | ACS requirements for ATG UE without CA had been specified in TS 38.101-1 sub-clause 7.5J. Since the example band combination is CA_n79C, we only consider case that frequency range larger than 3300MHz. The ACS value in clause 7.5A.1 could be reused or specified as the following tables. Table 4: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz NR CA bandwidth class. Rx Parameter; Units; C. ACS; dB; 33.0. Table 5: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1 Rx Parameter; Units; NR CA bandwidth class. C. Pw in Transmission Bandwidth Configuration, per CC; dBm; REFSENS + 14 dB. PInterferer; dBm; Aggregated power + 31.5 dB. BWInterferer; MHz; BWchannel CA. FInterferer (offset); MHz; BWchannel CA / -BWchannel CA. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1.. Test parameters case 2 in clause 7.5A.1 should to be updated. Table 6: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2 Rx Parameter; Units; NR CA bandwidth class. C. Pw in Transmission Bandwidth Configuration, per CC; dBm; -73.54 -61.55. PInterferer; dBm; -424 -305. BWInterferer; MHz; BWchannel CA. FInterferer (offset); MHz; BWchannel CA / -BWchannel CA. NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array.. Proposal 4: The ACS principle and requirements for in clause 7.5A.1 for intra-band contiguous CA could be reused expect for the test parameters case 2, and case 2 could be defined as Table 6. |
| In-band blocking | In-band blocking requirement requirements of TN UE are reused for single ATG UE without CA. Since there is no obvious effect for ATG UE with intra-band CA, the requirements specified in clause 7.6A.2.1 could be reused for ATG UE with intra-band contiguous CA Proposal 5: The requirements specified in clause 7.6A.2.1 could be reused for ATG UE with intra-band contiguous CA. |
| Out-of-band blocking | Similar to in-band blocking. Proposal 6: The requirements specified in clause 7.6A.3.1 could be reused for ATG UE with intra-band contiguous CA |
| Narrow band blocking | Proposal 7: There is no narrow band blocking requirement for ATG UE. |
| Spurious response | Since spurious response requirement requirements of TN UE are reused for single ATG UE without CA, the requirements specified in clause 7.7A.1 could be reused for ATG UE with intra-band contiguous CA Proposal 8: The requirements specified in clause 7.7A.1 could be reused for ATG UE with intra-band contiguous CA. |
| Intermodulation | Proposal 9: The intermodulation requirements specified in clause 7.8A.2.1 could be reused for ATG UE with intra-band contiguous CA. |

# 3. Conclusions

In this contribution, ATG UE with intra-band CA requirements are discussed with the following proposals:

Proposal 1: Use the following BCS for CA_n79C.

Table 1: BCS for CA_n79C

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 and 5 |

Proposal 2: Requirements of TDD contiguous intra-band CA in TS 38.101-1 sub-clause 7.3A.2.1 could be reused for ATG UE with intra-band CA.

Proposal 3: The max input level requirements for ATG UE with intra-band CA could be specified in table 3, and the principle in TS 38.101-1 sub-clause 7.4A.1 could be reused.

Proposal 4: The ACS principle and requirements for in clause 7.5A.1 for intra-band contiguous CA could be reused expect for the test parameters case 2, and case 2 could be defined as Table 6.

Proposal 5: The requirements specified in clause 7.6A.2.1 could be reused for ATG UE with intra-band contiguous CA.

Proposal 6: The requirements specified in clause 7.6A.3.1 could be reused for ATG UE with intra-band contiguous CA.

Proposal 7: There is no narrow band blocking requirement for ATG UE.

Proposal 8: The requirements specified in clause 7.7A.1 could be reused for ATG UE with intra-band contiguous CA.

Proposal 9: The intermodulation requirements specified in clause 7.8A.2.1 could be reused for ATG UE with intra-band contiguous CA.

# 4. References

[1] R4-2406594, WF on Rel-19 ATG UE requirements, CMCC