3GPP TSG-RAN WG4 Meeting #111	R4-2408496

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source:	Samsung

Title:	discussion on advanced receiver test parameters for MU-MIMO

Agenda item:	7.11.2.2

Document for:	Discussion

Introduction

In last RAN4 #110bis meeting, the open issues of the test setting for when UE is indicated Modulation order scenario are solved. Meanwhile some open issues of the test setting for when UE is not indicated Modulation order scenario still need further discussion [1]. In this contribution, we share our views on open issues of MU-MIMO Phase II test parameters and UE capability as below.

Test setting for when UE is indicated Modulation order (DCI index 1-5 is indicated) - Agreement: - For Rank 1+1 with 2T2R - Case2: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE - For Rank 2+2 with 4T4R: - Case7: Orthogonal precoding, TDLA30-10, ULA Low, MCS 17 (Table 1) for Target UE, 16QAM for co-UE. full FDRA for the co-UE - For Rank 1+1 with 2T4R: - Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE Test setting for when UE is not indicated Modulation order (DCI index 6 is indicated) - For Rank 1+1 with 2T2R, down-select among the following cases: - Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Case26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE - Case 20: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE (as priority for requirement definition) - Companies are encouraged to bring simulation results for all cases above - For Rank 2+2 with 4T4R: - Option 1: Introduce rank 2+2 4T4R requirements with modulation order blind detection - Option 1A (Case 32): Orthogonal precoding, TDLA30-10, XP medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Option 1B (Case 31): Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Option 2: Do not introduce rank 2+2 4T4R requirements with modulation order blind detection - For Rank 1+1 with 2T4R, if introduced, down-select among the following test cases: - Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Case 29: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE - Case 23: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE - Companies are encouraged to bring simulation results for all cases above Whether to tests UE not support BD-MO with R-ML with DCI index 6 is indicated - Agreement: - Do not introduce such test for UE not support BD-MO with R-ML Whether new R-ML requirements should be captured in new clauses or in the existing clauses for MMSE-IRC under intra-cell inter-user scenario - Agreement: - R-ML new requirements are introduced in the existing clauses for MMSE-IRC under intra-cell inter-user scenario. - R-ML receiver not intended to be release independent. - Update 38.307 Rel-17 and Rel-18 in Table B.3.3-1 state which tests are release independent from Rel-15. RRC assistant information configuration on the MCS table - For UEs not supporting modulation order blind detection: - Option 1: No need for the network to inform such information to the UE - Option 2: Signalled regardless of whether the UE supports MO BD - Option 2A: 256QAM MCS Table - Option 2B: 64QAM MCS table - For UEs supporting modulation order blind detection: - Agreement: - 256QAM MCS Table DCI assistant signalling configuration for tests with modulation order blind detection - Agreement: - For Rank 1+1: - DCI signalling should be index 6. - For Rank 2+2 (if introduced): - DCI signalling should be index 6. For UE supporting MO BD, whether to introduce applicability rule to skip test(s) with modulation order indicated - Candidate options: - Option 1: Introduce applicability rule to skip tests with modulation order indicated for UEs capable of BD MO - Option 2: Do not introduce applicable rule skip tests with modulation order indicated Updates to UE capability definition - Agreement: - For 36-1, update the note in capability granularity column as below: - UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA, NE-DC, EN-DC and NR-DC operation - Update the ‘Components’ column as below: - For 36-1: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212. - For 36-2a: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled. - For 36-2b: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 is signalled. - Proposals on 36-2a and 36-2b: - Proposal 1: Combine 36-2a and 36-2b and remove number of layer descriptions if RAN4 agrees to not define 2+2 test under DCI 6 Proposal 2: Remove FG 36-2b and keep 36-2a from UE feature list

Discussion

Test setting for when UE is not indicated Modulation order (DCI index 6 is indicated)

Rank 1+1 with 2T2R

The way forward in RAN4#110bis meeting for test assumptions of Rank 1+1 with 2T2R scenario: down-select among the following cases:

  - Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Case26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE

  - Case 20: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE (as priority for requirement definition)

The difference between Case#20 and Case#26 is the MCS level for Target UE and modulation order for co-UE, i.e. MCS17 vs MCS13, and 16QAM vs QPSK. From real network point of view, Case#26 is more reasonable as MU-MIMO usually prefer to pair high SNR UEs. At the same time, form the simulation results [2] and the updated simulation results in RAN4#110bis, the R-ML receiver gain of Case#26 is obvious enough for test and stable with 1.8dB span.

Proposal 1: For Rank 1+1 with 2T2R, prefer to use Case#26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE as the test assumptions.

Rank 2+2 with 4T4R

For Rank 2+2 with 4T4R:

  - Option 1: Introduce rank 2+2 4T4R requirements with modulation order blind detection

    - Option 1A (Case 32): Orthogonal precoding, TDLA30-10, XP medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

    - Option 1B (Case 31): Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE

  - Option 2: Do not introduce rank 2+2 4T4R requirements with modulation order blind detection

The only difference between Case#32 and Case#31 is the antenna correlation. Form the simulation results [2] and the updated simulation results in RAN4#110bis, the R-ML receiver gain of Case#32 is a bit better than Case#31 but with larger span. Therefore, if introduce rank 2+2 4T4R requirements we prefer to use Case#31.

Proposal 2: For Rank 2+2 with 4T4R, prefer to use Case#31: Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE as the test assumptions.

Rank 1+1 with 2T4R

Similar reason as above, for Rank 1+1 with 2T4R, if test requirements are introduced, we prefer to use high MCS level.

Proposal 3: For Rank 1+1 with 2T4R, if test requirements are introduced, prefer to select	Case#29: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE as the test assumptions.

RRC assistant information configuration on the MCS table

In RAN4#108bis meeting, about the RRC assistant information configuration on the MCS table we has below agreement:

Dedicated RRC signalling is provided to the UE (target UE) to indicate the information in each of the following bullets separately, when the information is available: …… - The MCS table with the highest modulation order among all MCS tables configured to the co-scheduled UE(s), which has the same DM-RS sequence as the target UE. The MCS table is one of the following: - 1024QAM MCS table(s) (Table 5.1.3.1-4 from TS38.214) - 256QAM MCS table(s) (Table 5.1.3.1-2 from TS38.214) - 64QAM MCS tables (Table 5.1.3.1-1 or 5.1.3.1-3 from TS38.214) ……

Therefore, the information of MCS table co-scheduled UEs used will be informed to the target UE. Obviously, if the target UE doesn’t have the capability of modulation order blind detection, this dedicated RRC signalling about MCS table is useless even it is transmitted to the target UE. Hence for UEs not supporting modulation order blind detection, no need for the network to inform MCS table information to the UE.

Proposal 4: For UEs not supporting modulation order blind detection, no need for the network to inform MCS table information to the UE.

For UE supporting MO BD, whether to introduce applicability rule to skip test(s) with modulation order indicated

From the simulation results summary [2] and the updated simulation results in RAN4#110bis, the SINR difference @70% of maximum throughput between these two cases is more than 1dB, and the cases with modulation order indicated has better performance. It seems it is not easy to promise UEs could pass test case(s) with MO indicated, although it could passed test case(s) of BD MO. However, considering the test work load, we think introducing applicability rule to skip tests with modulation order indicated for UEs capable of BD MO is acceptable.

Proposal 5: For UE supporting MO BD, introducing applicability rule to skip tests with modulation order indicated for UEs capable of BD MO is acceptable for us.

Updates to UE capability definition

UE capability 36-1 is defined for DCI index 1-5 which has modulation order information, while UE capability 36-2a and 36-2b are defined for DCI index 6 or 7 which without modulation order information. Since we have defined DCI index 0-7 for MU-MIMO advanced receiver, related capability information should be defined for each DCI index. The only difference between 36-2a and 36-2b is the maximum layers across target UE and co-scheduled UEs. Even rank 2+2 cases under DCI index 6 will not be defined in RAN4 demodulation requirements, it’s not reasonable to remove 36-2b considering the completeness of the feature definition. However, combine 36-2a and 36-2b as similar as 36-1 is acceptable.

Proposal 6: Keep 36-2a and 36-2b, or combine them as similar as 36-1 like “R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.”

Conclusion

In this contribution, we provide analysis and views on remaining open issues for MU-MIMO test parameters, the proposals could be summarized as

Proposal 1: For Rank 1+1 with 2T2R, prefer to use Case#26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE as the test assumptions.

Proposal 2: For Rank 2+2 with 4T4R, prefer to use Case#31: Orthogonal precoding, TDLA30-10, ULA Low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE as the test assumptions.

Proposal 3: For Rank 1+1 with 2T4R, if test requirements are introduced, prefer to select	Case#29: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE as the test assumptions.

Proposal 4: For UEs not supporting modulation order blind detection, no need for the network to inform MCS table information to the UE.

Proposal 5: For UE supporting MO BD, introducing applicability rule to skip tests with modulation order indicated for UEs capable of BD MO is acceptable for us.

Proposal 6: Keep 36-2a and 36-2b, or combine them as similar as 36-1 like “R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.”

Reference

[1] R4-2406114, “WF on the advanced receiver for MU-MIMO scenario”, China Telecom, 3GPP RAN4 #110bis meeting

[2] R4-2400805, “phase 2 result collection for MU-MIMO”, 3GPP RAN4 #110 meeting