3GPP TSG-RAN WG4 Meeting #111				                             				     R4-2408829

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.786 | CR | draftCR | rev | <Rev#> | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TR 38.786 for Rel-18 A-MPR simulation results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
|  |  |  |  |  |  |  |  |  |  | 0 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the simulation result for NS_29 and NS_54. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The simulation result for NS_29 and NS_54 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR to NS_29 and NS_54 is not defined properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.1, 6.1.3.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | . |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<start of change>>

6.1.3.2	A-MPR for SL-U with NS_29

6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.2.1.1	LG Electronics’ simulation results (R4-2404862)

<Unchanged parts omitted>

6.1.3.2.1.2	OPPO’ simulation results (R4-2408830)

For NS_29, the emission requirement and PSD requirement differ with different channel bandwidth and hence simulation are down with different channel bandwidth. The simulation result is shown below:

Table 1 Simulation results for 20 and 40MHz

|  | 20MHz channel bandwidth |  |  |  | 40MHz channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 kHz contiguous | 30 kHz contiguous | 15 kHz Interlaced | 30 kHz Interlaced | 15 kHz contiguous | 30 kHz contiguous | 15 kHz Interlaced | 30 kHz Interlaced |
| QPSK | 4.03 | 4.11 | 4.19 | 4.28 | 3.22 | 3.22 | 3.14 | 3.38 |
| 16QAM | 4.03 | 4.03 | 4.20 | 4.28 | 3.70 | 3.70 | 3.54 | 3.38 |
| 64QAM | 4.97 | 5.23 | 4.37 | 4.28 | 5.05 | 4.97 | 5.05 | 4.11 |
| 256QAM | 8.80 | 8.61 | 7.86 | 7.49 | 8.61 | 8.80 | 7.40 | 6.48 |

Table 2 Simulation results for 600 and 800MHz

|  |  | contiguous |  | Interlaced |  | Contiguous Bitmap |  |  |  |  |  |  |  | Interlaced Bitmap |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60MHz | 80 MHz | 60 | 80 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 |
| Contiguous | QPSK | 3.2 | 3.2 | 3.2 | 3.3 | 3.5 | 2.8 | 3.6 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 5.9 | 3.0 | 6.0 | 2.9 | 5.9 | 2.6 | 5.9 | 2.9 |
|  | 16QAM | 3.7 | 3.7 | 3.2 | 3.3 | 3.5 | 3.6 | 3.6 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 |
|  | 64QAM | 5.2 | 5.1 | 4.1 | 3.9 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 6.0 | 4.2 | 6.0 | 4.2 | 5.9 | 4.1 | 5.9 | 4.2 |
|  | 256QAM | 8.6 | 8.5 | 6.6 | 6.5 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 |

<Unchanged parts omitted>

6.1.3.6	A-MPR for SL-U with NS_54

6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.6.1.1	LG Electronics’ simulation results (R4-2404862)

<Unchanged parts omitted>

6.1.3.6.1.2	OPPO’ simulation results (R4-2408830)

For NS_54, the band edge spectrum emission limit -27 dBm/MHz is the dominant factor and hence the band edge will suffer lager A-MPR while for the channels in the middle of the band can have smaller A-MPR. The requirement is shown as below.

Table 3 Spectrum emission limit for NS_54

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

For the A-MPR simulation result, they are captured below for edge channel and non-edge channel. For QPSK and 16QAM single CC with smaller bandwidth, the edge effect is more obvious. For higher order modulation as 64QAM and 256QAM, the dominant factor is more like EVM and hence no big difference found for edge and non-edge channel.

Table 4 A-MPR for single CC NS_54 edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.4 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

Table 5 A-MPR for single CC NS_54 non-edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 |

The wide-band operation simulation result is further provided below:

Table 6 A-MPR for Wideband operation NS_54 edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 2.8 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Table 7  A-MPR for Wideband operation NS_54 non-edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 |

<end of change>