3GPP TSG-RAN WG4 Meeting #111	R4-2408975

# Fukuoka, Japan, 20 - 24 May, 2024

Title: 	Discussion on UE demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.8.2

Document for:	Discussion

Background

During RAN4#110b meeting, WF [1] on NR_NTN_enh_SAN_UE_demod was approved. In this contribution, we share our views on UE demodulation requirements for NR NTN enhancements.

Discussion

PDSCH demodulation requirements for above 10 GHz bands

In previous meeting, we have already achieved the agreement that PN impact is not considered in ideal simulation results alignment.

- Agreements - Do not consider PN impact in ideal simulation results alignment. The phase noise impact can be considered in impairment results.

Also considering the carrier frequency is up to 20GHz and the maximum modulation order is 16QAM, the phase noise impact is negligible. Therefore, we propose to not configure PTRS for all PDSCH demodulation requirements for above 10 GHz bands.

Do not configure PTRS for all PDSCH demodulation requirements for above 10 GHz bands.

PDCCH demodulation requirements for above 10 GHz bands

- Agreement - AL8 and AL16, with the following test configurations Case; Bandwidth (MHz); SCS (kHz); CORESET RB; CORESET duration; Aggregation level; CCE to REG mapping type; REG bundle size; Interleaver size; Shift index; DCI format; Payload (without CRC). 1; 200; 120; 60; 1; 8; Interleaved; 2; 3; 0; [1-0]; [40]. 2; 200; 120; 60; 2; 16; Interleaved; 2; 3; 0; [1-1]; [56].

Based on the offline discussion, considering 200MHz channel bandwidth is used, we confirm to use CORESET RB = 132 and CORESET duration = 1 for PDCCH demodulation requirements for above 10 GHz bands. For the case 1 with the DCI format 1_0, we confirm to use the payload size = 42.

Use CORESET RB = 132 and CORESET duration = 1 for PDCCH demodulation requirements for above 10 GHz bands.

For the case 1 with the DCI format 1_0, use the payload size = 42.

Proposals

In this contribution, we discuss on UE demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- Do not configure PTRS for all PDSCH demodulation requirements for above 10 GHz bands.

Use CORESET RB = 132 and CORESET duration = 1 for PDCCH demodulation requirements for above 10 GHz bands.

For the case 1 with the DCI format 1_0, use the payload size = 42.

Reference

R4-2406024, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110b, Huawei, HiSilicon