3GPP TSG-RAN WG4 Meeting # 111	R4-2409008

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on remaining issues for 8Rx requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.3.1

Document for:	Discussion

Background

In last meeting, bigCR[1] was agreed, but there are still some issues found during the email discussion. In this contribution we provide our views on these issues.

Discussions

For R4-2406059 (draftCR for 38.101-4 Introduction of 8Rx Applicability Rule for single carrier), the applicability rules for different number of Rx antenna ports and UE receiver types are captured in same clause (clause 5.1.1.2 Applicability of requirements for different number of Rx antenna ports). The problem is that there are two dedicate clauses defined for these two applicability rules, so they should be captured in specific clauses rather than same clause.  Below is my suggestion:

#### 5.1.1.2 Applicability of requirements for different number of RX antenna ports The number of RX antenna ports for different RF operating bands is up to UE declaration. The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1. Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list; Exceptions. UE supports only 2RX; PDSCH; All tests in Clause 5.2.2. PDCCH; All tests in Clause 5.3.2. PBCH; All tests in Clause 5.4.2. UE supports only 4RX or both 2RX and 4RX; PDSCH; All tests in Clause 5.2.3 (Note 2). PDCCH; All tests in Clause 5.3.3 (Note 2). PBCH; All tests in Clause 5.4.2 or 5.4.3 (Note 1). UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3.(Note 2, 3) Tests 1-1, 3-1, 5-1 in Clause 5.2.4.1 and tests 1-1, 3-1, 5-1 in Clause 5.2.4.2(Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3.(Note 2,3). UE supports only 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3(Note 1). UE supports only 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2.(Note 2, 4) Tests 1-1, 3-1, 5-1 in Clause 5.2.4.1 and tests 1-1, 3-1, 5-1 in Clause 5.2.4.2(Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2.(Note 2,4). PBCH; All tests in Clause 5.4.2. UE supports only 8Rx; PDSCH; Tests 1-1, 3-1, 5-1 in Clause 5.2.4.1 and tests 1-1, 3-1, 5-1 in Clause 5.2.4.2(Note 2). PDCCH; N/A. PBCH; N/A. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3.(Note 2, 3) Tests 2-1, 4-1 in both Clause 5.2.4.1 and tests 2-1, 4-1 in Clause 5.2.4.2(Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3.(Note 2,3). UE supports only 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3(Note 1). UE supports only 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2.(Note 2, 4) Tests 2-1, 4-1 in both Clause 5.2.4.1 and tests 2-1, 4-1 in Clause 5.2.4.2(Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2.(Note 2,4). PBCH; All tests in Clause 5.4.2. UE supports only 8Rx; PDSCH; Tests 2-1, 4-1 in both Clause 5.2.4.1 and tests 2-1, 4-1 in Clause 5.2.4.2(Note 2). PDCCH; N/A. PBCH; N/A. Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied.. #### 5.1.1.3 Applicability of requirements for optional UE features The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only. Table 5.1.1.3-1: Requirements applicability for optional UE features UE feature/capability [14]; Test type; Test list; Applicability notes. SU-MIMO Interference Mitigation advanced receiver; FR1 FDD; PDSCH; Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1). FR1 TDD; PDSCH; Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1). Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt); FR1 FDD; PDSCH; Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2). FR1 TDD; PDSCH; Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2). Basic DL NR-NR CA operation (supportedBandCombinationList); NR CA; SDR; Clause 5.5A.1; 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time. Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h; FR1 FDD; PDSCH; Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1). FR1 TDD; PDSCH; Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1). Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt); FR1 FDD; PDSCH; Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6. FR1 TDD; PDSCH; Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6. CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt); FR1 FDD; PDSCH; Clause 5.2.2.1.5 Clause 5.2.3.1.5. FR1 TDD; PDSCH; Clause 5.2.2.2.5 Clause 5.2.3.2.5. PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots); FR1 FDD; PDSCH; Clause 5.2.2.1.6 Clause 5.2.3.1.6. FR1 TDD; PDSCH; Clause 5.2.2.2.6 Clause 5.2.3.2.6. UE PDSCH processing capability #2 (pdsch-ProcessingType2); FR1 FDD; PDSCH; Clause 5.2.2.1.7 Clause 5.2.3.1.7. FR1 TDD; PDSCH; Clause 5.2.2.2.7 Clause 5.2.3.2.7. Pre-emption indication for DL (pre-EmptIndication-DL); FR1 FDD; PDSCH; Clause 5.2.2.1.8 Clause 5.2.3.1.8. FR1 TDD; PDSCH; Clause 5.2.2.2.8 Clause 5.2.3.2.8. Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16); FR1 FDD; PDSCH; Clause 5.2.2.1.11 Clause 5.2.3.1.11. FR1 TDD; PDSCH; Clause 5.2.2.2.11 Clause 5.2.3.2.11. Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16); FR1 FDD; PDSCH; Clause 5.2.2.1.12 Clause 5.2.3.1.12. FR1 TDD; PDSCH; Clause 5.2.2.2.12 Clause 5.2.3.2.12. Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16); FR1 FDD; PDSCH; Clause 5.2.2.1.13 Clause 5.2.3.1.13. FR1 TDD; PDSCH; Clause 5.2.2.2.13 Clause 5.2.3.2.13. Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16); FR1 FDD; PDSCH; Clause 5.2.2.1.14 Clause 5.2.3.1.14. FR1 TDD; PDSCH; Clause 5.2.2.2.14 Clause 5.2.3.2.14. Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16); FR1 FDD; PDSCH; Clause 5.2.2.1.14 Clause 5.2.3.1.14; The requirements apply only when maxNumberTCI-states-r16 = 2.. FR1 TDD; PDSCH; Clause 5.2.2.2.14 Clause 5.2.3.2.14. DRX Adaptation (drx-Adaptation-r16); FR1 FDD; PDCCH; Clause 5.3.2.1.3; If the Test 1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.2.1.1.. FR1 TDD; PDCCH; Clause 5.3.2.2.3; If the Test 1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.2.2.1.. FR1 FDD; PDCCH; Clause 5.3.3.1.3; If the Test 1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.3.1.1.. FR1 TDD; PDCCH; Clause 5.3.3.2.3; If the Test 1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.3.2.1.. Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16); FR1 TDD; PDSCH; Clause 5.2.2.2.15 Clause 5.2.3.2.15 Clause 5.2A.2.3 Clause 5.2A.3.3; The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI. Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both; FR1 TDD; PDSCH; Clause 5.2.2.2.15 Clause 5.2.3.2.15; The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode. 1024QAM modulation for PDSCH for FR1 (pdsch-1024QAM-FR1-r17 or pdsch-1024QAM-2MIMO-FR1-r17); FR1 FDD; PDSCH; Clause 5.2.2.1.1 (Test 1-8) Clause 5.2.3.1.1 (Test 1-8). FR1 TDD; PDSCH; Clause 5.2.2.2.1 (Test 1-12) Clause 5.2.3.2.1 (Test 1-12). SDR; Clause 5.5.1 Clause 5.5A.1; 1024QAM MCS indexes are used only if UE supports 1024QAM for FR1 DL.. Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17); FR1 FDD; PDSCH; Clause 5.2.2.1.18 Clause 5.2.3.1.17; UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band.. FR1 TDD; PDSCH; Clause 5.2.2.2.19 Clause 5.2.3.2.18. Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17); FR1 FDD; PDSCH; Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1); The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position.. FR1 TDD; PDSCH; Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1). Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17); FR1 FDD; PDSCH; Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1); If the Test 2-1 in Clause 5.2.2.1.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.1.19. If the Test 2-1 in Clause 5.2.3.1.18 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.1.18.. FR1 TDD; PDSCH; Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1); If the Test 2-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.2.20. If the Test 2-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.2.19.. CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17); FR1 TDD; PDSCH; Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2); The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position.. CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17); FR1 TDD; PDSCH; Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2); If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19.. Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17); FR1 FDD; PDSCH; Clause 5.2.2.1.20 Clause 5.2.3.1.19. FR1 TDD; PDSCH; Clause 5.2.2.2.21 Clause 5.2.3.2.20. Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17); FR1 FDD; PDSCH; Clause 5.2.2.1.21 Clause 5.2.3.1.20. FR1 TDD; PDSCH; Clause 5.2.2.2.22 Clause 5.2.3.2.21. Support for PDCCH with intra-slot repetition (mTRP-PDCCH-Repetition-r17); FR1 FDD; PDCCH; Clause 5.3.2.1.5 Clause 5.3.3.1.4. FR1 TDD; PDCCH; Clause 5.3.2.2.5 Clause 5.3.3.2.4. Support for TDD-TDD intra-band non-colocated NR-CA deployment (intraBandNR-CA-non-collocated-r18); FR1 TDD; PDSCH; Clause 5.2A.2.6; The requirements apply on in case the UE indicates support of 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1). Baseline SU-MIMO 8Rx receiver; FR1 FDD; PDSCH; Clause 5.2.4.1.1(Tests 1-1,3-1). FR1 TDD; PDSCH; Clause 5.2.4.2.1(Tests 1-1,3-1). Simplified SU-MIMO 8Rx receiver; FR1 FDD; PDSCH; Clause 5.2.4.1.1(Tests 2-1,4-1). FR1 TDD; PDSCH; Clause 5.2.4.2.1(Tests 2-1,4-1).

Proposal 1: A new CR for introduction of 8Rx Applicability Rule for single carrier is needed to capture all the changes above.

New CRs related to single carrier and CA performance requirements need to be requested to capture updated simulation results if necessary.

Proposal 2: New CRs related to single carrier and CA performance requirements need to be requested to capture updated simulation results if necessary.

Conclusion

Proposal 1: A new CR for introduction of 8Rx Applicability Rule for single carrier is needed to capture all the changes above.

Proposal 2: New CRs related to single carrier and CA performance requirements need to be requested to capture updated simulation results if necessary.

Reference

[1]  R4-2405154 Draft BigCR on 38.101-4 for introduction of 8Rx performance requirements. Huawei, HiSilicon.