3GPP TSG-RAN4 Meeting # 111	R4-2409117

# Changsha, China,  15 – 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | xxxx | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.181: FR2 introduction in Clause 9.7.5 OTA spurious emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of FR2-NTN and SAN Type 2-O in TS 38.108 implies update of TS 38.181 with similar features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of spurious emission requirements for FR2-NTN, also aligned with 38.141-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 support not existing in clause 9.7.5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | At implementation check that Table 4.1.2.2-3 for FR2-NTN exists as well |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## -------- start of change ---------

### 9.7.5	OTA transmitter spurious emissions

#### 9.7.5.1	General

Unless otherwise stated, all requirements are measured as mean power.

The OTA transmitter spurious emission limits for FR1-NTN shall apply from 30 MHz to to the 5th harmonic of the upper frequency edge of the DL operating band., excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

NOTE 1: The lower limit and upper limit are as per ITU-R recommendation SM.329, Table 1: For systems operating within 600 MHz and 5.2 GHz, the lower limit is 30 MHz and the upper limit is the 5th harmonic of the higher frequency.

NOTE 2: The lower limit of 30 MHz can be replaced as per ITU-R SM.329-12: Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency.

The OTA transmitter spurious emission limits for FR2-NTN shall apply from 30 MHz to the 2nd harmonic of the upper frequency edge of the DL operating band, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

#### 9.7.5.2	General OTA transmitter spurious emissions requirements

##### 9.7.5.2.1	Definition and applicability

The general OTA transmitter spurious emissions requirements are specified as TRP per RIB, per cell, unless otherwise specified.

##### 9.7.5.2.2	Minimum requirement

The minimum requirement for SAN type 1-O is specified in TS 38.108 [2], clause 9.7.5.2.2.

The minimum requirement for SAN type 2-O is specified in TS 38.108 [2], clause 9.7.5.3.2.

##### 9.7.5.2.3	Test purpose

The test purpose is to verify if the radiated spurious emissions from the SAN at the RIB are within the specified minimum requirements.

##### 9.7.5.2.4	Method of test

###### 9.7.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see clause 4.9.1:

For FR1-NTN:

-	B when testing from 30 MHz to the lower frequency edge of the SAN transponder bandwidth minus 2 times BWSAN

-	T when testing from the upper frequency edge of the SAN transponder bandwidth plus 2 times BWSAN to 5th harmonic

For FR2-NTN:

- B when testing from 30 MHz to the lower frequency edge of the SAN transponder bandwidth minus 2 times BWSAN

- T when testing from upper frequency edge of the SAN transponder bandwidth plus 2 times BWSAN to 2nd harmonic

RF bandwidth positions to be tested in single-band multi-carrier operation, see clause 4.9.1:

For FR1-NTN:

-	BRFBW when testing from 30 MHz to the lower frequency edge of the SAN transponder bandwidth minus 2 times BWSAN

-	TRFBW when testing from the upper frequency edge of the SAN transponder bandwidth plus 2 times BWSAN to 5th harmonic

For FR2-NTN:

- BRFBW when testing from 30 MHz to the lower frequency edge of the SAN transponder bandwidth minus 2 times BWSAN

TRFBW when testing from the upper frequency edge of the SAN transponder bandwidth plus 2 times BWSAN to 2nd harmonic

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see TS 38.141-2 [18], annex I)

###### 9.7.5.2.4.2	Procedure

The following procedure for measuring TRP is based on directional power measurements as described in TS 38.141-2 [18], annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7.

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the SAN with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 9.7.5.2.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-2 for FR1-NTN and Table 4.1.2.2-3 for FR2-NTN.

5)	Set the SAN to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model in clause 4.9.2 (i.e. NR-SAN-FR1-TM1.1 for SAN type 1-O and NR-SAN-FR2-TM1.1 for SAN type 2-O), at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier operation, set the RIB to transmit according to the corresponding test model in clause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.7.2 and 4.8.

6)	Orient the positioner (and SAN) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see TS 38.141-2 [18], annex I).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see TS 38.141-2 [18], annex I).

NOTE 1:	The TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep or the TRP measurement grid sweep may be done in any order.

9)	Calculate TRP at each specified frequency using the directional measurements.

##### 9.7.5.2.5	Test requirement

###### 9.7.5.2.5.1	Test requirement for SAN type 1-O

For a SAN type 1-O, the TRP of any spurious emission shall not exceed the limits in table 9.7.5.2.5.1-1.

Table 9.7.5.2.5.1-1: General OTA SAN transmitter spurious emission limits for SAN type 1-O

| Spurious frequency range | Prated,c,TRP (dBm) | Test limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,t,TRP – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [4], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [4], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [4], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

###### 9.7.5.2.5.2	Test requirement for SAN type 2-O

For a SAN type 2-O, the TRP of any spurious emission shall not exceed the limits in table 9.7.5.2.5.2-1.

Table 9.7.5.2.5.2-1: General OTA SAN transmitter spurious emission limits for SAN Type 2-O

| Spurious frequency range | Prated,c,TRP (dBm) | Test limit (dBm) | Measurement bandwidth (kHz) | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 2nd harmonic of the upper frequency edge of the DL operating band | ≤ 47 | -13 | 4 | NOTE 1, NOTE 2, NOTE 3 |
|  | > 47 | Prated,t,TRP – 60dB |  |  |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [4], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [4], s2.5 table 1. NOTE 3: The lower frequency limit is replaced by 0.7 times the waveguide cut-off frequency, according to ITU-R SM.329 [4], for systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off. |  |  |  |  |

## 9.8	OTA transmitter intermodulation

The requirement is not applicable in this version of the specification.

## -------- end of change ---------