3GPP TSG-RAN WG4 Meeting #111             	R4-2409475

Fukuoka, Japan, 20th-24th May, 2024

Agenda item:	7.17.2

Source:	Samsung

Title:	Discussion and simulation results for further coverage enhancement

Document for:	Discussion

- Introduction

In the last RAN4# meeting, the test scope and test setup for BS demodulation requirement for further coverage enhancement was under discussion.  The related agreement was captured in the WF [1] as following

- Multiple PRACH transmission requirement - PRACH repetition number for BS performance requirements for Multiple PRACH transmission - Agree PRACH repetition number 2 - Sub Carrier Spacing for BS performance requirements for PRACH repetitions - Only consider 120kHz SCS for FR2-1: - Manufacturer declarations for supported SCS(s) for PRACH repetition test requirements - Candidate options - Option1: Introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition - Option2: No need to introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition - PRACH repetition interval and TDD pattern for Multiple PRACH transmission - Agreements on detailed network configurations: - Msg1-FDM =1 - TDD pattern: 3D1S1U for 15KHz, and 120kHz, 7D1S2U for 30kHz - msgA-SSB-PerRACH-Occasion: - msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 CHOICE - {one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}} - On PRACH interval based on the above configurations: - Option 1: Simulation assumption based on the following PRACH repetition interval - For FDD and TDD with 7D1S2U 30kHz SCS: Consecutive slots. - FFS whether PRACH repetition can be within 1 slot for PRACH format A2 and C2. - For TDD with DDDSU - Option 1A: Consecutive slots - Option 1B: DDDSUDDDSU for 15KHz and 120KHz SCS - Option 1C: DDDSUDDDSUDDDSUDDDSU for 120KHz SCS only - Option 2: Cover fixed PRACH interval for all the SCSs, i.e.,. 10ms - Companies are encouraged to bring simulation results of all options in the next meeting - Companies are encouraged to bring detailed PRACH configuration for the above options in the next meeting. - Companies to check if PRACH B4 can be configured in FDD 15kHz SCS based on available PRACH Configuration Index - PRACH configuration (for information) - Option 1 - FR1 TDD - Take PRACH Configuration Indexes {100, 159, 202} for A2, B4 and C2 respectively. - No msg1-RepetitionTimeOffsetROGroup is configured - FR2-1 TDD - Take PRACH Configuration Indexes {41, 135, 185} for A2, B4 and C2 respectively. - For A2 and C2, no msg1-RepetitionTimeOffsetROGroup is configured. - For B4, msg1-RepetitionTimeOffsetROGroup = n4 is configured. - Option 2 - PRACH configuration index for FR1 and paired spectrum: A2: 135 ¦ B4: 217 ¦ C2: 254 - PRACH configuration index for FR2 and unpaired spectrum: A2: 58 ¦ B4: 143 ¦ C2: 201 - Option 3 - 15KHz SCS - Format A2: PRACH configuration index, 127, number of RO within 10ms =3, number of SSB within 10ms=3 - Format B4: PRACH configuration index, 210, number of RO within 10ms =1, number of SSB within 10ms=1 - Format C2: PRACH configuration index, 246, number of RO within 10ms =2, number of SSB within 10ms=2 - 30KHz SCS - Format A2: PRACH configuration index 96, number of RO within 10ms =3, number of SSB within 10ms=3 - Format B4: PRACH configuration index, 156, number of RO within 10ms =1, number of SSB within 10ms=1 - Format C2: PRACH configuration index, 201, number of RO within 10ms =2, number of SSB within 10ms=2 - 120KHz SCS - Format A2: PRACH configuration index 41, number of RO within 10ms=4, number of SSB within 10ms =4 - Format B4: PRACH configuration index, 124, number of RO within 10ms=4, number of SSB within 10ms =4 - Format C2: PRACH configuration index, 185, number of RO within 10ms =4, number of SSB within 10ms=4

In this contribution, the view on the remaining issue on the test setup was provided, and the simulation results are provided for alignment purpose based on our proposed PRACH configuration index.

# 2	Discussion

Manufacturer declarations for supported SCS(s) for PRACH repetition test requirements

Regarding the manufacturer, in the Rel-15, the declaration on supported SCS and formats were introduced to allow different implementation of infra-vendors as following

| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 2-O: 60 kHz, 120 kHz or both. | c | x | x |
| --- | --- | --- | --- | --- | --- |

For frequency range, as agreed, only 120KHz requirement was introduced, and the requirement with both 15KHz and 120KHz are introduced. As for PRACH with multiple preamble transmission, different repetitions will be supported on top of the basic PRACH format feature. In general, if BS is declared to support one PRACH and one SCS, e.g., C2 with 120KHz SCS, and if BS is also declared to support format C2 with repetition, then, this BS should support PRACH format C2 with repetition in 120KHz SCS. Similarly, if BS is declared to support format C2 with 15KHz SCS and 30KHz SCS, and if BS is also declared to support format C2 with repetition, then, this BS should support PRACH format C2 with repetition in both 15KHz SCS and 30KHz SCS. It may be there is no such scenario that this BS only support the repetition in one SCS either 15KHz or 30KHz. Therefore, there is no need to introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition. While, we are also open to further discuss, if companies think it is necessary to add it, considering there is no harm to add this additional information.

Proposal 1: No need to introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition, The Manufacturer declarations of PRACH format for multiple PRACH transmission can be considered as following

| D.10x | PRACH format for Multiple PRACH transmission | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: A2, B4, C2 | c | x | x |
| --- | --- | --- | --- | --- | --- |

Feasibility checking of PRACH B4

In the last meeting, there is one open issue to check whether it is feasible to configure PRACH format B4 in FDD 15KHz SCS based on available PRACH configuration index. The following are candidate PRACH configuration index for FDD in Table 6.3.3.2-2

| PRACH Configuration Index | Preamble format | nf mod x=y |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 198 | B4 | 16 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 199 | B4 | 16 | 1 | 4 | 0 | 2 | 1 | 12 |
| 200 | B4 | 8 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 201 | B4 | 8 | 1 | 4 | 0 | 2 | 1 | 12 |
| 202 | B4 | 4 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 203 | B4 | 4 | 0 | 4 | 0 | 2 | 1 | 12 |
| 204 | B4 | 4 | 1 | 4,9 | 0 | 2 | 1 | 12 |
| 205 | B4 | 2 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 206 | B4 | 2 | 0 | 1 | 0 | 2 | 1 | 12 |
| 207 | B4 | 2 | 0 | 4 | 0 | 2 | 1 | 12 |
| 208 | B4 | 2 | 0 | 7 | 0 | 2 | 1 | 12 |
| 209 | B4 | 1 | 0 | 1 | 0 | 2 | 1 | 12 |
| 210 | B4 | 1 | 0 | 4 | 0 | 2 | 1 | 12 |
| 211 | B4 | 1 | 0 | 7 | 0 | 2 | 1 | 12 |
| 212 | B4 | 1 | 0 | 1,6 | 0 | 2 | 1 | 12 |
| 213 | B4 | 1 | 0 | 2,7 | 0 | 2 | 1 | 12 |
| 214 | B4 | 1 | 0 | 4,9 | 0 | 2 | 1 | 12 |
| 215 | B4 | 1 | 0 | 1,4,7 | 0 | 2 | 1 | 12 |
| 216 | B4 | 1 | 0 | 0,2,4,6,8 | 0 | 2 | 1 | 12 |
| 217 | B4 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 0 | 2 | 1 | 12 |
| 218 | B4 | 1 | 0 | 1,3,5,7,9 | 0 | 2 | 1 | 12 |

To reduce the test effort, only X =1 and Y=0 is considered, taking the PRACH configuration index 210 as example, The symbol position  for first RO is given by

l=l0+ntRANdurRA+14nslotRA

Since the targeting SCS is 15KHz, according to the spec, 211, then the related  should be 0

if ∆fRA∈1.25, 5, 15, 60 kHz, then

Therefore, there is no issue to support PRACH repetition with format B4 in 15KHz FDD, where the interval between 2 ROs is 10ms

- Observation 1: It is feasible to consider the format B4 in 15KHz SCS FDD with repetition transmission, where the interval between 2 ROs is 10ms

FFS on the PRACH repetition can be within 1 slot for PRACH format A2 and C2.

For PRACH format A2 and C2, the PRACH duration for A2 and C2 are 4 or 6, respectively. Pending on configured PRACH index, which will indicate the starting symbol and number of RO within a PRACH slot, it can support PRACH repetition at last with 2 repetitions, from spec perspective, such as PRACH configuration indexes

| PRACH Configuration Index |  | Preamble format |  | nf mod x=y |  |  | Subframe number |  | Starting symbol |  | Number of PRACH slots within a subframe |  | , number of time-domain PRACH occasions within a PRACH slot |  | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 130 | A2 |  | 1 |  | 0 | 1 |  | 0 |  | 2 |  | 3 |  | 4 |  |  |
| 131 | A2 |  | 1 |  | 0 | 7 |  | 0 |  | 2 |  | 3 |  | 4 |  |  |
| 246 | C2 |  | 1 |  | 0 | 4 |  | 0 |  | 1 |  | 2 |  | 6 |  |  |

While from the practical deployment scenario, it is too dense considering multiple ROs transmission within 10ms. For example, there would be 20ROs for FDD in 15KHz SCS. Since it was agreed that the SSB-perRACH-Occasion mapping 1, then, the number of SSB will be 10 in case PRACH repetition within 1 slot for PRACH format A2 and C2, where the number of SSB is exceed the maximum number of SSB as 4 in 15KHz SCS, and the maximum number of SSB as 8 in 30KHz SCS. Therefore, it is not practical to consider the PRACH repetition within 1 slot for PRACH format A2 and C2.  Therefore, we prefer to choose the proper PRACH confirmation index, where less RO is transmitted within 10ms.

Proposal 2: Not consider the PRACH repetition within 1 slot for PRACH format A2 and C2

PRACH interval

With multiple PRACH transmission, the interval between multiple PRACH transmission is pending on the RO patten based on PRACH configuration. The PRACH detection performance in fading channel depends on the interval between two ROs.  With large value interval comparing doppler frequency, the performance is similar regardless of the interval. While in terms of the interval value is close to the doppler frequency, the performance can be worse. As shown in the company’s results, the performance between different interval will be diverge.  And with large number of intervals, the performance will be convergence regardless the number of intervals.

Regarding the valid PRACH occasions, it also depends on FDD or TDD configuration. In the last meeting, several candidate repetition intervals and TDD pattern are considered.

The following table is summary for valid RO PRACH occasion for each FDD and TDD pattern

| SCS (KHz) | TDD pattern | Number of valid RO within 10ms | Number of SSB within 10ms for RO repetition |
| --- | --- | --- | --- |
| 15 | FDD | 10 | 5 |
| 15 | DDDSU | 2 | 1 |
| 30 | FDD | 20 | 10 |
| 30 | 7D1S2U | 8 | 4 |
| 120 | DDDSU | 8 | 4 |
| Note: Assuming consecutive slots without multiple ROs repetition within 1 slot |  |  |  |

Since there is one UE transmission during the testing, which means that 2ROs with repetitions will be transmitted with one SSB, given that large performance among difference interval, we propose to apply the large interval between two ROs for simulation alignment and requirement derivation, such as 10ms.  Also, applying 10ms in FR1 and 5ms in FR2 is accepted for us.

Proposal 3: Applying the interval between two repetition ROs as 10ms for requirement derivation of PRACH with repetition transmission.

Proposal 4: Applying the interval between two repetition ROs as 10ms for requirement derivation of PRACH with repetition transmission in FR1, and applying the interval between two repetition ROs as 5ms for requirement derivation of PRACH with repetition transmission in FR2

In summary, as our preference to guarantee the interval between 2ROs is 10ms, the following PRACH configuration index can be considered as starting point

Proposal 5:  The following PRACH configuration index can be considered as starting point

- 15KHz SCS

  - Format A2

    - PRACH configuration index, 127, number of RO within 10ms =3, number of SSB within 10ms=3

  - Format B4

    - PRACH configuration index, 210, number of RO within 10ms =1, number of SSB within 10ms=1

  - Format C2

    - PRACH configuration index, 246, number of RO within 10ms =2, number of SSB within 10ms=2

- 30KHz SCS

  - Format A2

    - PRACH configuration index 96, number of RO within 10ms =3, number of SSB within 10ms=3

  - Format B4

    - PRACH configuration index, 156, number of RO within 10ms =1, number of SSB within 10ms=1

  - Format C2

    - PRACH configuration index, 201, number of RO within 10ms =2, number of SSB within 10ms=2

- 120KHz SCS

  - Format A2

    - PRACH configuration index 41, number of RO within 10ms=4, number of SSB within 10ms =4

  - Format B4

    - PRACH configuration index, 124, number of RO within 10ms=4, number of SSB within 10ms =4

  - Format C2

    - PRACH configuration index, 185, number of RO within 10ms =4, number of SSB within 10ms=4

# 3	Simulation Results

In this part, the simulation results for PRACH repetition are provided for both FR1 and FR2-1 based on the interval of 2ROs as 10ms

Table 1: initial simulation for PRACH with repetition transmission in FR1

| Test cases | SCS&BW | Format | length | Channel | Frequency offset (Hz) | Number of Rep | Number of Tx and Rx | Time error tolerance | Ncs | Logical sequence index | v | SNR@1%Missing detection |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz | B4 | 139 | AWGN | 0 | 2 | 1T2R | 0.52 us | 23 | 0 | 0 | -21.62 |
| 2 | 15KHz | A2 | 139 | AWGN | 0 | 2 | 1T2R | 0.52 us | 23 | 0 | 0 | -16.85 |
| 3 | 15KHz | C2 | 139 | AWGN | 0 | 2 | 1T2R | 0.52 us | 23 | 0 | 0 | -16.80 |
| 4 | 30KHz | B4 | 139 | AWGN | 0 | 2 | 1T2R | 0.26 us | 46 | 0 | 0 | -21.43 |
| 5 | 30 KHz | A2 | 139 | AWGN | 0 | 2 | 1T2R | 0.26 us | 46 | 0 | 0 | -16.78 |
| 6 | 30 KHz | C2 | 139 | AWGN | 0 | 2 | 1T2R | 0.26 us | 46 | 0 | 0 | -16.69 |
| 7 | 15 KHz | B4 | 139 | TDLC 300-100 | 400 | 2 | 1T2R | 2.33us | 23 | 0 | 0 | -15.32 |
| 8 | 15 KHz | A2 | 139 | TDLC 300-100 | 400 | 2 | 1T2R | 2.33us | 23 | 0 | 0 | -10.54 |
| 9 | 15 KHz | C2 | 139 | TDLC 300-100 | 400 | 2 | 1T2R | 2.33us | 23 | 0 | 0 | -10.67 |
| 10 | 30KHz | B4 | 139 | TDLC 300-100 | 400 | 2 | 1T2R | 1.77 us | 46 | 0 | 0 | -16.58 |
| 11 | 30KHz | A2 | 139 | TDLC 300-100 | 400 | 2 | 1T2R | 1.77us | 46 | 0 | 0 | -12.11 |
| 12 | 30KHz | C2 | 139 | TDLC 300-100 | 400 | 2 | 1T2R | 1.77us | 46 | 0 | 0 | -12.30 |

Table 2: initial simulation for PRACH with repetition transmission in FR2-1

| Test cases | SCS&BW | Format | length | Channel | Frequency offset(Hz) | Number of Rep | Number of Tx and Rx | Time error tolerance | Ncs | Logical sequence index | v | SNR@1%Missing detection |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 120KHz | B4 | 139 | AWGN | 0 | 2 | 1T2R | 0.07 us | 69 | 0 | 0 | -20.90 |
| 2 | 120KHz | A2 | 139 | AWGN | 0 | 2 | 1T2R | 0.07 us | 69 | 0 | 0 | -16.70 |
| 3 | 120KHz | C2 | 139 | AWGN | 0 | 2 | 1T2R | 0.07 us | 69 | 0 | 0 | -16.61 |
| 4 | 120KHz | B4 | 139 | TDLA30-300 | 4000 | 2 | 1T2R | 0.22 us | 69 | 0 | 0 | -15.45 |
| 5 | 120KHz | A2 | 139 | TDLA30-300 | 4000 | 2 | 1T2R | 0.22 us | 69 | 0 | 0 | -11.81 |
| 6 | 120KHz | C2 | 139 | TDLA30-300 | 4000 | 2 | 1T2R | 0.22 us | 69 | 0 | 0 | -11.69 |

# 4	Conclusion

In this contribution, the view on the remaining issue of BS demodulation requirement for further coverage enhancement is provided. And the initial results are provided based on our preference for PRACH configuration index

Proposal 1: No need to introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition, The Manufacturer declarations of PRACH format for multiple PRACH transmission can be considered as following

| D.10x | PRACH format for Multiple PRACH transmission | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: A2, B4, C2 | c | x | x |
| --- | --- | --- | --- | --- | --- |

- Observation 1: It is feasible to consider the format B4 in 15KHz SCS FDD with repetition transmission, where the interval between 2 ROs is 10ms

Proposal 2: Not consider the PRACH repetition within 1 slot for PRACH format A2 and C2

Proposal 3: Applying the interval between two repetition ROs as 10ms for requirement derivation of PRACH with repetition transmission.

Proposal 4: Applying the interval between two repetition ROs as 10ms for requirement derivation of PRACH with repetition transmission in FR1, and applying the interval between two repetition ROs as 5ms for requirement derivation of PRACH with repetition transmission in FR2

Proposal 5:  The following PRACH configuration index can be considered as starting point

- 15KHz SCS

  - Format A2

    - PRACH configuration index, 127, number of RO within 10ms =3, number of SSB within 10ms=3

  - Format B4

    - PRACH configuration index, 210, number of RO within 10ms =1, number of SSB within 10ms=1

  - Format C2

    - PRACH configuration index, 246, number of RO within 10ms =2, number of SSB within 10ms=2

- 30KHz SCS

  - Format A2

    - PRACH configuration index 96, number of RO within 10ms =3, number of SSB within 10ms=3

  - Format B4

    - PRACH configuration index, 156, number of RO within 10ms =1, number of SSB within 10ms=1

  - Format C2

    - PRACH configuration index, 201, number of RO within 10ms =2, number of SSB within 10ms=2

- 120KHz SCS

  - Format A2

    - PRACH configuration index 41, number of RO within 10ms=4, number of SSB within 10ms =4

  - Format B4

    - PRACH configuration index, 124, number of RO within 10ms=4, number of SSB within 10ms =4

  - Format C2

    - PRACH configuration index, 185, number of RO within 10ms =4, number of SSB within 10ms=4

# References

R4-2406025, WF on [109][325] NR_cov_enh2_demod

R4-2406025