3GPP TSG-RAN WG4 Meeting #111	R4-2409681

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	(NR_NTN_Solutions) On the definition of geosynchronous satellites

Agenda item:	4.1

Document for:	Discussion

Introduction

This paper aims at discussing the terminology adopted in the current specification for NTN in TS 38.101-5 (Rel-17 and Rel-18).

There are several instances in the specification where GEO terminology is used whereas in others GSO terminology is used. It is necessary to clarify the intent behind the terminology and align the wording for a clearer behavior. Currently, TS 38101-5 where the definition of GSO is not aligned with other specifications related to the NTN work item.

Discussion

In TS 38.101-5, GSO satellites are defined as “Geoestationary satellite orbit”, while GEO is defined as “Geostationary Earth Orbit”.

The current version of the abbreviation was introduced in RAN4 #109, by CR R4-2319859. And the main justification was that GSO, in ITU terms represents a “Geostationary orbit”, and 3GPP should align its terminology with that adopted by ITU, such that confusion can be avoided.

Although the justification has some merits, the idea behind the concept cannot be changed by reinterpretation of the terminology. Throughout the specification of NTN WI, several specifications groups addressed the terminology GSO as “Geosynchronous Orbit”. Observe from table 1 below:

Table 1. Definition of GSO per 3GPP specification

| GSO: Geosynchronous Orbit | GSO: Geostationary orbit |
| --- | --- |
| 36.331 36.102 36.133 36.300/38.300 36.521-3 36.521-4 38.306 | 38.101-5 |

GSO is defined as geosynchronous satellite in other specifications, including other RAN4 specifications and the specification of UE capabilities (TS 38.306).

Maintaining the current definition will cause some attrition between the capabilities defined by 38.306, which is supposed to indicate the UE supports Geosynchronous satellite and a specification that only covers for Geostationary satellites.

Besides, the current definition is not harmonious with the same specification, as in the same meeting, RAN4 #109, Annex A.4.2 has been updated to indicate that doppler values for GSO satellite might reach values up to 0.93 ppm, which unequivocally indicate the  spirit of the specification was addressing geosynchronous satellites.

Inside 38.101-5, Annex 4 clearly indicates that GSO is interpreted as a geosyncrhnous satellite.

Based on the discussion above we propose:

Update the definition of GSO satellites in 38.101-5 to Geosyncrhonous satellites, in order to keep the harmony among all other specifications.

Conclusion

This paper discussed the alignment of the terminology for satellite orbits in RAN4 specification TS 38.101-5. The discussion encompass GSO and NGSO definitions, and the following observations and proposals were made:

Observation 1: GSO is defined as geosynchronous satellite in other specifications, including other RAN4 specifications and the specification of UE capabilities (TS 38.306).

Observation 2: Inside 38.101-5, Annex 4 clearly indicates that GSO is interpreted as a geosyncrhnous satellite.

Proposal 1: Update the definition of GSO satellites in 38.101-5 to Geosyncrhonous satellites, in order to keep the harmony among all other specifications.

- References

- TR 38.821. Solutions for NR to support non-terrestrial networks (NTN). V16.2.0. 3GPP.

- TS 38.300. R; NR and NG-RAN Overall description; Stage-2 V18.1.0. 3GPP.

- TS 38.331. NR; Radio Resource Control (RRC); Protocol specification. V18.1.0. 3GPP.

- TR 38.133. NR; Requirements for support of radio resource management V18.5.0. 3GPP.

- R4-2303687, Updated Rel-17 RAN4 UE feature list for NR, CMCC, Athens, Greece, February 27 – March 3, 2022

- R4-2316967, WF on NTN RAN task, MediaTek inc, Xiamen, China, 9th – 13th October 2023