3GPP TSG-RAN WG4 Meeting # 111											R4-2410606

Fukuoka, Japan, 20th – 24th May 2024

Title: 	WF on MSD Analysis for HPUE for CA with PC2 on FDD carrier

Agenda Item:	6.18.2

Source: 	China Unicom

Document for:	Approval

1. Background

This WF captures the analysis and provides a way forward on MSD for HPUE for CA with PC2 on FDD carrier in RAN4-111 meeting [1], with following sub-topics:

Issue 1-1-1: MSD for PC2 DL_n25A-n41A-UL_n25

Issue 1-1-2: MSD for PC2 DL_CA_n2-n66 and CA_n25-n66

Issue 1-1-3: MSD for PC2 DL_n41A-n66A-UL_n66

Issue 1-1-4: MSD for PC2 DL_CA_n25-n77-UL_n25

Issue 1-1-5: MSD for PC2 DL_CA_n8-n41-UL_n8

Issue 1-1-6: MSD for PC2 DL_n71-n77-UL_n71

Issue 1-1-7: MSD for PC2 DL_n8-n79-UL_n8

Issue 1-1-8: MSD for PC2 DL_n71-n85-UL_n71

Issue 1-1-9: MSD for PC2 DL_n71(2A)-UL_n71

Issue 1-1-9-1: PC3 n71(2A) MSD

Issue 1-1-9-2: PC2 CA_n71(2A) MSD

Issue 1-1-9-3: Test points for PC3 and PC2 CA_n71(2A) REFSENS

Issue 1-1-10:  MSD for PC2 DL_n71B-UL_n71

Issue 1-1-10-1: PC3 n71B MSD

Issue 1-1-10-2: PC2 CA_n71B MSD

<Topic 1 MSD Analysis Overview>

For following sub-topics, TPs were submitted, the discussion will be focused on the TP documents and the revisions.

Issue 1-1-1: MSD for PC2 DL_n25A-n41A-UL_n25

| R4-2407710 | T-Mobile USA | TP for TR 38.850 DL CA_n25A-n41A UL n25A PC2 |
| --- | --- | --- |

Issue 1-1-2: MSD for PC2 DL_CA_n2-n66 and CA_n25-n66

| R4-2407215 | AT&T | TP for TR 38.850 Addition of Single UL PC2 FDD for CA_n2-n66 |
| --- | --- | --- |
| R4-2407711 | T-Mobile USA | TP for TR 38.850 DL CA_n25A-n66A UL n25A PC2 n66A PC2 |

Issue 1-1-5: MSD for PC2 DL_CA_n8-n41-UL_n8

| R4-2407946 | CMCC, Huawei, HiSilicon, Murata | (HPUE_FR1_FDD_NR_CADC_R18) TP for TR 38.850 to introduce PC2 CA_n8A-n41A on n8 with TxD |
| --- | --- | --- |

Issue 1-1-7: MSD for PC2 DL_n8-n79-UL_n8

| R4-2407947 | CMCC, Huawei, HiSilicon, Murata | (HPUE_FR1_FDD_NR_CADC_R18)TP for TR 38.850 to introduce PC2 CA_n8A-n79A on UL n8 with TxD |
| --- | --- | --- |

Issue 1-1-9: MSD for PC2 DL_n71(2A)-UL_n71

| R4-2407708 | T-Mobile USA | TP for TR 38.850 DL CA_n71(2A) UL PC2 n71A |
| --- | --- | --- |

Issue 1-1-10: MSD for PC2 DL_n71B-UL_n71

| R4-2407712 | T-Mobile USA | TP for TR 38.850 DL CA_n71B UL PC2 n71A |
| --- | --- | --- |

<Topic 2 MSD Analysis>

For the following sub-topics, there are handled in the following TP:

| R4-2407709 | T-Mobile USA | TP for TR 38.850 Replacing TBD MSD for TxD |
| --- | --- | --- |

Issue 1-1-3: MSD for PC2 DL_n41A-n66A-UL_n66

Issue 1-1-4: MSD for PC2 DL_CA_n25-n77-UL_n25

Issue 1-1-6: MSD for PC2 DL_n71-n77-UL_n71

Issue 1-1-8: MSD for PC2 DL_n71-n85-UL_n71

<Sub-topic 1 MSD for PC2 DL_n41A-n66A-UL_n66>

  - Option 1: (R4-2407165, Skyworks) Band n41 is not affected by cross-band isolation interference from band n66 PC2 operation.

  - Option 2: (R4-2408856, Qualcomm) MSD for PC2 CA_n41A-n66A was provided with the following results.

PC2 1TX:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n41 | 1760 | 45 | 15 | 240 (RBstart=2) | 2501 | 10 | 1.2 | >ACLR2 |

PC2 2TX:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n41 | 1760 | 45 | 15 | 240 (RBstart=2) | 2501 | 10 | 1.4 | >ACLR2 |

<Way forward>: Capture the agreed values in the TP for this meeting.

<Sub-topic 2 MSD for PC2 DL_CA_n25-n77-UL_n25>

  - Option 1: (R4-2407160, Skyworks) Introducing a single MSD test point for 10MHz CBW. And adopting the following PC2 test points.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD3 | MSD4 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 26.9 | 31.8 | NOTE X | UL2/DL1 |
| NOTE 3: Applicable to UE supporting PC2 with single Tx. NOTE 4: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

<Way forward>: Capture the agreed values in the TP for this meeting.

<Sub-topic 3 MSD for PC2 DL_n71-n77-UL_n71>

  - Option 1: (R4-2407162, Skyworks) For Band n77 UL3/DL1 UL harmonic MSD in CA_n8-n41, consider adopting the following PC2 test points.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD3 | MSD4 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n71 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 12.8 | 17.9 | NOTE X | UL5/DL1 |
| NOTE 3: Applicable to UE supporting PC2 with single Tx. NOTE 4: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

<Way forward>: Capture the agreed values in the TP for this meeting.

<Sub-topic 4 MSD for PC2 DL_n71-n85-UL_n71>

  - Option 1: (R4-2407580, Murata) Use 2TX PC2 MSD as shown in Table.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 15.9x | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 32.3y | ACLR1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE x: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE y: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

<Way forward>: Capture the agreed values in the TP for this meeting.

Reference

[1] R4-2408924	Topic summary for [111][113] HPUE_Basket_FDD		Moderator (China Unicom)	RAN4-111